SOP HISTORICAL RECORD

| DATE | WRITTEN/REVISED* BY | SUPERVISOR REVIEW/APPROVAL | MD REVIEW/APPROVAL |
| --- | --- | --- | --- |
| 4-1-14 | C. Spaniol | B. Hall 4-1-14 | R. Khera 4-1-14 |
| 2-6-15 | C. Spaniol | B. Hall 2-6-15 | R. Khera 2-6-15 |
| 1-30-17 | C. Spaniol | B. Hall 1-30-17 | R. Khera 1-30-17 |
|  |  | B. Hall 5-4-18 |  |
|  |  | B. Hall 5-1-2020 |  |
|  |  | B. Hall 5/16/22 |  |

| *REVISION DATE | REVISION DESCRIPTION | ALL COPIES REVISED? |
| --- | --- | --- |
| 2-6-15 | Revised 6 month competency to be optional | yes |
|  | Removed “education in-services” as method of competency |  |
| 1-30-17 | Updated references | yes |

DOCUMENT CONTROL

| EFFECTIVE DATE: | 4-1-14 |  |
| --- | --- | --- |
| LOCATIONS OF ALL COPIES: | [MASTER - electronic] Lab-Dept. Shares |  |
|  | Ancillary manual-lab |  |
|  | Ancillary manuals-glucometer locations |  |
| REMOVED FROM SERVICE: |  |  |

Purpose

To provide dependable and accurate results for POC (Point-of Care) Glucose testing through in-service, knowledge and skills testing that certify users as competent.

Equipment

ACCU-CHEK Inform II meters

Accu-Chek Inform II test strips (cat. no. 05942861001)

Accu-Chek Inform II controls (cat. no. 05213509001)

Procedure

| STEP | ACTION |
| --- | --- |
| 1.initial | New employee orientation will include a training in-service covering glucometer operation. A test is administered at this in-service and the employee performs two levels of controls. Upon successful completion (70% or higher) of the test and demonstration of proper operator technique, the employee is given a unique ID number (barcode) and may perform testing for one (1) year. |
| 2. 6 month optional | Competency may be re-assessed at 6 months after the initial training by checking QC and/or patient testing records. This will be at the discretion of the Ancillary Testing Coordinator. If the employee has not done any testing during this period, the Ancillary Testing Coordinator may send an e-mail to the operator requesting the performance of at least 2 levels of QC in order to remain certified. If the operator does not comply, certification will be removed by operator lock-out. |
| 3. yearly | Yearly re-certification is required and will be demonstrated by two of the following: Testing an “unknown” specimen(s) Periodic direct observation of testing Monitoring operator’s quality control performance Written testing specific to the method used |
| 4. renewal | Authorization of testing personnel will be renewed annually thereafter, unless otherwise indicated by performance on routine QC, proficiency testing or lack of procedure compliance. Renewal will consist of demonstrating clinical competency through routine QC monitors and satisfactory completion of the authorization renewal program established for each ancillary testing procedure. Failure to complete the yearly re-certification process will result in operator lock-out from the meter. |
| 5. removal | Individuals not showing satisfactory QC and proficiency records or who are not following the stated policies and procedures will be removed from the list of authorized personnel. Reinstatement will occur only after additional training and clinical competency are demonstrated. |
| 6. barcode replacement | If an operator’s barcode becomes unreadable, or a new barcode is needed due to ID badge replacement, the Ancillary Testing Coordinator must be contacted for issue of a replacement. |

The Ancillary Testing Coordinator will maintain documentation of staff training and competency in P&LMS.

References

MCM 113-10 Ancillary (Waived) Blood Glucose Bedside Testing

VA P&LMS National Enforcement Program: Waived Testing Regulations Guideline