Training Checklist for Coaguchek XS Plus Meter

EMPLOYEE NAME:___________________________

|  | Date Completed |
| --- | --- |
| 1. Identification |  |
| - Patient ID (SSN) |  |
| 2. Test Preparation |  |
| - Coaguchek XS Plus Meter sign in |  |
| - Coaguchek XS Plus test strips |  |
| - Code Chip included in each box (know where to insert chip) |  |
| 3. Controls |  |
| - Perform external Quality Control on the first of the month. If the first falls on a weekend or Holiday run the QC the next working day. |  |
| - When running a new lot of QC insert new code chip until it snaps into place. |  |
| - Know how to properly prepare controls solution for test. |  |
| - Verify controls are within range. Follow proper troubleshooting if controls are not acceptable. |  |
| - Internal QC is done on every test device. |  |
| 4. Test Procedure |  |
| - Wash hands and put on gloves |  |
| - Properly prepare lancet device |  |
| - Remove test strip from container and replace cap tightly |  |
| - Insert test strip into meter |  |
| - Perform fingerstick when meter begins 180 sec countdown. |  |
| - Apply blood to strip when indicated |  |
| - Read and record result |  |
| - Discard used strip and lancet properly |  |
| - Clean meter with hospital approved wipes after each patient. |  |
| - Clean test strip guide with approved Alcohol wipes. |  |
| 4. Safety |  |
| - Universal Precautions |  |
| 6. Miscellaneous |  |
| - Know how to recall results from memory |  |
| - Aware that Roche Diagnostics Technical Service is 24 hour service and number is 1-800-428-4674. |  |
| - Know that battery is rechargeable and meter needs to be on the base for charging. |  |
| 7. Perform a test. |  |

Employee has completed training and is COMPETENT/ NOT COMPETENT to begin testing.

Employee Signature : ______________________________________________________

Evaluator Signature:_______________________________________________________

Ancillary Testing Coordinator Signature________________________________________