#### HEMACYTOMETER – INCYTO C-CHIP

| ##### Policy | Hemacytometer- INCYTO c-chip is used to do cell counts on body fluids and blood as required. |
| --- | --- |

| ##### Workplace safety | All laboratory employees are expected to maintain a safe working environment and an injury-free workplace. Laboratory employees are responsible for their own safety, the safety of others and adhering to all departmental and medical center safety policies and procedures. - For standard precautions and safety practices in the laboratory; see LGM 8000, specifically, but not limited to, equipment safety, proper body mechanics, sharps exposure and proper use of personal protective equipment (PPE). - For Universal Body Substance precautions, see LGM 8005, specifically, but not limited to, exposure to body fluids. - For proper hand washing, see LGM 8010, specifically, but not limited to, proper hand washing. - For proper infection control, see LGM 8004, specifically, but not limited to, proper use of gloves. - For proper handling of regular and infectious waste, see LGM 8006, specifically, but not limited to proper disposal of regular and biohazardous waste. - For proper cleaning of work area, see LGM 8007 - Cleaning Work Areas. - For proper handling of chemicals and reagents, see the Chemical Hygiene Plan. For proper storage and disposal of chemical hazardous waste, see LGM 8012.. |
| --- | --- |

| ##### Procedure | The C-chip is a disposable plastic hemocytometer used for manual cell counting. It is composed of two counting chambers that are separated by a horizontal grooved canal and bordered on each side by a similar vertical moat. The C-chip is exactly the same as the Neubauer Improved. It consists of 9 large squares, each measuring 1x1 mm, and the depth of the chamber is 0.1 mm. The area of each chamber is 3mm x 3mm= 9 mm2. The volume of the chamber is 0.9 mm3. The C-chip is used to enumerate blood cells and other cellular elements. See below for general formula for calculating cell counts. |
| --- | --- |

Large Square (W) = 1 mm2

Small Square (R) = 0.04 mm2   for one small square

= 0.2 mm2 for all 5 small squares

#### HEMACYTOMETER – INCYTO C-CHIP, Continued

General formula for cell counts

Total cells/ l = Total Average Count x 10 cells/l x dilution factor # of square mm counted

| ##### Analysis | Refer to LHM204 Body Fluid Analysis and LHM287 Fluid Quality Control Procedure for step by step usage of instrument |
| --- | --- |
| ##### Non-controlled documents |  |

| 1 | Clinical Diagnosis and Management by Laboratory Methods, 17th ed. W.B. Sounders Co. Philadelphia, 1984, pg 589-591 |
| --- | --- |
| 2 | Clinical Diagnosis and Management by Laboratory Methods, 18th ed. W.B. Sounders Co. Philadelphia, 1991, pg 564-566 |
| 3 | Hematology for medical technologists, Seiverd, Charles, Lea&Fegiger, Philadelphia, 1983, pg. 93-177. |

| ##### Controlled documents | See list below |
| --- | --- |

| # |  |
| --- | --- |
| LGM8000 | Standard precautions and safety practices in the laboratory |
| LGM8005 | Universal Body Substance precautions |
| LGM8010 | Proper hand-washing |
| LGM8006 | Infection control |
| LGM8007 | Cleaning Work Areas |
| LGM8012 | Proper storage and disposal of chemical hazardous waste |
| LHM204 | Body Fluid Analysis |
| LHM287 | Fluid Quality Control Procedure |

| ##### Author | Alvin Castillo, CLS |
| --- | --- |