#### GRAM STAIN

| Purpose | This document will define the procedure for Gram staining bacteriology specimens. The proper staining and reading of Gram stains is essential to physicians for their diagnosis and treatment of patients’ conditions, especially regarding whether or not to use antibiotics, and the choice of antibiotics. The qualities of the specimen, as well as the presence of infection, are among the many clues discernable from a well-prepared and well-read Gram stain. |
| --- | --- |

| Policy | - Pre made control slides of gram positive and gram negative organisms is provided by the Regional laboratory. |
| --- | --- |

| ##### Work place safety | All laboratory employees are expected to maintain a safe working environment and an injury-free workplace. Laboratory employees are responsible for their own safety, and the safety of others and adhering to all departmental and medical center safety policies and procedures. - For standard precautions and safety practices in the laboratory; see LGM 8000, specifically, but not limited to, equipment safety, proper body mechanics, sharps exposure and proper use of personal protective equipment (PPE). - For Universal Body Substance precautions, see LGM 8005, specifically, but not limited to, exposure to body fluids. - For proper hand-washing, see LGM 8010, specifically, not limited to, proper hand-washing. - For proper infection control, see LGM 8004, specifically, but not limited to, proper use of gloves. - For proper handling of regular and infectious waste, see LGM 8006, specifically, but not limited to, proper disposal of regular and bio hazardous waste. - For proper cleaning of work area, see LGM 8007 – Cleaning Work Areas. - For proper handling of chemicals and reagents, see the Chemical Hygiene Plan. - For proper storage and disposal of chemical hazardous waste, see LGM 8012. |
| --- | --- |

- GRAM STAIN, Continued

| - Specimen | - Any suitable specimen from the body which requires microscope identification of Gram positive or Gram negative organisms by using the Gram stain procedure. This may include any body fluid, any specimens for wounds, tissue, abscesses, lesions, aspirates, and drainage specimens as the primary specimen. - Note: - If received for gram stain, the following types of samples are not suitable for gram stain and must be rejected; e-swab, stool, blood, tissue, catheter tip, blood culture, any other swab received in gel media |
| --- | --- |

| ##### Equipment and reagents | Equipment - Microscopic slides - Staining rack - Microscope - Immersion oil Reagents - Crystal Violet Solution - Gram Iodine solution - Acetone-Alcohol - Safranin - Water Note: all reagents received pre-made and ready to use |
| --- | --- |

| ##### Quality control | Commercially prepared controls slides of gram positive and gram negative organisms are used for controls. - Gram stain control slides are stained once per day by the day shift to check on the quality of the staining procedure, of materials, and also on the gram staining technique. Each control slide includes both gram positive and gram negative bacteria. - A Laboratory Assistant may stain the slide for the Technologist to read - All Quality control results are recorded on the gram stain Quality Control sheet in the Bacteriology Quality Control notebook. |
| --- | --- |

Continued on next page

#### GRAM STAIN, Continued

| ##### Procedure | Follow the steps outlined below to Gram stain |
| --- | --- |

| Step | Action |
| --- | --- |
| 1 | Verify that QC is performed for the day and that QC results are acceptable prior to testing any patients. |
| 2 | Depending on the specimen, a smear is prepared by use of a sediment centrifuge specimen, un-spun specimen, swab, or directly from the specimen received. With the use of a pipette or swab, transfer a portion of the specimen to a previously flamed microscope slide and thin out as evenly as possible to approximate the size of a nickel. Specimens from swabs may be smeared in a horizontal direction thin enough to approximate a layer of specimen. See the cytospin procedure for preparation of the cytospin specimens for gram stain. |
| 3 | Allow the smear to air dry. |
| 4 | Flood the slide with crystal violet solution. |
| 5 | After 2 minutes pour off the crystal violet and gently wash the slide with water |
| 6 | Flood the slide with iodine solution. |
| 7 | After 1 minute rinse off the iodine solution with water |
| 8 | Decolorize with the acetone-alcohol solution for 30 seconds and wash immediately with water |
| 9 | Flood with safranin solution for 30 seconds and rinse off with water |
| 10 | Blot the slide dry with a clean filter paper or a clean paper towel. |
| 11 | Examine the smear under oil immersion (100X) |
| 12 | Reviewing Sterile Specimens with Positive/Abnormal results for Gram Stain - Sterile specimens with positive/abnormal results for Gram stain should be reviewed by another/second CLS. - The slide should be reviewed by scanning the entire slide, same procedure the first CLS examined the slide |

The waste stream from the staining process is drained to a 5 gallon flammable hazardous waste container. When the container is full, it is stored in a large flammable cabinet located at room 1818 for disposal. Replace with an empty 5 gallon flammable waste container available at the same large flammable cabinet in room 1818.

Continued on next page

#### GRAM STAIN, Continued

| Interpretation | - Gram positive bacteria stain blue or purple against a red or pink background while gram negative bacteria stain red or pink against a red or pink background. - All bacterial organisms are reported as well as any other organisms / elements present. This also includes the presence of fungi, white blood cells, mucus and epithelial cells if present. - Shapes of bacteria may be cocci (round), rods (elongate), diplococci (kidney bean shaped pairs), and coccobacilli (rounded and elongated). - The report should reflect the degree (Many, Moderate, Few, None) of quantity of organisms, fungi, and white cells (polys) present. If organisms are in clumps this must be noted. Fungi may be reported as budding or with hyphae. - If organisms appear gram variable it may be due to poor staining properties, therefore, the controls should be rechecked for proper staining characteristics and then a new smear of the patient should be prepared before reporting as gram variable. - If there are no organisms present, it must be stated as such. |
| --- | --- |

| REPORTING RESULTS: |  |
| --- | --- |
| Step | Action |
| 1 | Log onto CERNER. |
| 2 | On your CERNER Apps: - Click on Result Entry for Microbiology |
| 3 | At the Result Entry: - Enter the accession number of the patient to be resulted on the Accession Field - Type FIN on the Entry Field (final report) |

Continued on next page

GRAM STAIN, Continued

| 4 | At the Final Report window. - Under the Response section, you will have cells to input the necessary display code for reporting. - Press F2 to access the Response List of Values - Filter the by choosing Report Response - Click on the necessary response code for reporting and press OK - After inputting the codes for resulting - Choose Perform to hold the result if it needs to be called to a provider. (Positive/Abnormal results for CSF and Body Fluid should be called) - Choose Verify to release the Final report. |
| --- | --- |

5	         Inputting a comment for Positive/Abnormal  results for CSF and Body Fluid

- After choosing Perform, under Microbiology Result Entry

- click on the  Comment button

- Under the Comment window,  choose the Order Note tab and

press the Edit button

- Press F2 to access the templates used for called results.

- Fill up the necessary information like date, time, and name of the

person  called.

- Close the comment window

- Double click on the final report to be verified

- Choose the Verify button to result out the Final Report

Continued on next page

GRAM STAIN, Continued

Grading to follow for reporting results

Enter all elements seen semi-quantitatively.

Rare (2-5 / 100X)

Few (5-10 / 100X)

Moderate (10-15 / 100X)

Many (>15 / 100X)

| DISPLAY | DESCRIPTION |
| --- | --- |
| CORRWP | Test performed on wrong patient |
| CRPT | Corrected Report |
| EPIS | Epithelial cells seen |
| F | Few |
| FES | Fungal elements seen |
| FUN | Fungal elements present |
| GNC | Gran negative cocci |
| GRPB | Gram positive rods, bacillus-like |
| GVCB | Gram variable coccobacilli |
| Intra | Intracellular |
| KOHN | No fungal elements observed |
| KOHY | Yeast observed |
| M | Many |
| Mix | Mixed flora, no predominant organism |
| Mod | Moderate |
| NF | Normal flora isolated |
| NFES | No fungal elements seen |
| No | No |

Continued on next page

GRAM STAIN, Continued

| DISPLAY | DESCRIPTION |
| --- | --- |
| Nohy | No hyphal elements seen |
| NOS | No organism seen |
| NSQC | No squamous cells |
| NWBC | No white blood cells seen |
| OCC | Occasional |
| PLEO | Pleomorphic |
| PMN | Polymorphonuclear leukocytes |
| PMNS | Polymorphonuclear neutrophils |
| R | Rare |
| RBC | Red Blood Cells |
| S | Sparse |
| SGL | In single |
| Squam | Squamous epithelial cells |
| Tet | In tetrads |
| WBC | White blood cells |
| WBCS | White blood cells seen |

Continued on next page

GRAM STAIN, Continued

| ##### Procedural notes | The Gram stain procedure consists of staining a fixed film with crystal violet solution containing ammonium oxalate, washing this off with water, and flooding with Gram’s iodine solution. The iodine solution is washed after a time and the film decolorized with acetone-alcohol, and counterstained with a dye of contrasting color, such as safranin. By this means, bacteria may be divided into two groups: those which retain the original crystal violet in spite of the decolorization and appear dark purple on a background of the counterstain and are considered as gram-positive bacteria. Those which are decolorized and are colored by the counterstain are considered as gram-negative. Certain bacteria are gram-variable, sometimes retaining the stain and sometimes not. But this group is not large enough to detract from the practical differential value of the stain. Sometimes, inaccuracies may occur due to either under or over decolorization of the slide during the acetone-alcohol phase of staining. But this situation is usually corrected by repetition of the stain. Certain organisms such as Gardnerella vaginalis may stain gram-variable (showing both gram negative and gram positive characteristics). |
| --- | --- |

Continued on next page

GRAM STAIN, Continued

| ##### Uncontrolled documents | - Lennette, E. H., Balows, A., Hausler, W Manual of clinical Microbiology, Am. Scc. For Microbiology, Washington D. C., 3rd ed., 1980 - Finegold, S. M., Mortin, W. J., Bailey and Scott’s diagnostic microbiology, The C. V. Mosby Co., St. Louis, Toronto, London, 9th ed., 1994 - Balows, A., Hausler, W. Jr., Hermann, K. H. D., Shadony, H. J., Manual of Clinical Microbiology, 5TH ED., 1991 |
| --- | --- |

| ##### Controlled documents | See below |
| --- | --- |

| Documents | Title |
| --- | --- |
| LUS 5090 | Quality Control Urine |
| LGM 2022 | Quality Manual |
| LGM 8000 | Safety Practices |
| LGM 8004 | Infection Control |
| LGM 8005 | Universal Body Substance Precautions |
| LGM 8006 | Handling of Regular and Infectious Waste |
| LGM 8007 | Cleaning Work Areas |
| LGM 8010 | Hand-washing Policy |
| LGM 8012 | Storage and disposal of Chemical Hazardous Waste |

| ##### Author | Alvin Castillo, CLS |
| --- | --- |

Continued on next page

GRAM STAIN, Continued

#### Reviewed and approved by:

| SIGNATURE | DATE |
| --- | --- |
| Alvin Castillo, CLS Supervisor Los Angeles Medical Center |  |
| Julie Toti, CLS, MS Manager, Clinical Laboratory Los Angeles Medical Center |  |
| Joseph Thompson, M.D. Director, Clinical Laboratory Los Angeles Medical Center |  |

Continued on next page

GRAM STAIN, Continued

Document History Page

Effective Date:

| Change type: New, Major, Minor etc. | Changes Made to SOP – describe | Signature responsible person/date | Med. Dir. Reviewed/ Date | Lab Manager reviewed/ date | Date change implemented |
| --- | --- | --- | --- | --- | --- |