#### MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD

| Policy | It is our policy to perform manual urine microscopic examination, whenever automated method is not available due to low volume of specimen; error run of instrument caused by turbidity, clogs, or bloody specimens; or when instrument is out of order. |
| --- | --- |

| Workplace Safety | All laboratory employees are expected to maintain a safe working environment And an injury-free workplace. Laboratory employees are responsible for their own safety, and the safety of others and adhering to all departmental and medical center safety policies and procedures. - For standard precautions and safety practices in the laboratory; see LGM 8000, specifically, but not limited to, equipment safety, proper body mechanics, sharps exposure and proper use of personal protective equipment (PPE). - For Universal Body Substance precautions, see LGM 8005, specifically, but not limited to, exposure to body fluids. - For proper hand-washing, see LGM 8010, specifically, not limited to, proper hand-washing. - For proper infection control, see LGM 8004, specifically, but not limited to, proper use of gloves. - For proper handling of regular and infectious waste, see LGM 8006, specifically, but not limited to, proper disposal of regular and bio hazardous waste. - For proper cleaning of work area, see LGM 8007 – Cleaning Work Areas. - For proper handling of chemicals and reagents, see the Chemical Hygiene Plan. - For proper storage and disposal of chemical hazardous waste, see LGM 8012 |
| --- | --- |

Continued on next page

#### MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD, Continued

| ##### Procedure | Microscopic examination is performed on all abnormal urine samples positive for any of the following parameters, RBC, leukocyte esterase, nitrite, and protein 1+or> and if a urine microscopic analysis is specifically ordered by a physician. Follow the procedure outlined below for routine urine microscopic analysis. |
| --- | --- |

| Step | Action |
| --- | --- |
| 1 | Pour 10 ml of urine into a bar-coded conical tube for a specimen requiring microscopic examination. Alternatively the tube used for routine urinalysis can be used. Note: If less than 10 ml urine is available see instructions in step 3 for decanting. |
| 2 | Spin specimen at 2000 RPM for 5 minutes. |
| 3 | Decant 9 ml of supernatant, leaving 1 ml. Do not disturb sediment while decanting. Note: Less than 10 ml is spun decant 9/10 of supernatant leaving 1/10, for example 3 ml urine decant 2.7 ml of supernatant |
| 4 | Mix sediment with the remaining supernatant. Load well mixed suspension either into a Kova slide or place a drop on a microscope slide and coverslip. |
| 5 | Examine the sample under reduced illumination, first on low power (10x). Scan the field for the presence of casts. |
| 6 | Switch to high power (40x) and check for RBCs, WBCs, epithelial cells, mucus, crystals, bacteria, yeast, spermatozoa and any other abnormal and normal elements. |
| 7 | Some of the common abnormal and normal urine sediment elements are listed below. For more comprehensive description and pictures consult a Urinary Sediment Atlas or textbook. |

Continued on next page

#### MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD Continued

Procedure (continued)

|  | Common urine sediment components |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Element | Description |  |
|  |  | Cells |  |  |
|  |  | Squamous | 30-50, flat with central nucleus |  |
|  |  | Transitional | 20-30, spheric , with central spheric nucleus |  |
|  |  | Renal | 20-30, polyhedral, clinically significant in large numbers |  |
|  |  | RBC | 7-8 , biconcave disc, 0-3 RBC normal, larger number indicates disease |  |
|  |  | WBC | 12-15 , spheric, PMN most common, 0-5 normal, >5 inflamation, infection |  |
|  |  | Spermatozoa | 45-60, tadpole, motile head 4-6 normal in male urine. |  |
|  |  | Normal crystals in acid urine: |  |  |
|  |  | Uric acid | Yellow, colorless, needle or rhombic shape, soluble in NaOH |  |
|  |  | Amorphous urates | Yellowish, amorphous granules, soluble in NaOH |  |
|  |  | Calcium oxalate | Octahedral, dumbell, soluble in NaOH, dilute HCl, HNO3 |  |
|  |  | Hippuric Acid | Colorless pale, needles, six-sided prisms, soluble in NaOH, heat, ether, alcohol |  |
|  |  | Normal crystals in alkaline urine |  |  |
|  |  | Amorphous phosphates | Colorless, white, amorphous granules, soluble in HCl and CH3COOH |  |

Continued on next page

#### MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD Continued

Procedure (continued)

|  |  | Triple phosphate | Colorless, prisms(coffin lid), soluble in HCl |  |
| --- | --- | --- | --- | --- |
|  |  | Calcium phosphate | Colorless, flat irregular plates, wedge-shaped prisms, soluble in HCl, CH3COOH |  |
|  |  | Calcium carbonate | Colorless, dumbell, granules soluble in NaOH + NH3 |  |
|  |  | Abnormal crystals in acid urine |  |  |
|  |  | Tyrosine | Colorless, yellow, fine silky needles soluble in NaOH, HCl, heat |  |
|  |  | Leucine | Yellowish brown, spheroids with central striations, soluble in NaOH, hot CH3COOH |  |
|  |  | Cystine | Colorless hexagonal plates soluble in NaOH, HCl, NH4OH |  |
|  |  | Cholesterol | Colorless, flat plates with corners chipped out, soluble in CHCl3, ether, hot alcohol |  |
|  |  | Sulfa | Colorless, amorphous fan shaped bundles, soluble in strong CH3Cooh, NaOH, and acetone. |  |
|  |  | Bilirubin | Bile colored, granules and needles soluble in CH3COOH, HCl, NaOH, CHCl3, acetone ,ether |  |
|  |  | Starch | Colorless, irregularly round with dark striations to the enter, asymmetric Maltese cross in polarized light. |  |

Continued on next page

#### MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD Continued

Procedure (continued)

|  |  |  | Acellular casts: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Hyaline cast |  | Smooth or finely wrinkled surface, transparent |  |  |  |
|  |  |  | Granular cast |  | Fine or coarse granules on surface, originate from pre-existing cellular casts |  |  |  |
|  |  |  | Waxy cast |  | Thick, opaque, broken-off ends, notched edges |  |  |  |
|  |  |  | Fatty cast |  | Yellowish tan fat globules on cast surface |  |  |  |
|  |  |  | Cellular casts: |  |  |  |  |  |
|  |  |  | White blood cell cast |  | PMNs on surface of hyaline matrix |  |  |  |
|  |  |  | Red blood cell cast |  | RBCs on surface of hyaline matrix may show hemoglobin pigmentation |  |  |  |
|  |  |  | Epithelial cell cast |  | Renal epithelial cells on hyaline matrix, cells may be in rows |  |  |  |
|  |  |  | Mixed casts |  | Combination of the above casts |  |  |  |
|  |  |  | Other elements: |  |  |  |  |  |
|  |  |  | Bacteria |  | Not normally present, if present are either rod or cocci forms |  |  |  |
|  |  |  | Yeast, fungi |  | Not normally present, if found , round clear center with budding forms, often in clumps, hyphal forms resembling hair can also be seen |  |  |  |
|  |  |  | Mucus |  | This wavy strands that can be confused with hyaline casts |  |  |  |
|  |  |  |  | Fat droplets |  | Clear fat droplets either intracellular (oval fat bodies) or free forms. |  |  |

Continued on next page

#### MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD Continued

| Procedure (continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Trichomonas vaginalis | 15-20 with two pairs of anteriorly situated flagella and short midline undulating membrane |  |  |
|  |  |  | Enterobius vermicularis(pin worm) | 25x50, oval shaped with thick wall |  |  |

| ##### Urinalysis results entry into the computer | Follow the steps outlined below when resulting urinalysis results. |
| --- | --- |

| Step | Action |
| --- | --- |
| 1 | Log onto Cerner GenLab |
| 2 | Click Accession Result Entry (ARE) |
| 3 | Scan Cerner barcode. Click Retrieve |
| 4. | Start resulting microscopic component one by one as indicated below. |
| 4. | After entering all results, click VERIFY. |

Continued on next page

#### MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD Continued

| ##### Microscopic results entry for urinalysis | Follow the directions below for reporting urine microscopic results. Verify that urine chemistry results correlate with urine microscopic results prior to release. |
| --- | --- |

| Component | Results expands to |
| --- | --- |
| RBC | 0/hpf 0-3/hpf 4-10/hpf 11-25/hpf 26-50/hpf >50/hpf |
| WBC | 0 0-2/hpf 3-5/hpf 6-10/hpf 11-25/hpf >25/hpf >25hpf |
| Epithelial cells and bacteria | None Trace Few Moderate Many |
| Renal | None Few Moderate Many |
| Yeast, Mucous, Trich | None Present |
| Hyaline cast | 0 0-4/lpf 5-10/lpf 11-20/lpf >20/lpf |
| Cast–quantitation other than hyaline cast | 0/lpf 0-2/lpf 3-5/lpf 6-10/lpf 11-20/lpf >20/lpf |

Continued on next page

#### MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD Continued

Microscopic results entry for urinalysis (continued)

| Component | Results Expand to |
| --- | --- |
| #### TP04 Crys CaOx Crys CaPhos Crys CaCB Crys Uric Crys | None Few Moderate Many |
| #### Abnormal crystals: Leucine, tyrosine, cystine | None Positive |
| Hyphae Yeast | Non Present |
| Trans Epi | None Few Moderate Many |
| Oval Fat | None Present |
| Fat | None Few Moderate Many |

| ##### Non-controlled documentation | Haber Meryl, Urinary Sediment , A textbook Atlas, 1983 |
| --- | --- |

Continued on next page

#### MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD Continued

| Controlled documents |  |
| --- | --- |

| Document No | Document name |
| --- | --- |
| LIMM209.2 | Results entry into the LMS |
| LGM 2022 | Quality Manual |
| LGM 8000 | Safety Practices |
| LGM 8004 | Infection Control |
| LGM 8005 | Universal Body Substance Precautions |
| LGM 8006 | Handling of Regular and Infectious Waste |
| LGM 8007 | Cleaning Work Areas |
| LGM 8010 | Hand-washing Policy |
| LGM 8012 | Storage and disposal of Chemical Hazardous Waste |