#### MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD

| Policy | It is our policy to perform manual urine microscopic examination, whenever automated method is not available due to low volume of specimen; error run of instrument caused by turbidity, clogs, or bloody specimens; or when instrument is out of order. |
| --- | --- |

| Workplace Safety | All laboratory employees are expected to maintain a safe working environment And an injury-free workplace. Laboratory employees are responsible for their own safety, and the safety of others and adhering to all departmental and medical center safety policies and procedures. - For standard precautions and safety practices in the laboratory; see LGM 8000, specifically, but not limited to, equipment safety, proper body mechanics, sharps exposure and proper use of personal protective equipment (PPE). - For Universal Body Substance precautions, see LGM 8005, specifically, but not limited to, exposure to body fluids. - For proper hand-washing, see LGM 8010, specifically, not limited to, proper hand-washing. - For proper infection control, see LGM 8004, specifically, but not limited to, proper use of gloves. - For proper handling of regular and infectious waste, see LGM 8006, specifically, but not limited to, proper disposal of regular and bio hazardous waste. - For proper cleaning of work area, see LGM 8007 – Cleaning Work Areas. - For proper handling of chemicals and reagents, see the Chemical Hygiene Plan. - For proper storage and disposal of chemical hazardous waste, see LGM 8012 |
| --- | --- |

Continued on next page

#### MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD, Continued

| ##### Procedure | Microscopic examination is performed on all abnormal urine samples positive for any of the following parameters, RBC, leukocyte esterase, nitrite, and protein 1+or> and if a urine microscopic analysis is specifically ordered by a physician. Follow the procedure outlined below for routine urine microscopic analysis. |
| --- | --- |

| Step | Action |
| --- | --- |
| 1 | Pour 10 ml of urine into a bar-coded conical tube for a specimen requiring microscopic examination. Alternatively the tube used for routine urinalysis can be used. Note: If less than 10 ml urine is available see instructions in step 3 for decanting. |
| 2 | Spin specimen at 2000 RPM for 5 minutes. |
| 3 | Decant 9 ml of supernatant, leaving 1 ml. Do not disturb sediment while decanting. Note: Less than 10 ml is spun decant 9/10 of supernatant leaving 1/10, for example 3 ml urine decant 2.7 ml of supernatant |
| 4 | Mix sediment with the remaining supernatant. Load well mixed suspension either into a Kova slide or place a drop on a microscope slide and coverslip. |
| 5 | Examine the sample under reduced illumination, first on low power (10x). Scan the field for the presence of casts. |
| 6 | Switch to high power (40x) and check for RBCs, WBCs, epithelial cells, mucus, crystals, bacteria, yeast, spermatozoa and any other abnormal and normal elements. |
| 7 | Some of the common abnormal and normal urine sediment elements are listed below. For more comprehensive description and pictures consult a Urinary Sediment Atlas or textbook. |

Continued on next page

#### MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD Continued

Procedure (continued)

|  | Common urine sediment components |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Element | Description |  |
|  |  | Cells |  |  |
|  |  | Squamous | 30-50, flat with central nucleus |  |
|  |  | Transitional | 20-30, spheric , with central spheric nucleus |  |
|  |  | Renal | 20-30, polyhedral, clinically significant in large numbers |  |
|  |  | RBC | 7-8 , biconcave disc, 0-3 RBC normal, larger number indicates disease |  |
|  |  | WBC | 12-15 , spheric, PMN most common, 0-5 normal, >5 inflamation, infection |  |
|  |  | Spermatozoa | 45-60, tadpole, motile head 4-6 normal in male urine. |  |
|  |  | Normal crystals in acid urine: |  |  |
|  |  | Uric acid | Yellow, colorless, needle or rhombic shape, soluble in NaOH |  |
|  |  | Amorphous urates | Yellowish, amorphous granules, soluble in NaOH |  |
|  |  | Calcium oxalate | Octahedral, dumbell, soluble in NaOH, dilute HCl, HNO3 |  |
|  |  | Hippuric Acid | Colorless pale, needles, six-sided prisms, soluble in NaOH, heat, ether, alcohol |  |
|  |  | Normal crystals in alkaline urine |  |  |
|  |  | Amorphous phosphates | Colorless, white, amorphous granules, soluble in HCl and CH3COOH |  |

Continued on next page

#### MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD Continued

Procedure (continued)

|  |  | Triple phosphate | Colorless, prisms(coffin lid), soluble in HCl |  |
| --- | --- | --- | --- | --- |
|  |  | Calcium phosphate | Colorless, flat irregular plates, wedge-shaped prisms, soluble in HCl, CH3COOH |  |
|  |  | Calcium carbonate | Colorless, dumbell, granules soluble in NaOH + NH3 |  |
|  |  | Abnormal crystals in acid urine |  |  |
|  |  | Tyrosine | Colorless, yellow, fine silky needles soluble in NaOH, HCl, heat |  |
|  |  | Leucine | Yellowish brown, spheroids with central striations, soluble in NaOH, hot CH3COOH |  |
|  |  | Cystine | Colorless hexagonal plates soluble in NaOH, HCl, NH4OH |  |
|  |  | Cholesterol | Colorless, flat plates with corners chipped out, soluble in CHCl3, ether, hot alcohol |  |
|  |  | Sulfa | Colorless, amorphous fan shaped bundles, soluble in strong CH3Cooh, NaOH, and acetone. |  |
|  |  | Bilirubin | Bile colored, granules and needles soluble in CH3COOH, HCl, NaOH, CHCl3, acetone ,ether |  |
|  |  | Starch | Colorless, irregularly round with dark striations to the enter, asymmetric Maltese cross in polarized light. |  |

Continued on next page

#### MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD Continued

Procedure (continued)

|  |  |  | Acellular casts: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Hyaline cast |  | Smooth or finely wrinkled surface, transparent |  |  |  |
|  |  |  | Granular cast |  | Fine or coarse granules on surface, originate from pre-existing cellular casts |  |  |  |
|  |  |  | Waxy cast |  | Thick, opaque, broken-off ends, notched edges |  |  |  |
|  |  |  | Fatty cast |  | Yellowish tan fat globules on cast surface |  |  |  |
|  |  |  | Cellular casts: |  |  |  |  |  |
|  |  |  | White blood cell cast |  | PMNs on surface of hyaline matrix |  |  |  |
|  |  |  | Red blood cell cast |  | RBCs on surface of hyaline matrix may show hemoglobin pigmentation |  |  |  |
|  |  |  | Epithelial cell cast |  | Renal epithelial cells on hyaline matrix, cells may be in rows |  |  |  |
|  |  |  | Mixed casts |  | Combination of the above casts |  |  |  |
|  |  |  | Other elements: |  |  |  |  |  |
|  |  |  | Bacteria |  | Not normally present, if present are either rod or cocci forms |  |  |  |
|  |  |  | Yeast, fungi |  | Not normally present, if found , round clear center with budding forms, often in clumps, hyphal forms resembling hair can also be seen |  |  |  |
|  |  |  | Mucus |  | This wavy strands that can be confused with hyaline casts |  |  |  |
|  |  |  |  | Fat droplets |  | Clear fat droplets either intracellular (oval fat bodies) or free forms. |  |  |

Continued on next page

#### MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD Continued

| Procedure (continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Trichomonas vaginalis | 15-20 with two pairs of anteriorly situated flagella and short midline undulating membrane |  |  |
|  |  |  | Enterobius vermicularis(pin worm) | 25x50, oval shaped with thick wall |  |  |

| ##### Correlation of microscopic findings with macroscopic results | Microscopic results should be correlated with the physical and chemical findings to ensure the accuracy of the report. Specimens in which the results do not correlate must be rechecked for both technical and clerical errors. The table shows some of the more common correlations in the urinalysis; however the amount of formed elements or chemicals must also be taken into consideration, as must the possibility of interference with chemical tests and the age of the specimen. |
| --- | --- |

|  | MICROSCOPIC ELEMENTS; PHYSICAL; CHEMICAL. Casts; -----; + Protein. Red Blood Cells; Turbidity Red Color; + Blood. White Blood Cells; Turbidity; + Protein + Leukocytes + Nitrite. Epithelial Cells; Turbidity; -----. Crystals; Turbidity Color; pH. Bacteria; Turbidity; + Nitrite pH. |
| --- | --- |

MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD Continued

| ##### Urinalysis results entry into the computer | Follow the steps outlined below when resulting urinalysis results. |
| --- | --- |

| Step | Action |
| --- | --- |
| 1 | Log onto Cerner GenLab |
| 2 | Click Accession Result Entry (ARE) |
| 3 | Scan Cerner barcode. Click Retrieve |
| 4. | Start resulting microscopic component one by one as indicated below. |
| 4. | After entering all results, click VERIFY. |

Continued on next page

#### MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD Continued

| ##### Microscopic results entry for urinalysis | Follow the directions below for reporting urine microscopic results. Verify that urine chemistry results correlate with urine microscopic results prior to release. |
| --- | --- |

| Component | Results expands to |
| --- | --- |
| RBC | 0/hpf 0-3/hpf 4-10/hpf 11-25/hpf 26-50/hpf >50/hpf |
| WBC | 0 0-2/hpf 3-5/hpf 6-10/hpf 11-25/hpf >25/hpf |
| Epithelial cells and bacteria | None Trace Few Moderate Many |
| Renal | None Few Moderate Many |
| Yeast, Mucous, Trich | None Present |
| Hyaline cast | 0 0-4/lpf 5-10/lpf 11-20/lpf >20/lpf |
| Cast–quantitation other than hyaline cast | 0/lpf 0-2/lpf 3-5/lpf 6-10/lpf 11-20/lpf >20/lpf |

Continued on next page

#### MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD Continued

Microscopic results entry for urinalysis (continued)

| Component | Results Expand to |
| --- | --- |
| #### TP04 Crys CaOx Crys CaPhos Crys CaCB Crys Uric Crys | None Few Moderate Many |
| #### Abnormal crystals: Leucine, tyrosine, cystine | None Positive |
| Hyphae Yeast | Non Present |
| Trans Epi | None Few Moderate Many |
| Oval Fat | None Present |
| Fat | None Few Moderate Many |

| ##### Non-controlled documentation | Haber Meryl, Urinary Sediment , A textbook Atlas, 1983 |
| --- | --- |

Continued on next page

#### MICROSCOPIC EXAMINATION OF URINE, MANUAL METHOD Continued

| Controlled documents |  |
| --- | --- |

| Document No | Document name |
| --- | --- |
| LIMM209.2 | Results entry into the LMS |
| LGM 2022 | Quality Manual |
| LGM 8000 | Safety Practices |
| LGM 8004 | Infection Control |
| LGM 8005 | Universal Body Substance Precautions |
| LGM 8006 | Handling of Regular and Infectious Waste |
| LGM 8007 | Cleaning Work Areas |
| LGM 8010 | Hand-washing Policy |
| LGM 8012 | Storage and disposal of Chemical Hazardous Waste |