| ##### Purpose | This document will define the procedure for the staining, detection and quantification of white blood cells in a stool specimen. |
| --- | --- |
| ##### Workplace Safety | All laboratory employees are expected to maintain a safe working environment and an injury-free workplace. Laboratory employees are responsible for their own safety, the safety of others and adhering to all departmental and medical center safety policies and procedures. - For standard precautions and safety practices in the laboratory; see LGM 8000, specifically, but not limited to, equipment safety, proper body mechanics, sharps exposure and proper use of personal protective equipment (PPE). - For Universal Body Substance precautions, see LGM 8005, specifically, but not limited to, exposure to body fluids. - For proper hand-washing, see LGM 8010, specifically, but not limited to, proper hand-washing. - For proper infection control, see LGM 8004, specifically, but not limited to, proper use of gloves. - For proper handling of regular and infectious waste, see LGM 8006, specifically, but not limited to proper disposal of regular and biohazardous waste. - For proper cleaning of work area, see LGM 8007 - Cleaning Work Areas. - For proper handling of chemicals and reagents, see the Chemical Hygiene Plan. For proper storage and disposal of chemical hazardous waste, see LGM 8012. |

| ##### Specimen | - Fresh random stool specimen collected in a sterile container without any preservatives delivered to the laboratory as soon after collection as possible. - Stool specimens contaminated with urine are rejected. |
| --- | --- |

| ##### Supplies needed | The following is a list of supplies needed before proceeding with the procedure: - Glass slide - Wooden applicator sticks. - Saline. - Hema-tek slide stainer(Wright-Giemsa Stain) |
| --- | --- |

| ##### Preparing slide | Follow the steps below to prepare a slide: |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | If the stool is … | then you would … |  |
|  |  | liquid | place a drop of the liquid stool on a glass slide |  |
|  |  | formed | - Place a drop of saline on the slide - Using a wooden applicator stick pick up a small portion of stool. - Mix the stool with the saline on the slide. |  |
|  |  | full of mucus | Smear some of the mucus drops into an area about 1/3 of the slide using a wooden applicator stick. |  |

| ##### Completing analysis | Follow the steps below to complete the analysis for white blood cells in stool specimens. |
| --- | --- |

| Step | Action |
| --- | --- |
| 1. | Allow smears to air dry. |
| 2. | Place dry smears on the Hema-Tek slide stainer for staining. |
| 3. | View the slide using the oil immersion lens (100x), looking for the presence of white blood cells(polymorphonuclear neutrophils only). |
| 4. | Semi-quantitate the white blood cells if any are present. |
| 5. | Results are reported in Cerner Micro Result Entry |
| 6 | Type FIN under Entry field to show Final Report window |
| 7 | After the Final Report window pops up, Press F2 and choose Report responses to show Response List of Values |
| 8 | Report responses are: - Rare WBC’s seen = 0-1 WBC/oil immersion field. - Few WBC's seen = 1-5 WBC/oil immersion field. - Moderate WBC"S seen = 5-10 WBC/oil immersion field. - Many WBC's seen = >10 WBC/oil immersion field. |
| 9 | Choose the necessary response code combination to report Fecal WBC. |
| 10 | Click Verify to report Final Result |

| ##### Procedural note | Identification of white cells is more difficult if the slide is too thick. The slide should be remade using less specimen. |
| --- | --- |

| ##### Quality of stain | No Fecal WBC QC is performed, we check Quality of stain as part of the Hema-tek stainer Daily Maintenance. |
| --- | --- |

| ##### Uncontrolled documents | Balows, A. Hauser, W.J. Jr. Herrman, K.L. Isenberg, H.D. Shadony, H.J: Manual of clinical Microbiology, 5th ed. Pgs. 30, 365, 1308, 1991 |
| --- | --- |

| ##### Controlled Documents | See list below |
| --- | --- |

| LGM8000 | Standard precautions and safety practices in the laboratory |
| --- | --- |
| LGM8005 | Universal Body Substance precautions |
| LGM8010 | Proper hand-washing |
| LGM8006 | Infection control |
| LGM8007 | Cleaning Work Areas |
| LGM8012 | Proper storage and disposal of chemical hazardous waste |

| ##### Author | Alvin Castillo, CLS |
| --- | --- |

HISTORY PAGE

| Change type: New, Major, Minor etc. | Changes Made to SOP – describe | Signature responsible person/date | Quality officer reviewed/date | Lab. Director reviewed/date | Med. Director reviewed/date | Date change implemented |
| --- | --- | --- | --- | --- | --- | --- |
| NEW |  |  |  |  |  |  |

Reviewed and approved by:

| SIGNATURE | DATE |
| --- | --- |
| Alvin Castillo, CLS Hematology supervisor Los Angeles Medical Center |  |
| Jocelyn Javier, CLS, MT Quality Officer, Clinical Laboratory Los Angeles Medical Center |  |
| Julie Toti, CLS, MS Laboratory Director, Clinical Laboratory Los Angeles Medical Center |  |
| Joseph Thompson, M.D. Medical Director, Clinical Laboratory Los Angeles Medical Center |  |