#### KPPI HAND HELD INFECTION PREVENTION FOR ISOLATION ROOM

| ##### Purpose | This procedure provides instructions for safe handling of equipment and infection prevention and control when using the KPPI hand held device during inpatient phlebotomy. |
| --- | --- |

| ##### Workplace Safety | - For equipment safety, proper body mechanics, sharp exposure, other safety practices, see LGM 8000 - Safety Practices. - For proper use of gloves, see LGM 8004 - Infection Control. - For exposure to body fluids, see LGM 8005 - Universal Body Substance Precautions. - For proper handling of regular and biohazardous waste, see LGM 8006 - Handling of Regular and Infectious Waste. - For proper cleaning of work areas, see LGM 8007 - Cleaning Work Areas. - For proper handwashing, see LGM 8010 - Handwashing Policy |
| --- | --- |

| ##### Scope | This procedure is intended for all laboratory staff using the hand held KPPI scanning device. |
| --- | --- |

| ##### Policy | - The Kaiser Proper Patient Identification device (KPPI) is to be used each time to identify a patient prior to specimen collection. - Do not bring phlebotomy cart into the room |
| --- | --- |

| ##### Procedure | - Follow the steps below on KPPI Hand Held Infection Prevention Step; Action. 1; Take phlebotomy cart to the entrance of the patient’s room on isolation. Do not take the cart into the room.. 2; Prior to entering the room, use hand gel sanitizer.. 3; Don the appropriate PPE prior to entering the room.. 4; - Place the KPPI handheld scanning device in a clean plastic bag and prepare all supplies needed for the phlebotomy procedure: tourniquet, needle, needle holder, gauze, alcohol prep, tape, and required specimen tubes.. 5; - Perform the phlebotomy according to phlebotomy procedure LPhM 200 Performing Venipuncture at KP LAMC Hospital. 6; - Discard sharps in sharps container.. 7; - Discard tourniquet, gauze, and needle cap in waste receptacle.. |
| --- | --- |

Continued on next page

#### KPPI HAND HELD INFECTION PREVENTION FOR ISOLATION ROOM, Continued

| ##### Procedure, Continued | Step; Action. 8; After using the device, without passing the threshold, remove KPPI scanner from plastic bag by letting it slide out onto the “clean” surface of phlebotomy cart while not allowing the plastic bag to come in contact with the surface and place specimens on one end of the phlebotomy cart counter. Discard plastic bag in waste receptacle.. 9; Remove gown and gloves, and / or surgical mask just inside the threshold of the room. Exception Airborne isolation: remove and discard N-95 mask outside the isolation room.. 10; Sanitize hands using foam or gel hand sanitizer or soap and water. Exception: after exiting Contact Plus isolation room must use soap and water to clean hands.. 11; Disinfect phlebotomy cart counter with sanicloth.. |
| --- | --- |

| Controlled Document(s) | The table below lists the controlled document(s) for this policy. Document Number; Document Name. LGM 8000; Standard Precautions and Safety Practices in the Laboratory. LGM 8004; Infection Control. LGM 8005; Universal Body Substance Precautions. LGM 8006; Handling of Regular and Infectious Waste. LGM 8007; Cleaning Work Areas. LGM 8010; Proper Hand-Washing. LPhM 202; In-patient Identification. Regional Policy; Adverse Reaction in Phlebotomy. LPhM 213; Specimen Collection one label – one tube policy. |
| --- | --- |

End of page