#### USE OF BD VACUTAINER URINE TUBES FOR URINE TESTING – MOB LABS

| ##### Purpose | The purpose of this procedure is to provide transfer and labeling instructions for BD vacutainer Urine tubes for urine testing in the Outlying and Medical Office Building Laboratories. |
| --- | --- |

| ##### Policy | - The acceptable minimum volume is 7ml. - If urine received is less than 7 ml and has tests other than urine analysis: Shared specimen to LAMC; Shared specimen to SWL. If shared with drug screen test – send urine specimen in urine cup as – SHARED Specimen. All labels should be included.; If shared specimen with C&S, chlamydia, etc, transfer specimen into required container, accordingly.. |
| --- | --- |

| ##### Workplace safety | All laboratory employees are expected to maintain a safe working environment and an injury-free workplace. Laboratory employees are responsible for their own safety, the safety of others and adhering to all departmental and medical center safety policies and procedures. - For standard precautions and safety practices in the laboratory; see LAMC-PPP-0123, specifically, but not limited to, equipment safety, proper body mechanics, sharps exposure and proper use of personal protective equipment (PPE). - For Universal Body Substance precautions, see LAMC-PPP-0128, specifically, but not limited to, exposure to body fluids. - For proper hand washing, see LAMC-PPP-0132, specifically, but not limited to, proper hand washing. - For proper infection control, see LAMC-PPP-0127, specifically, but not limited to, proper use of gloves. - For proper handling of regular and infectious waste, see LAMC-PPP-0129, specifically, but not limited to proper disposal of regular and biohazardous waste. - For proper cleaning of work area, see LAMC-PPP-0130 - Cleaning Work Areas. - For proper handling of chemicals and reagents, see the Chemical Hygiene Plan. For proper storage and disposal of chemical hazardous waste.. |
| --- | --- |

Continued on next page

#### USE OF BD VACUTAINER URINE TUBES FOR URINE TESTING – MOB LABS

| ##### Procedure | Transfer all urine samples intended for urine analysis into BD vacutainer tube. Follow the steps outlined in the table below. |
| --- | --- |

| Step | Action |
| --- | --- |
| 1 | Accession specimens, accordingly. |
| 2 | Transfer specimens within 1-2 hours of collection. If transfer cannot be done within1-2 hours of collection refrigerate specimen to avoid bacterial growth. Note: If specimen has less than 7ml, send specimen in urine cup instead. |
| 3 | Label urine tube with accession number Make sure that label is placed straight and that level of urine can be seen. |
| 4 | Open urine container, and submerge transfer straw aseptically into urine specimen. You may tilt the container at an angle if urine volume is limited. |
| 5 | Place BD urine tube into the holder with the stopper down. Push the tube down over the puncture point to pierce the stopper. Note: If specimen is shared with C&S order, C&S has to be done first before this procedure. |
| 6 | Hold in position until urine stops flowing into the tube. (Minimum fill – 7 ml / Maximum fill- 8 ml) |
| 7 | Remove the tube from the transfer device, and shake vigorously to ensure complete dissolution of the preservative. |
| 8 | Dispose the transfer straws into the sharps container. |

Continued on next page

#### USE OF BD VACUTAINER URINE TUBES FOR URINE TESTING – MOB LABS

| ##### Non controlled documents | BD Vacutainer Product Information pamphlet, 2003 |
| --- | --- |

| ##### Controlled documents | See list below |
| --- | --- |

| # |  |
| --- | --- |
| LAMC-PPP-0123 | Standard precautions and safety practices in the laboratory |
| LAMC-PPP-0128 | Universal Body Substance precautions |
| LAMC-PPP-0132 | Proper hand-washing |
| LAMC-PPP-0127 | Infection control |
| LAMC-PPP-0130 | Cleaning Work Areas |
| LAMC-PPP-0134 | Proper storage and disposal of chemical hazardous waste |

| ##### Author | Rowena B. Pablico, CLS |
| --- | --- |