#### REAGENT LABELING AND STORAGE

| ##### Purpose | To establish guidelines for the labeling and proper storage of reagents, stains and solutions. |
| --- | --- |

| ##### Policy | All reagents, stains and solutions will be labeled with the following: - If prepared in-house, date of preparation and initials of preparer - If prepared commercially, date of receipt and opening (or reconstitution) and lot number - Identity of the substance - Titer, strength or concentration - Recommended storage temperature - Expiration date - Cautionary/accessory information, if any, related to handling and quality control - Carefully check all reagents, chemicals, controls, calibrators, stains, inks, and solutions prior to use. Reagents, stains and solutions will be stored in accordance with manufacturer's instructions. Reagents, stains, solutions, chemicals, and media are used within their indicated expiration dates. Flammable or explosive reagents, stains and solutions will not be stored in quantities exceeding a two-day supply. Such supplies will be stored in the red and yellow flammable storage cabinets only. Unrefrigerated reagents, stains and solutions will not be stored less than 4 inches from the floor or less than 18 inches from the ceiling. Reagents, stains and solutions will not be stored with acids. When transferring from primary to a secondary container, secondary labels must contain safety hazard information and logos, the date the product was opened and expiration date if applicable. Components of a reagent kit must be used only within the kit lot unless otherwise specified by the manufacturer. |
| --- | --- |

REAGENT LABELING AND STORAGE, Continued

CLS, MLTs, and PAs are responsible for labeling reagents, QC materials, stains, solutions and other materials as appropriate and/or as required by regulatory requirements.

The Area Laboratory Manager or Supervisor will be responsible for routine audits to check that reagents, stains and solutions are labeled and stored properly.

- Reagents transferred from one container to another will be labeled with the same information as the original container.

The individual who transfers a reagent from one container to another shall be responsible for ensuring that the secondary container is labeled properly

If NO expiration date is indicated by the manufacturer for the reagent, chemical or solution, then the reagent, chemical or solution should be evaluated for its reactivity on an annual basis as needed.

- Assess reagents, chemical, controls, calibrators, stains, inks and solutions for:

- Frequency of use

- Stability

- Storage condition

- Label readability

- Check if reagents, chemical, controls, calibrators, stains, inks and solutions show signs of deterioration, discoloration, contamination or humidity absorption.

- Reagents, chemical, controls, calibrators, stains, inks and solutions without established expiration date will be disposed of one (1) year after opening.

End of page