- WHITE CELL DIFFERENTIALS, MANUAL

| - PRINCIPLE | A blood smear is stained with Wright’s Stain and then examined microscopically. One hundred white blood cells are counted and differentiated as to classification. White cell, red cells and platelets are examined and any abnormal findings are noted and reported. |
| --- | --- |
| - METHOD | Manual |
| - MATERIALS & REAGENTS | Hema-Tek Stainer Sysmex SP-1000i Hema-Tek Stain Pak Phosphate Buffer (pH 6.6-7.2) Slides Harleco-Wright’s Stain Methanol-Anhydrous De-ionized water NOTE: Waste from these reagents are hazardous and should be accumulated in the appropriate container (Stains, Alcohols). |

PROCEDURE

- Prepare a stained blood smear using Sysmex SP1000i procedure (see Hematology P&P 04-009-01). Or prepare a blood smear as described in the policy for Preparation of Blood Films (see Hematology P&P 01-130-01) and stain the smear with Wright’s Stain using the Hema-Tek Slide Stainer.

- Perform a White Blood Cell Differential Count using the CellaVision (see Hematology P&P 04-011-01) or by regular microscope.

- WBC differentials should always be reported in per cent.  The total reported should always equal 100%.

- If the total WBC is 1,000 or greater, 100 cells should be counted.  The number counted should be reported directly as percent.

- If the total WBC is less than 1,000 but greater than 500, 50 cells should be counted.  The cells counted should be multiplied by 2 and reported as percent.  It should be noted in the comment section that the differential is based on 50 cells.

- If the total WBC is 500 or less, 25 cells should be counted. The cells counted should be multiplied by 4 and reported as per cent.  It should be noted in the comment section that the differential is based on 25 cells.

- If the required number of cells cannot be found, a second blood smear should be made and the WBC’s counted until the required number is found.

- Differentiation of white blood cells.

Polymorphonuclear neutrophils (Segs): These cells are recognized by the presence of a thin filament connecting at least two lobes of nuclear material.  The filament is composed of apposition of two layers of nuclear membrane.  No recognizable chromatin is present in the filament.

Band: A typical band or “stab" cell has a recognizable nuclear indentation more than one-half the diameter of the theoretical circular nucleus.  Recognizable material is present in the connecting bridge and there can be variable extent of parallel margins of nuclear membrane.  This means that any granulocyte having an indentation greater than one-half the diameter of the nucleus with some chromatin present causing a thickened connecting strand should be identified as a band cell.

Metamyelocyte:  A granulocyte is considered a metamyelocyte if nuclear indentation is less than one-half the diameter of the nucleus or parallelism of the constricted side is not present.

Folded cells and cells with nuclei folded upon themselves so that the entire nuclear outline is not visible should be identified as a “poly" so long as the cytoplasmic criteria for neutrophilic granulocyte are fulfilled.

Lymphocyte: All normal and atypical lymphocytes should be reported as the total lymphocyte per cent.  Atypical lymphocytes should be reported as a % of the total lymphocyte count. This per cent is calculated:

Atypical lymphocytes / total lymphocytes = %

Blast:  Large, round to oval cells, 10-20 um in diameter and nuclear to cytoplasmic ratio is high varying from 7:1 to1:1. There are 3 types of blast cells. All of them have central nuclei with fine, uncondensed chromatin and prominent nucleoli.

- Type1 blast= lack cytoplasmic granules

- Type 2 blast= have small number of primary (azurophilic ) granules.

- Type 3 blast= are similar to type 1 with more abundant azurophilic granules.

All smears with blasts (unknown cases) or unidentifiable cells must be sent to the Pathologist for review.  The pathologist will review the next workday or immediately if paged by the provider.

All Other Types: All other types of leukocytes should be differentiated and reported as described in: "The Morphology of Human Blood Cells" L.W. Diggs, MD, Dorothy Strum, and Ann Bell. Abbott Laboratories.

NOTE: Cell and parasite identification can be difficult.  If you have any doubt of the correct identification, you may consult your co-worker, supervisor, or pathologist for assistance.

- Microscopic procedure:

Inspect smear under low power.  Observe the distribution of leukocytes and choose that portion of the smear, usually near the thin end, where there is no overlapping of erythrocytes. Apply a layer of oil to slide. Shift to 40X or 50X objective.

Move the slide from the extreme upper edge of the smear to the extreme lower edge, counting and classifying each leukocyte in the successive fields.  Shift over one field and proceed to the upper edge, still classifying each leukocyte.  Continue in this fashion until the required number of cells is counted.

- WBC abnormalities: Any WBC abnormalities seen should be reported.  These should include toxic granulation, hypersegmentation and Dohle bodies.

- WBC estimation:  An estimation of the total WBC count should be made from the smear and compared to instrument or manual count as follows:

No/High-Power Field	  Estimated Count

2     -	4		4,000     -  7,000
	4     -	6		7,000	- 10,000
	6     -	10		10,000	- 13,000
	10   -	20		13,000	- 18,000

- RBC morphology:  RBC morphology should be examined and reported as normal or if abnormal the type of abnormality should be reported qualitatively.

Abnormalities in size:	Anisocytosis

Microcytosis

Macrocytosis

Abnormalities in shape:	Discocyte	Biconcave disk	Echinocyte

Crenated cell 	Stomatocyte  	Acanthocyte
Codocyte	Target cell 	Dacryocyte

Teardrop cell	Drepanocyte	Sickle cell
Elliptocyte	Ovalocyte	Schistocyte

Burr cell

Inclusions:	Howell-jolly bodies		Cabot rings

Heinz bodies		Siderotic granules

Basophilic stippling		Polychromasia

Hypochromasia		Hyperchromasia

The following terms should be used when reporting RBC morphology:

| Slight | 2-5cells/100x |
| --- | --- |
| Few | 5-10 cells/100x |
| Moderate | 10-15 cells/100x |
| Marked | >15 cells/100x |

- Platelets:  Platelets should be estimated and reported if the instrument does not print out a platelet result.  If a result is printed it should be verified by the slide estimate. Abnormal platelets should be reported semi-qualitatively.  If slide estimate does not agree with automated result, platelet clumping in EDTA is suspected. Do platelet count on citrate. Multiply x 1.1 and report “platelet count done on citrate”. See Policy 01-200-01. The platelet estimate should be reported as follows:

| Increased | More than 25 platelets/100X or >400,000 |
| --- | --- |
| Adequate | Less than 25 platelets/100X but greater than 7 platelets/l00X or 130-400,000 |
| Decreased | Less than 7 platelets/l00X or <130,000 |

- Refer to Hematology policy and procedure 01-140 for slides that need to be referred to a pathologist.

| REFERENCE | Technical Hematology Arthur Simmons, 2nd edition, J.B. Lippencott Company, Philadelphia. p.103. Laboratory Medicine Hematology, John B. Miale, 6th edition, O.V. Mosby Company, St. Louis. p.475, 869. American Journal of Clinical Pathology, Committee for Clarification of the nomenclature of Cells and Diseases of the Blood and Blood Forming Organs: second report, 56:19 (1949). Technical Improvement Service, What is a Band', Thomas F. Deutcher, MD., Commission on Continuing Education of the Society of Clinical Pathologists, No. 15 (1973) pg. 10-19. Pediatric Reference Ranges, 1995, S. Soldin, J. Hicks, Editor, AACC Press. |
| --- | --- |

Document History Page

| Change type: New, Major, Minor etc. | Changes Made to SOP – describe | Signature responsible person/date | Med. Dir. Reviewed/ Date | Lab Manager reviewed/ date | Date change Imp. |
| --- | --- | --- | --- | --- | --- |
| Minor | Page 2 under Blasts: clarification of pathologists review of unidentifiable cells | Debbie Chantry 6-22-10 |  |  |  |
| Minor | 1) Updated format. 2) Updated materials and reagent section. 3) Updated Procedure: Step # 1. Preparation of blood smears using Sysmex SP1000i procedure for automation is added. Step # 2. Option using CellaVision P&P is added. 4) Remove reference range portion on page 4. | Julius Salomon 5/28/14 |  |  |  |

Imp. =Implemented