- EOSINOPHIL COUNT, AUTOMATED

| - PRINCIPLE | Eosinophil counts are calculated from the automated CBC result utilizing the white blood count and eosinophil percentage. |
| --- | --- |

| - MATERIALS & REAGENTS | Hematology Analyzer |
| --- | --- |

| - SAFETY | All specimens should be handled as though capable of transmitting infectious diseases. Wear appropriate personal protective equipment when running patient samples or performing scheduled maintenance. Refer to: Policy and Procedures Safety Manual Infection Control and Procedures 11-030-01 |
| --- | --- |

PROCEDURE

| Step | Action |
| --- | --- |
| 1. | Perform Automated CBC per procedure. |
| 2. | Multiply the WBC X Eosinophil percentage. Then multiply the result by 1000 to convert to cu/mm. |
| 3. | Write your results in the manual hematology log under Eosinophil count. |
| 4. | Results are manually entered in Cerner under the Accession Result Entry (ARE) mode. For detailed instructions of entering results, please refer to Laboratory Informatics – Cerner Genlab Policies & Procedures Manual, “Resulting in Cerner GenLab: Manual Entry” LIS.SCPMG.041 document. |

For example, if you have a WBC of 6.0 x 10*3 and an Eosinophil result of 3%        then you would calculate as follows:

WBC (6.0) x Eosinophil percent (.03) = 0.18.

Multiply this result by 1000 to convert the result to 180 cumm.

| - REFERENCE - RANGE | 150-300/cumm |
| --- | --- |

Document History Page

| Change type: New, Major, Minor etc. | Changes Made to SOP – describe | Name of responsible person/date | Med. Dir. Reviewed/ Date | Lab Manager reviewed/ date | Date change Imp. |
| --- | --- | --- | --- | --- | --- |
| Major | Updated format, added the safety and materials/reagents section, added #3 & 4 in procedure to reflect LIS change. | Julius Salomon 5/12/2014 |  |  |  |

Imp. =Implemented