Department of Veterans Affairs Lexington, KY

Chemical Hygiene/Hazardous Communication Plan

Pathology and Laboratory Medicine Service

11/21/2013

Pathology and Laboratory Medicine Service Chemical Hygiene/Hazardous Communication Plan

| Prepared by: | Vicki B. Richardson |  | Authorized by: | Dr. Bonnie Mitchell |
| --- | --- | --- | --- | --- |
| Title: | Lab Safety Officer |  | Title: | Chief PLMS |
| Date Prepared: | 12/9/09 |  | Date Authorized: | 12/31/2009 |

| Review Authority: | Chief PLMS |
| --- | --- |
| Review Authority: | Lab Safety Officer |

| Reviewed (Date) | ###### By |  | Revised (Date) | ###### By |
| --- | --- | --- | --- | --- |
| 12-31-2009 | Dr. Bonnie Mitchell |  | 11/19/2010 | Vicki Richardson |
| 11/23/2010 | Dr. Bonnie Mitchell |  | 1/27/2012 | Vicki Richardson |
| 11/16/2011 | Dr. Luis Samayoa |  | 11/21/2013 | Vicki Richardson |
| 1/27/2012 | Dr. Luis Samayoa |  |  |  |
| 8/28/2012 | Dr. David Hunt |  |  |  |
| 11/21/2013 | Dr. David Hunt |  |  |  |

Table of Contents

I.	PURPOSE	5

II.	POLICY	5

III.	SCOPE	5

IV.	RESPONSIBILITIES	6

V.	HAZARD DETERMINATION	9

VI.	LOCATION	10

VII.	PROCEDURES	10

A.	Purchase Request and Acquisition	10

B.	Storage	10

C.	Hazardous Chemical Delivery	12

D.	Hazardous Substance Labeling	12

E.	Components of Chemical Hazard Labels and their Meanings	14

F.	Non-routine Tasks	21

G.	Chemical Inventory	21

H.	SDS Files	21

I.	Hazard Analysis Documentation	22

J.	Hazardous Chemicals in Our Laboratory	23

K.	Standard Operating Procedures for Select Hazards	33

L.	Accident Reporting	35

M.    Training	36

N.	Contractor Hazard Information Exchange	36

O.	Record Maintenance	37

P.	Chemical Spill Response	37

Q.	Mercury disposal/spills	37

R.	Chemical Waste Disposal	38

S.	Monitoring for Exposure	40

T.	Medical Surveillance	42

U.	Emergency First Aid Procedures	42

V.	Policy regarding medical evaluation of employees who habitually demonstrate unsafe work practices.	43

W.    Personal Protective Equipment	43

VIII.	References	45

Attachment A:  The OSHA Laboratory Standard (29 CFR 1910.1450)	46

Attachment B:  Memorandum 138-07 Hazard Communication Program	65

Attachment C:  Hazard Analysis Worksheet	80

Attachment D:  Request for a VA Chemical Pickup Procedure	82

Attachment E:  Memorandum No. 138-16 Hazardous Waste Managament	86

Attachment F:   NIOSH Carcinogen List	90

Attachment G: 29 CFR Part 1910, Subpart Z, Toxic and Hazardous Substances, OSHA	93

Attachment H:  International Agency for Research on Cancer (Groups 1, 2a, and 2b)	98

Attachment I:  National Toxicology Program Report on Carcinogens (RoC)-12th RoC (2011)	115

Attachment J:  National Toxicology Program 12th Report on Carcinogens "Reasonably anticipated to be human carcinogens"	117

Attachment K:  Target Organ Poster	121

Attachment L:  Chemical Incompatibility Chart	122

Attachment M:  Using CEOSH to pull up SDS’s	124

Attachment N:  Printing Chemical labels using the 3E SDS Library	129

Attachment O:  Formaldehyde VA Medical Center Lexington, KY  SOP-ENG-138-12 (Formerly Memorandum 138-31)	130

Attachment P:  Standard, 29CFR 1910.1048 and Appendices A, C, D, and E	134

Attachment Q:  Formaldehyde OSHA Fact Sheet	149

# PURPOSE

The purpose of the written hazard communication (hazcom) program is to establish policy, assign responsibility, and outline procedures for ensuring that the transmittal of information regarding hazardous chemicals is accomplished by means of a comprehensive hazard communication program, which includes container labeling, Safety Data Sheets (SDS), and employee training.  This will ensure and promote a safe work environment and protect employees, patients, and visitors from exposure to hazards by educating laboratory workers about the health hazards associated with hazardous chemicals.

# POLICY

This written hazard communication program complies with OSHA requirements and ensures employees are effectively informed concerning potential and existing chemical hazards on a continuous basis and protects them from unsafe practices and health hazards associated with hazardous chemicals in the laboratory.  Through education and information on the risks posed by chemicals on the job, workers will be better prepared to handle materials, prevent overexposure and adverse health effects and respond to emergencies.  P&LMS will strive to keep exposure limits below permissible limits specified in 29CFR1910 Subpart Z and will comply with pertinent State and local regulations.

The major elements of this program are as follows:

- Labels and other forms of warning

- Safety Data Sheets (SDS) from suppliers

- Safety Data Sheet Automated Library: e-MSDS through the Center for Engineering and Occupational Safety and Health (CEOSH)

- Employee information and training

- List of hazardous chemicals known to be present in the workplace

- Methods for informing employees of hazards of non-routine tasks

- Methods for informing contractor employers of hazards their employees may be exposed to while working in the laboratory

The Hazard Communication Program applies to chemicals in the work place that may pose a hazard to employees under normal conditions when performing non-routine tasks or responding to foreseeable emergencies. This Hazard Communication Program relies on Safety Data Sheets (SDS) from manufacturers and suppliers for the purpose of hazard determination.

# SCOPE

To provide the necessary training to laboratory personnel regarding safe work practices, hazardous chemicals and materials to which they may be exposed, by means of safety training, a hazard communication program, product labeling, safety data sheets, training and monitoring compliance of the personnel.

# RESPONSIBILITIES

- The Medical Center or Hospital Director

Ensure that laboratory employee exposures to OSHA regulated substances do not exceed the permissible exposure limits specified in 29CFR1910 Subpart Z

Provide a safe workplace environment for all employees

Implement an effective safety program which meets the requirements of the various accrediting agencies

Provide a medical surveillance program through the Employee Health Section for employees exposed to chemicals at or above the action level or PEL (permissible exposure limits) for any OSHA-regulated substance.

- The Facility Safety Manager or Industrial Hygienist

- Evaluate the content and effectiveness of the provisions of the P&LMS Infection Control and the Chemical Hygiene/Hazard Communication Plan

Monitor exposure to hazardous chemicals, such as formaldehyde and xylene, on a regular basis

Develop and review of a hazard communication program

Designated as SDS Coordinator

- Duties include maintaining the Medical Center SDS file and the Medical Center chemical inventory

Provide a comprehensive hood maintenance program to ensure proper and adequate performance

Coordinate and monitor the disposal of hazardous chemicals

- The Chief Pathologist, Pathology & Laboratory Medicine Service and the Laboratory Manager

- Provide a safe workplace environment for the employees and volunteers within the service

Enforce the medical center/hospital policy and all relevant procedures regarding employee safety, and volunteers performing tasks within the PLMS ensuring that the requirements of various accrediting agencies are fulfilled

Provide the designated laboratory safety officer with necessary support in obtaining resources and personnel to implement the Infection Control and Chemical Hygiene Plan

Ensure that corrective action is taken for any problems/deficiencies identified

- Monitor compliance within the laboratory

- Implement a Hazard Communication Program within Pathology and Laboratory Medicine Service including the additional requirements contained in the Federal Regulation 29 CFR1910.1450, “Occupational Exposure to Hazardous Chemicals in Laboratories.”  Implementation will also include maintaining a chemical inventory, service specific SDS files, and ensuring that training is received/conducted and documented according to VA MEDICAL CENTER MEMORANDUM 138-07 “HAZARD COMMUNICATION PROGRAM.”

- The designated Laboratory Safety Officer

- Serve as HazCom Service Coordinator (HCSC) and Chemical Hygiene Officer

- Develop policies and procedures for operations that involve hazardous chemicals

Review and evaluate the Infection Control and Chemical Hygiene Plan and Hazardous Communication Plan annually

Educate and train employees in procedures relating to handling of hazardous chemicals

Monitor and enforce compliance with P&LMS and hospital/medical center policies

Document compliance with safe work practices via the Quality Improvement Program of P&LMS

Maintain a current and detailed inventory of all chemicals used in the laboratory and forward an electronic listing to the Safety Office/SDS Coordinator (138CDD)

Review all accident reports and ensure proper documentation

Serve as Chairman of the Laboratory Safety Subcommittee

Perform annual evaluation of the effectiveness of the Laboratory Safety Infection Control and Chemical Hygiene Plan by reviewing accidents, checking for patterns of accidents, prevention efforts, and presenting this information in the form of graphical analysis as related to the employee’s work section and type of accident

Thoroughly investigate incidents or unsafe conditions concerning laboratory safety and chemicals, and assuring prompt action is taken to prevent recurrence

Ensure training of employees with regard to laboratory safety in general and observation of Medical Center safety policies and procedures

Perform monthly inspections of P&LMS and assure compliance of the facility’s safety program.

- The Section Supervisor/Lead Technologist

- Maintain a current Hazardous Chemical List which will be placed at the beginning of the MSDS book. The Hazard Chemical List will serve as an index and will assist in located a SDS quickly. Employees may request a copy of a SDS if they wish.

- Make the chemical inventory and SDS’s accessible to employees during each work shift

- Conduct specific training on the use of hazardous chemicals used in the workplace, including the use of appropriate protective equipment

- Make sure that labels and other forms of warning are not damaged or removed from chemical containers.

- Ensure that the labels that are damaged or removed from chemical containers are relabeled appropriately

- Ensure that chemicals that are prepared or diluted in-house are labeled appropriately

- Ensure that all laboratory work practices comply with service specific policies, as detailed in this plan

Enforce safe work practices and the use of personal protective equipment within the assigned section

Display the appropriate signage, for all areas in which hazardous chemicals are being stored or handled

Assist the designated Laboratory Safety Officer in ensuring that all employees receive adequate training and that this training is documented in the employee’s training record

Implement appropriate corrective action when problems or deficiencies are identified

Advise employees of safe medical treatment or first aid should the need arise

Investigate all reported accidents within the assigned section

Ensure completion of VA Form 2162, "Report of Accident" and the CA-1 U.S. Department of Labor form for each employee reported accident

Coordinate the entry of employee reported accident data into the ASISTS program

Ensure that those employees assigned non-routine tasks are informed of any hazards associated with infrequently used or unusual chemicals

Update the chemical inventory in January and July of each year

- Each employee

- Follow established policies and procedures designed to protect employees

Recognize potential safety hazards and reporting them to appropriate supervisors in a timely manner

Report all incidents or accidents so that steps may be taken to prevent their recurrence

Complete mandatory training assignments in a timely manner

Review the P&LMS Hazardous Communication and Chemical Hygiene Plan as needed

Safe use, handling and labeling of all chemicals and materials, and for becoming familiar with properties of the hazardous chemicals in their work area including those not routinely used

# HAZARD DETERMINATION

- For the purpose of this policy, the term “hazardous” is a physical or health hazard that is listed as such in either:

  - 29 CFR Part 1910 subpart Z, Toxic and Hazardous Substances, OSHA

Threshold Limit Values for Chemical Substances and Physical Agents in the Work Environment (latest edition), American Conference of Governmental Industrial Hygienists (ACGIH)

  - A “hazardous substance” is regarded as a carcinogen or potential carcinogen if it is identified as such by: Chemicals that must be handled as potential carcinogens include those defined by OSHA as "select carcinogens."  OSHA defines select carcinogens as any substance that is:

- Regulated as a carcinogen by OSHA, has been classified as "known to be carcinogenic" by the NTP, or listed as a group I carcinogen by the IARC

- Has been classified as "reasonably anticipated to be carcinogenic" by the NTP or listed as a group 2A or 2B carcinogen by the IARC

Manufacturers, importers and distributors will be relied upon to perform the appropriate hazard determination for the substances they produce or sell.

The following materials are not covered  by the Hazard Communication Standard:

  - Any hazardous waste as defined by the Solid Waste Disposal Act, as amended by the Resource Conservation and Recovery Act (RCRA) of 1976, as amended (42 USC 6901 et seq.) when subject to regulations issued under that act by the Environmental Protection Agency.

  - Tobacco or tobacco products

  - Wood or all wood products except wood dust

  - Consumer products (including pens, pencils, adhesive tape) used in the work place under typical consumer usage.

  - Foods, drugs, or cosmetics intended for personal consumption by employees while in the workplace.

# LOCATION

- Paper copy of the Hazardous Communication Plan will be kept on the bookshelf outside of B112, Room A606 and the outpatient phlebotomy areas.

- 2.  An electronic copy of the Hazardous Communication Plan will be kept on the P&LMS SharePoint site at this link: https://vaww.v09.r03.portal.va.gov/sites/lexvamc/pathology/DocumentReferences/Chemical%20Hygiene%20-%20Hazardous%20Communication%20Plan.doc

# PROCEDURES

## Purchase Request and Acquisition

- All chemicals must be purchased through normal purchasing and distribution channels.

- When any chemical or chemical product is purchased initially, the Service Chief or his/her designee will ensure that the words, “Hazardous: - SDS required”, are included in the description of all purchase requests.  This requirement applies to procurement requests for A&MMSL.

- The Acquisition and Material Management Service Logistic Manager (A&MMSL) will ensure that the hazardous material requirements in the Federal Acquisition Regulation (FAR) Subpart 23.3 are contained in the acquisition document if the purchase request includes this requirement.

- The Contracting Officer is responsible for utilizing the technical assistance of the Industrial Hygienist (IH) for assuring the SDS meets the requirements of 29 CFR 1910.1200(g). If the item requested is a new hazardous material/chemical not currently on the service inventory, the Industrial Hygienist must be informed prior to acquisition for evaluation.

- If the Contracting/Purchasing Agent receives a SDS, it will be sent to the Warehouse or Logistics.  When the hazardous material or chemical is delivered, the SDS will be given to the using service with the chemical. If the chemical is received and the SDS has not been included, the Warehouse or Logistics will notify the Contracting Officer/Purchasing Agent and hold the chemical until the SDS is received or upon notification from the Industrial Hygienist to deliver. The Industrial Hygienist may allow immediate delivery if the SDS for the specific hazardous material manufactured by the same manufacturer is already on file.

## Storage

- Flammables

  - Small quantities (working amounts) may be stored in the laboratory. These quantities should be kept to a minimum and containers must be clearly labeled.  Exposure to heat or direct sunlight shall be avoided.  Bulk storage (5 gallons or more) must be stored in an approved flammable storage cabinet.

  - Storage of flammable liquids outside approved flammable storage cabinets and safety cans must not exceed 10 gallons per 100 square feet of laboratory space (This includes waste).

  - The transfer of flammables from one metal container to another metal container should be preceded by electrical bonding in order to avoid a static spark that may result from pouring the solvent.

  - Safety cans are used instead of glass bottles for volumes of flammable solvents larger than one quart if the purity required does not mandate glass storage.

  - Stored chemicals shall be examined annually for replacement, deterioration, and container integrity.

  - Chemicals shall be stored by reactive class (i.e. flammable with flammables).  Incompatible chemicals must be segregated from each other in storage. Acids and Bases should be stored separately near floor level.

## Hazardous Chemical Delivery

- When a hazardous chemical or chemical product is received with its SDS, the SDS will be attached to the receiving report to be delivered with the chemical to the requesting service line.

If a hazardous chemical or chemical product is received without its SDS, the chemical will be delivered to the requesting service.  The Service Chief and/or HCSC must ensure that the chemical is not used until sufficient information is available to assure safe handling.  The Service Chief and/or HCSC must contact the manufacturer and request that the SDS be faxed or mailed or access the VHA – CEOSH website (http://vaww.ceosh.med.va.gov/default.htm).

The Logistic Manager, A&MMSL will be notified to ensure that the purchase contract includes the required request for the SDS.

## Hazardous Substance Labeling

- There should never be any unlabeled containers in the work area.

- Labels must be legible, in English, and prominently displayed. Other languages may be displayed in addition to English

All chemicals will be kept in properly labeled containers.  The original labels will not be removed or defaced.  If the original label is lost or becomes illegible, it will be relabeled or marked immediately with required information.  It is not required to label portable containers into which hazardous chemicals are transferred from labeled containers when they are intended only for the immediate use by the employee who performs the transfer.  For example, the pouring of a chemical from a stock bottle into a measuring container when preparing a solution.

OSHA has adopted new hazardous chemical labeling requirements as a part of its recent revision of the Hazard Communication Standard, 29 CFR 1910.1200 (HCS), bringing it into alignment with the United Nations’ Globally Harmonized System of Classification and Labeling of Chemicals (GHS). These changes will help ensure improved quality and consistency in the classification and labeling of all chemicals, and will also enhance worker comprehension. As a result, workers will have better information available on the safe handling and use of hazardous chemicals, thereby allowing them to avoid injuries and illnesses related to exposures to hazardous chemicals.

- From now until June 1, 2015, P&LMS will see several hazardous labeling and marking systems P&LMS will use the NFPA (National Fire Protection Agency)/HMIS® (Hazardous Material Identification System) and/or the GHS (Global Harmonizing System) labels for chemical labeling.  After December 1, 2015, chemical manufacturers, chemical receivers, and chemical purchasers will be required to use the GHS labels.

- NFPA/HMIS®

The NFPA/HMIS® label is a label that combines the NFPA and HMIS® classification onto a single label. This label will provide the following information:

- Product Identifier

- Supplier Identification

- Client Custom Values

- NFPA Rating

- HMIS® Rating

- GHS

GHS labels will provide the following information:

• Name, address and telephone number of the chemical manufacturer

• Product identifier

• Signal word

• Hazard statement(s)

• Precautionary statement(s)

• Pictogram

It is the responsibility of the supervisor to assure that each container of a hazardous substance in the laboratory (whether original, diluted or prepared in-house) is marked, labeled or tagged with the manufacturer’s label, an NFPA/HMIS label, or a GHS label.

If a substance is transferred from its original container into a portable container which is not labeled, the portable container must be labeled with an NFPA/HMIS and/or GHS label to identify the contents of the container.  All unattended containers shall be labeled.

Containers used by outside service contractors shall be properly labeled with a manufacturer’s label, an HMIS/NFPA label and/or a GHS label prior to the use of the hazardous substance.

Always regard unlabeled containers as dangerous!

All chemical solutions of 1% or greater must bear the same hazard warning label as the concentrated chemical.

  - Date received.

  - Expiration date, if any.

  - Date opened.

  - Storage requirements.

The P&LMS Safety Officer or a supervisor will print NFPA/HMIS or GHS labels on a color printer as needed.

## Components of Chemical Hazard Labels and their Meanings

After June 1, 2015, the OSHA HCS will require chemical manufacturers, importers, or distributors to ensure that each container of hazardous chemicals leaving the workplace is labeled, tagged or marked with the following information:

• Name, address and telephone number of the chemical manufacturer

• Product identifier

• Signal word

• Hazard statement(s)

• Precautionary statement(s)

• Pictogram

- Hazard Communication Standard (HCS) Labels (mandatory after June 1, 2015)

  - Name, Address and Telephone Number of the chemical manufacturer, importer or other responsible party.

  - Product Identifier

- This is how the hazardous chemical is identified. This can be (but is not limited to) the chemical name, code number or batch number. The manufacturer, importer or distributor can decide the appropriate product identifier. The same product identifier must be both on the label and in section 1 of the SDS.

  - Signal Words

- Signal words are used to indicate the relative level of severity of the hazard and alert the reader to a potential hazard on the label. There are only two words used as signal words, “Danger” and “Warning.” Within a specific hazard class, “Danger” is used for the more severe hazards and “Warning” is used for the less severe hazards. There will only be one signal word on the label no matter how many hazards a chemical may have. If one of the hazards warrants a “Danger” signal word and another warrants the signal word “Warning,” then only “Danger” should appear on the label.

  - Hazard Statements

- Hazard Statements describe the nature of the hazard(s) of a chemical, including, where appropriate, the degree of hazard. For example: “Causes damage to kidneys through prolonged or repeated exposure when absorbed through the skin.” All of the applicable hazard statements must appear on the label. Hazard statements may be combined where appropriate to reduce redundancies and improve readability. The hazard statements are specific to the hazard classification categories, and chemical users should always see the same statement for the same hazards no matter what the chemical is or who produces it.

  - Precautionary Statements

- Precautionary Statements describe recommended measures that should be taken to minimize or prevent adverse effects resulting from exposure to the hazardous chemical or improper storage or handling. For example, a chemical presenting a specific target organ toxicity (repeated exposure) hazard would include the following on the label: “Do not breathe dust/fume/gas/mist/vapors/spray. Get medical advice/attention if you feel unwell. Dispose of contents/container in accordance with local/regional/ national and international regulations.”  There are four types of precautionary statements:

    - prevention (to minimize exposure)

    - response (in case of accidental spillage or exposure emergency response, and first-aid)

    - storage

    - disposal

  - Pictogram

- Pictogram is a symbol plus other graphic elements, such as a border, background pattern, or color that is intended to convey specific information about the hazards of a chemical. Each pictogram consists of a different symbol on a white background within a red square frame set on a point (i.e. a red diamond). There are nine pictograms under the GHS. However, only eight pictograms are required under the HCS (OSHA Hazard Communication Standard).

- HCS Pictograms and Hazards

| Health Hazard | Flame | Exclamation Mark |
| --- | --- | --- |
| • Carcinogen • Mutagenicity • Reproductive Toxicity • Respiratory Sensitizer • Target Organ Toxicity • Aspiration Toxicity | • Flammables • Pyrophorics • Self-Heating • Emits Flammable Gas • Self-Reactives • Organic Peroxides | • Irritant (skin and eye) • Skin Sensitizer • Acute Toxicity (harmful) • Narcotic Effects • Respiratory Tract Irritant • Hazardous to Ozone Layer (Non Mandatory) |
| Gas Cylinder | Corrosion | Exploding Bomb |
| • Gases under Pressure | • Skin Corrosion/ burns • Eye Damage • Corrosive to Metals | • Explosives • Self-Reactives • Organic Peroxides |
| Flame over Circle | Environment (Non Mandatory) | Skull and Crossbones |
| • Oxidizers | • Aquatic Toxicity | • Acute Toxicity (fatal or toxic) |

- GHS Pictograms and Hazard Classes

| GHS Pictograms and Hazard Classes |  |  |
| --- | --- | --- |
| - Oxidizers | - Flammables - Self Reactives - Pyrophorics - Self-Heating - Emits Flammable Gas - Organic Peroxides | - Explosives - Self Reactives - Organic Peroxides |
| - Acute toxicity (severe) | - Corrosives | - Gases Under Pressure |
| - Carcinogen - Respiratory Sensitizer - Reproductive Toxicity - Target Organ Toxicity - Mutagenicity - Aspiration Toxicity | - Environmental Toxicity | - Irritant - Dermal Sensitizer - Acute toxicity (harmful) - Narcotic Effects - Respiratory Tract - Irritation |

- Transport  Pictograms

| Transport "Pictograms" |  |  |
| --- | --- | --- |
| Flammable Liquid Flammable Gas Flammable Aerosol | Flammable solid Self-Reactive Substances | Pyrophorics (Spontaneously Combustible) Self-Heating Substances |
| Substances, which in contact with water, emit flammable gases (Dangerous When Wet) | Oxidizing Gases Oxidizing Liquids Oxidizing Solids | Explosive Divisions 1.1, 1.2, 1.3 |
| Explosive Division 1.4 | Explosive Division 1.5 | Explosive Division 1.6 |
| Compressed Gases | Acute Toxicity (Poison): Oral, Dermal, Inhalation | Corrosive |
| Marine Pollutant | Organic Peroxides |  |

- Acute Oral Toxicity

| ACUTE ORAL TOXICITY - Annex 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Category 1 | Category 2 | Category 3 | Category 4 | Category 5 |
| LD50 | £ 5 mg/kg | > 5 < 50 mg/kg | ³ 50 < 300 mg/kg | ³ 300 < 2000 mg/kg | ³ 2000 < 5000 mg/kg |
| Pictogram |  |  |  |  | No symbol |
| Signal word | Danger | Danger | Danger | Warning | Warning |
| Hazard statement | Fatal if swallowed | Fatal if swallowed | Toxic if swallowed | Harmful if swallowed | May be harmful if swallowed |

- The Hazardous Materials Identification System (HMIS) is a numerical hazard rating that incorporates the use of labels with color-coded bars as well as training materials. It was developed by the American Coatings Association as a compliance aid for the OSHA Hazard Communication Standard.  The four bars are color-coded, using the modern color bar symbols with blue indicating the level of health hazard, red for flammability, orange for a physical hazard, and white for Personal Protection. The number ratings range from 0-4 (see 3a starting on the following page).

The HMIS label will provide the following information:

- Product Identifier

- Supplier Identification

- Client Custom Values

- HMIS® Rating

- The NFPA labeling system operates on the same principle as the HMIS (Hazardous Materials Identification System) labels.  Blue indicates health hazard, red indicates flammability, yellow indicates instability, and special information (such as what personal protective equipment to wear) will be provided in the white section.  It also uses a numerical system from 0-4 to indicate the severity of the hazards

The NFPA label will provide the following information:

  - Product Identifier

  - Supplier Identification

  - Client Custom Values

  - NFPA Rating

  - Client Note

- Key To Label Numerical Ratings

| HEALTH |  |
| --- | --- |
| 4 | Deadly: even the slightest exposure to this substance would be life threatening. Only specialized protective clothing, for these materials, should be worn. |
| 3 | Extreme Danger: serious injury would result from exposure to this substance. Do not expose any body surface to these materials. Full protective measures should be taken. |
| 2 | Dangerous: exposure to this substance would be hazardous to health. Protective measures are indicated. |
| 1 | Slight Hazard: irritation or minor injury would result from exposure to this substance. Protective measures are indicated. |
| 0 | No Hazard: exposure to this substance offers any significant risk to health. |

| FLAMMABILITY |  |
| --- | --- |
| 4 | Flash Point Below 73ºF and Boiling Point Below 100ºF: this substance is very flammable, volatile or explosive depending on its state. Extreme caution should be used in handling or storing of these materials. |
| 3 | Flash Point Below 100ºF: flammable, volatile or explosive under almost all normal temperature conditions. Exercise great caution in storage or handling of these materials. |
| 2 | Flash Point Below 200ºF: moderately heated conditions may ignite this substance. Caution procedures should be employed in handling. |
| 1 | Flash Point Above 200ºF: this substance must be preheated to ignite. Most combustible solids would be in this category. |
| 0 | Will Not Burn: substances that will not burn. |
| INSTABILITY |  |
| 4 | May Detonate: substances that are readily capable of detonation or explosion at normal temperatures and pressures. Evacuate area if exposed to heat or fire. |
| 3 | Explosive: substances that are readily capable of detonation or explosion by a strong initiating source, such as heat, shock or water. Monitor from behind explosion-resistant barriers. |
| 2 | Unstable: violent chemical changes are possible at normal or elevated temperatures and pressures. Potentially violent or explosive reaction may occur when mixed with water. Monitor from a safe distance. |
| 1 | Normally stable: substances that may become unstable at elevated temperatures and pressures or when mixed with water. Approach with caution. |
| 0 | Stable: substances which will remain stable when exposed to heat, pressure or water. |

The white space is reserved for indicating what personal protective equipment to use.  It also uses a numerical system from 0 – 4 to indicate the severity of the hazard.  This is by far the largest area of difference between the NFPA and HMIS systems. In the NFPA system, the white area is used to convey special hazards whereas HMIS uses the white section to indicate what personal protective equipment (PPE) should be used when working with the material.

- The NFPA/HMIS® label is a label that combines the NFPA and HMIS® classification onto a single label.

The NFPA/HMIS® label will provide the following information:

- Product Identifier

- Supplier Identification

- Client Custom Values

- NFPA Rating

- HMIS® Rating

## Non-routine Tasks

Anytime an employee is confronted with a task involving hazardous chemicals that is not of a routine nature, or is a totally unexpected situation, they are to report to their supervisor for instructions on safe procedures and the use of personal protective equipment before proceeding.  If the supervisor does not have sufficient information in the work areas, or within the organization, he/she must contact the Safety Office for assistance.

## Chemical Inventory

- The SDS Coordinator will maintain an inventory of hazardous chemicals used in the Pathology and Laboratory Medicine Department.

Each department supervisor will conduct a semiannual inventory of chemicals and forward an electronic listing to the P&LMS Safety Officer.  The P&LMS Safety Officer will review the inventories for completeness and then forward the inventory to the Safety Office/SDS Coordinator (138CDD).

The chemical inventories must include the common or trade name of chemical, manufacturer’s name, storage location, and disposal method.

Each chemical must be evaluated for reproductive toxicity, acute toxicity and carcinogenicity.

The inventory must be submitted in January and July of each year.

The chemicals in the inventory will use the identity that is referenced on the appropriate SDS.  In other words, the chemical inventory will be used as an index and filed with the SDS.

## SDS Files

- Each P&LMS service section will maintain a three ring binder specific to each section and an electronic file of SDS’s for hazardous chemicals used in the workplace.  SDS’s shall be readily accessible to all employees during each work shift.

Each department’s SDS’s will be updated semiannually when the chemical inventory is performed.

Each department’s SDS binder will be the same color.  P&LMS’s SDS binders will be yellow and black.

Each department’s chemical inventory list will be used as the index for their SDS binders.

When an SDS is not received at the time of the first shipment, contact the company of origin to obtain the SDS before using the chemical.

SDS’s may also be obtained through VHA’s Center for Engineering and Occupational Safety and Health (CEOSH) Service (See Attachment M for complete directions)

Where employees must travel between workplaces during a work shift, i.e. their work is carried out at more than one geographical location, the safety data sheets may be kept at the primary workplace facility.  In this situation, the employer shall ensure that employees can immediately obtain the required information in an emergency.

## Hazard Analysis Documentation

(See Attachment C for Hazard Analysis Worksheet)

- Assessment of individual workstations regarding identification and properties of hazardous chemicals

- Each workstation will have a hazard analysis performed and the Hazard Analysis Worksheet will be available to all employees who use each workstation.  Use of the Hazard Analysis Worksheet will ensure the employer is:

  - Providing employees with effective information and training on hazardous chemicals in their work area at the time of their initial assignment and whenever a new physical or health hazard is introduced into the work area that the employees have not previously been trained in.  Chemical-specific information must always be available through labels and SDSs.

  - Informing employees of any physical and health hazards of the chemicals in the work area.

  - Informing employees of methods and observations that may be used to detect the presence or release of a hazardous chemical in the work area (such as monitoring conducted by the employer, continuous monitoring devices, visual appearance or odor of hazardous chemicals when being released, etc.).

  - Informing employees of the measures employees can take to protect themselves from these hazards, including specific procedures the employer has implemented to protect employees from exposure to hazardous chemicals, such as appropriate work practices, emergency procedures and personal protective equipment to be used.

  - Work station assessment regarding personal protective equipment (PPE)

- Per OSHA 1910.132, the employer shall assess the workplace to determine if hazards are   present, or are likely to be present, and which necessitate the use of personal protective equipment (PPE).

- If hazards are present, or likely to be present, the employer shall select, and have each affected employee use the types of PPE that will protect the affected employee from the hazards identified in the hazard assessment

- The employer shall verify that the required workplace hazard assessment has been performed through a written certification (the Hazard Analysis Worksheet) that identifies the workplace evaluated; the person certifying that the evaluation has been performed; the date(s) of the hazard assessment; and, which identifies the document as a certification of hazard assessment.

- The employer shall provide training to each employee who is required to use PPE.  When PPE is necessary, each affected employee shall be trained to know at least when PPE is necessary; what PPE is necessary; and the proper care, maintenance, useful life and disposal of the PPE.

- Each affected employee shall demonstrate an understanding of the training received and the ability to use PPE properly, before being allowed to perform work requiring the use of PPE.

- The employer shall verify that each affected employee has received and understood the required training through a written certification that contains the name of each employee trained, the date(s) of training, and that identifies the subject of the certification.

## Hazardous Chemicals in Our Laboratory

- Flammables

| Location | Product Name | Manufacturer | NFPA Fire Rating |
| --- | --- | --- | --- |
| Cytology | Gleme Glass Cleaner | Ric Mar Industries, Inc. | 4 |
| Histology | Histoprep Mold Releasing Agent | Fisher Scientific | 4 |
| AncillaryTesting | Purell Instant Hand Sanitizer | GoJo Industries, Inc | 3 |
| AncillaryTesting | Super Sani-Cloth Germicidal Disposable Wipes | Nice-Pak Products Inc. | 3 |
| AncillaryTesting | ThinPrep PreservCyt Solution | Hologic | 3 |
| BloodBank | Purell Instant Hand Sanitizer Foam | GOJO Industries, Inc. | 3 |
| BloodBank | Purell Instant Hand Sanitizer with Moisturizers and Vitamin E | GOJO Industries, Inc. | 3 |
| BloodBank | Super Sani-Cloth Germicidal Disposable Wipes | Nice-Pak Products Inc. | 3 |
| BloodBank | Temp-Chex Red Spirit | Streck / Streck Laboratories, Inc. | 3 |
| Chemistry | Purell Instant Hand Sanitizer | GoJo Industries, Inc | 3 |
| Cytology | Clarifier I | Richard-Allan Scientific | 3 |
| Cytology | Cyto-Stain | Richard-Allan Scientific | 3 |
| Cytology | Denatured Alcohol 95% Histoprep | Fisher Scientific | 3 |
| Cytology | Hema 3 Fixative | Fisher Scientific | 3 |
| Cytology | Histoprep 100% Alcohol | Fisher Scientific | 3 |
| Cytology | Permount Mounting Media | Fisher Scientific | 3 |
| Cytology | Reagent Alcohol, 95% | Leica MicroSystems | 3 |
| Cytology | ThinPrep PreservCyt Solution | Hologic | 3 |
| Hematology | Buffered Wright Stain | Richard-Allan Scientific | 3 |
| Hematology | Hemoccult Developer (Part A) | Beckman Coulter, Inc | 3 |
| Hematology | Isopropanol | Fisher Scientific | 3 |
| Hematology | Isopropyl Alcohol 70% In Water | Fisher Scientific | 3 |
| Hematology | Lens Cleaner | Scientific Device Laboratory | 3 |
| Hematology | Methyl Alcohol | EKI Industries Inc. | 3 |
| Hematology | Permount Mounting Media | Fisher Scientific | 3 |
| Hematology | Purell Foaming Hand Sanitizer | GOJO Industries, Inc. | 3 |
| Histology | Acetone | Fisher Scientific | 3 |
| Histology | Alkaline Alcohol 50% | Poly Scientific R&D Corp. | 3 |
| Histology | Cytoseal XYL | Richard-Allan Scientific | 3 |
| Histology | Denatured Alcohol 95% Histoprep | Fisher Scientific | 3 |
| Histology | Envirocide Surface Disinfectant/Decontaminant Cleaner | Metrex Research Corporation | 3 |
| Histology | Fisherbrand Hema 3 Fixative | Fisher Scientific | 3 |
| Histology | Hema 3 Fixative | Fisher Scientific | 3 |
| Histology | Histoprep 100% Alcohol | Fisher Scientific | 3 |
| Histology | Isopropanol | Fisher Scientific | 3 |
| Histology | Lymph Node Revealing Solution | Poly Scientific R&D Corp. | 3 |
| Histology | Mayer's Mucicarmine Solution | Poly Scientific R&D Corp. | 3 |
| Histology | Methanol | Fisher Scientific | 3 |
| Histology | Micromount Mounting Medium | Leica MicroSystems | 3 |
| Histology | Permount Mounting Media | Fisher Scientific | 3 |
| Histology | Reagent Alcohol 100% | Leica MicroSystems | 3 |
| Histology | Reagent Alcohol, 95% | Leica MicroSystems | 3 |
| Histology | Weigerts Hematoxylin Solution A | Poly Scientific R&D Corp. | 3 |
| Histology | X8000 | Xennia Technology Ltd, UK | 3 |
| Histology | Xylenes, mixed isomers with ethylbenzene | Fisher Scientific | 3 |
| Histology | Xylenes, Purified | Leica MicroSystems | 3 |
| Microbiology | 70% Isopropyl Alcohol | Enturia | 3 |
| Microbiology | Acid-Ethanol | Alpha-Tec Systems Inc | 3 |
| Microbiology | Alcohol Prep Pad | Medline Industries, Inc. | 3 |
| Microbiology | Bactidrop Ninhydrin Reagent | Remel Inc | 3 |
| Microbiology | BactidropTM Desoxycholate (10%) | Remel Inc | 3 |
| Microbiology | Ethanol 70% | Alpha-Tec Systems Inc | 3 |
| Microbiology | Ethyl Acetate, GR | EMD Chemicals | 3 |
| Microbiology | Gram Decolorizer | BD Diagnostics | 3 |
| Microbiology | Methanol | Fisher Scientific | 3 |
| Microbiology | Methanol | Pharmco-AAPER | 3 |
| Microbiology | Microscan Reagent Q.C. Peptidate | Dade Behring Inc | 3 |
| Microbiology | Modified Trichrome Stain Set | Scientific Device Laboratory | 3 |
| Microbiology | NIN | BioMérieux, Inc. | 3 |
| Microbiology | Purell Hand Sanitizer | GOJO Industries, Inc. | 3 |
| Microbiology | Purell Instant Hand Sanitizer with Aloe | GOJO Industries, Inc. | 3 |
| Microbiology | Reagent Alcohol 100% | Leica MicroSystems | 3 |
| Microbiology | Reagent Alcohol, 95% | Leica MicroSystems | 3 |
| Microbiology | Sanihol 70 | Decon Laboratories Inc. | 3 |
| Microbiology | Substrate Solutions | BD Biosciences | 3 |
| Microbiology | TB Carbolfuchsin KF | BD Diagnostics | 3 |
| Microbiology | TB Methylene Blue | BD Diagnostics | 3 |
| Microbiology | VP2 | bioMerieux, Inc. | 3 |
| Microbiology | Webcol/Curity Alcohol Prep Pads | Kendall Healthcare/ Tyco Healthcare/ Covidien | 3 |
| Microbiology | Xpert EV | Cepheid USA | 3 |
| Microbiology | Xpert Flu | Cepheid | 3 |
| Phlebotomy | Chloraprep 2.0% | Enturia | 3 |
| Phlebotomy | ChloraPrep with Orange Tint | Enturia | 3 |
| Phlebotomy | Super Sani-Cloth Germicidal Disposable Wipes | Nice-Pak Products Inc. | 3 |
| Phlebotomy | Webcol/Curity Alcohol Prep Pads | Kendall Healthcare/ Tyco Healthcare/ Covidien | 3 |

Oxidizers

- An oxidizer is a substance that gives up oxygen easily, to stimulate combustion of organic material.

| Location | Product Name | Manufacturer |
| --- | --- | --- |
| Histology | Chromic Acid 5% Aqueous | Poly Scientific R&D Corp. |
| Histology | Phosphotungstic Acid Hematoxylin (PTAH) | Poly Scientific R&D Corp. |
| Histology | Potassium Permanganate | Fisher Scientific |
| Histology | Shandon (Chemisorbent) Filter | Thermo Scientific |
| Histology | Silver Nitrate STK Solution B | Poly Scientific R&D Corp. |
| Histology | Sodium Iodate | Fisher Scientific |
| Microbiology | Hydrogen Peroxide, 30% | EMD Chemicals |
| Chemistry | Omnitrace Ultra Perchloric Acid | EMD Chemicals |

Corrosives

- A corrosive is a substance defined by the DOT, as causing visible destruction or permanent changes in human skin tissue at the site of contact, or is highly corrosive to steel.

| Location | Product Name | Manufacturer |
| --- | --- | --- |
| Chemistry | Cleaning Solution | NOVA Biomedical |
| Chemistry | Hydrochloric Acid Solutions, 0.5%-50% V/V, 0.01N-6.25N | LabChem Inc. |
| Hematology | Ictotest Tablets | Siemens Healthcare Diagnostics |
| Histology | Chromic Acid 5% Aqueous | Poly Scientific R&D Corp. |
| Histology | Clorox Regular-Bleach | The Clorox Company |
| Histology | Fouchet's Working Solution | Poly Scientific R&D Corp. |
| Histology | Gill's Hematoxylin III | Poly Scientific R&D Corp. |
| Histology | Glutaraldehyde, 50% Aqueous | Tousimis Research Corporation |
| Histology | Hydrochloric Acid 20% Aqueous NA1789 | Poly Scientific R&D Corp. |
| Histology | Iodine | Fisher Scientific |
| Histology | Mayer's Mucicarmine Solution | Poly Scientific R&D Corp. |
| Histology | Oxalic Acid 5% Aqueous Aquoeus | Poly Scientific R&D Corp. |
| Histology | Periodic Acid | EKI Industries Inc. |
| Histology | Periodic Acid 1% Aqueous | Poly Scientific R&D Corp. |
| Histology | Phosphotungstic Acid 5% Aqueous | Poly Scientific R&D Corp. |
| Histology | Schiff Reagent for Fungi | Poly Scientific R&D Corp. |
| Histology | Sodium Iodate | Fisher Scientific |
| Histology | Spill-X-FP (Dry Powder) | Tyco Fire Suppression And Building Products |
| Microbiology | Bactidrop Potassium Hydroxide (10%) | Remel Inc |
| Microbiology | Calcofluor White Stain Kit | Remel Inc |
| Microbiology | MicroScan 0.8% Sulfanilic Acid Reagent | Dade Behring Inc |
| Microbiology | Microscan Rapid Indole Reagent B1015-17 | Dade Behring Inc |
| Microbiology | PVA Fixative | Scientific Device Laboratory |
| Microbiology | RapID Nitrate A Reagent | Remel Inc |
| Microbiology | RapID Nitrate B Reagent | Remel Inc |
| Microbiology | Spill-X-FP (Dry Powder) | Tyco Fire Suppression And Building Products |
| Microbiology | Sulfuric Acid 1% Aqueous | Poly Scientific R&D Corp. |
| Microbiology | Sulfuric Acid 1%-250 Ml | Remel Inc |

| Location | Product Name | Manufacturer |
| --- | --- | --- |
| Microbiology | Xpert MRSA/SA BC | Cepheid USA |
| Special Reference Laboratory (SRL) | Anti-Jo-1 | Diamedix Corporation |
| Special Reference Laboratory (SRL) | Anti-Scl-70 | Diamedix Corporation |
| Special Reference Laboratory (SRL) | Anti-SM | Diamedix Corporation |
| Special Reference Laboratory (SRL) | Anti-SSA | Diamedix Corporation |
| Special Reference Laboratory (SRL) | Anti-SSB | Diamedix Corporation |
| Special Reference Laboratory (SRL) | CMV-IGG | Diamedix Corporation |
| Special Reference Laboratory (SRL) | CMV-IGM | Diamedix Corporation |
| Special Reference Laboratory (SRL) | ENA-6 Screens | Diamedix Corporation |
| Special Reference Laboratory (SRL) | H. Pylori IgG | Diamedix Corporation |
| Special Reference Laboratory (SRL) | Liaison Starter Kit Code 319102 | DiaSorin Inc. |
| Special Reference Laboratory (SRL) | Measles IgG | Diamedix Corporation |
| Special Reference Laboratory (SRL) | Mumps IgG | Diamedix Corporation |
| Special Reference Laboratory (SRL) | R2. Wash Solution Concentrate (30X) | Bio-Rad Laboratories USSD Clinical Diagnostics |
| Special Reference Laboratory (SRL) | R3. HIV-1/HIV-2 Plus O EIA Specimen Diluent | Bio-Rad Laboratories USSD Clinical Diagnostics |
| Special Reference Laboratory (SRL) | Rubella IgG | Diamedix Corporation |
| Special Reference Laboratory (SRL) | Toxoplasma IgG | Diamedix Corporation |
| Special Reference Laboratory (SRL) | Toxoplasma IgM Capture | Diamedix Corporation |
| Special Reference Laboratory (SRL) | VZV IgG (Varicella) | Diamedix Corporation |

Carcinogens

- IARC Group 1

- The chemical (mixture) is carcinogenic to humans. The exposure circumstance entails exposures that are carcinogenic to humans. This category is used when there is sufficient evidence of carcinogenicity in humans.

| Location | Product Name | Manufacturer | Carcinogenic Ingredient | CAS |
| --- | --- | --- | --- | --- |
| Histology | 10% Neutral Buffered Formalin | Richard-Allan Scientific | Formaldehyde | 000050-00-0 |
| Microbiology | 70% Isopropyl Alcohol | Enturia | 2-Propanol | 000067-63-0 |
| Ancillary Testing | Accu-Chek Inform II Battery Pack | ROCHE DIAGNOSTICS | Nickel | 007440-02-0 |
| Histology | Acid Alcohol 0.5% | Poly Scientific R&D Corp. | Ethanol | 000064-17-5 |
| Histology | Acid Alcohol 1% | Poly Scientific R&D Corp. | Ethanol | 000064-17-5 |
| Microbiology | Acid-Ethanol | Alpha-Tec Systems Inc | Ethyl Alcohol | 000064-17-5 |
| Histology | Alcian Yellow 1% Solution | Poly Scientific R&D Corp. | Ethanol | 000064-17-5 |
| Microbiology | Alcohol Prep Pad | Medline Industries, Inc. | 2-Propanol | 000067-63-0 |
| Histology | Alkaline Alcohol 50% | Poly Scientific R&D Corp. | Ethanol | 000064-17-5 |
| Microbiology | BactidropTM Desoxycholate (10%) | Remel Inc | Ethanol | 000064-17-5 |
| Histology | Bond Aspirating Probe Cleaning System | Leica Biosystems Newcastle Ltd | Sulfuric Acid | 007664-93-9 |
| Histology | Bouin's Fixative | Poly Scientific R&D Corp. | Formaldehyde | 000050-00-0 |
| Histology | Carbol Fuchsin Ziehl Neelsen | Poly Scientific R&D Corp. | Ethyl Alcohol | 000064-17-5 |
| Phlebotomy | Chloraprep 2.0% | Enturia | 2-Propanol | 000067-63-0 |
| Phlebotomy | ChloraPrep with Orange Tint | Enturia | 2-Propanol | 000067-63-0 |
| Phlebotomy | ChloraPrep with Tint | CareFusion | 2-Propanol | 000067-63-0 |
| Histology | Chromic Acid 4% Aqueous | Poly Scientific R&D Corp. | Chromium (Vi) Oxide, Chromic Acid | 001333-82-0 |
| Cytology | Clarifier I | Richard-Allan Scientific | Ethanol | 000064-17-5 |
| Washroom | Comet Powder Cleanser | Prestige Brands International | Quartz (SiO2) | 014808-60-7 |
| Cytology | Cyto-Stain | Richard-Allan Scientific | Ethanol | 000064-17-5 |
| Cytology | Denatured Alcohol 95% Histoprep | Fisher Scientific | Ethanol | 000064-17-5 |
| Hematology | Dyna-Hex Chlorhexidine Gluconate Solution 4.0% | Xttrium Laboratories, Inc. | 2-Propanol | 000067-63-0 |
| Histology | Envirocide Surface Disinfectant/Decontaminant Cleaner | Metrex Research Corporation | 2-Propanol | 000067-63-0 |
| Microbiology | Ethanol 70% | Alpha-Tec Systems Inc | Ethanol | 000064-17-5 |
| Histology | Fast Green Substitute for Lt Green Working Solution | Poly Scientific R&D Corp. | 2-Propanol | 000067-63-0 |
| Ancillary Testing | Formaldehyde Solution 10% W/W or Formalin Solution 10% | Fisher Scientific | Formaldehyde | 000050-00-0 |
| Microbiology | Formaldehyde Solution, Buffered 10% | Avantor Performance Materials | Formaldehyde | 000050-00-0 |
| Histology | Formaldehyde, 37 Wt% Solution, Stabilized with Methanol | Fisher Scientific | Formaldehyde | 000050-00-0 |
| Histology | Formalin, 10% Buffered | U.S. Biotex Corporation | Formaldehyde | 000050-00-0 |
| Cytology | Gleme Glass Cleaner | Ric Mar Industries, Inc. | Ethanol | 000064-17-5 |
| Blood Bank | GOJO Premium Foam Antibacterial Handwash | GOJO | Ethyl Alcohol | 000064-17-5 |
| Microbiology | Gram Crystal VIolet | BD Diagnostics | Ethanol | 000064-17-5 |
| Microbiology | Gram Decolorizer | BD Diagnostics | 2-Propanol | 000067-63-0 |
| Microbiology | Gram Safranin | BD Diagnostics | Ethanol | 000064-17-5 |
| Special Reference Laboratory (SRL) | H. Pylori IgG | Diamedix Corporation | Sulfuric Acid | 007664-93-9 |
| Histology | Harris Hematoxylin | Poly Scientific R&D Corp. | Ethanol | 000064-17-5 |
| Histology | Hematoxylin 5% Alcoholic | Poly Scientific R&D Corp. | Ethanol | 000064-17-5 |
| Cytology | Histoprep 100% Alcohol | Fisher Scientific | Ethanol | 000064-17-5 |
| Histology | Histoprep Mold Releasing Agent | Fisher Scientific | Ethyl Alcohol | 000064-17-5 |
| Hematology | Isopropanol | Fisher Scientific | 2-Propanol | 000067-63-0 |
| Hematology | Isopropyl Alcohol 70% In Water | Fisher Scientific | iso-Propyl alcohol | 000067-63-0 |
| Cytology | Joy | Procter & Gamble | Ethanol | 000064-17-5 |
| Special Reference Laboratory (SRL) | Kallestad Crithidia Luciliae Substrate | Bio-Rad Laboratories USSD Clinical Diagnostics | Cadmium selenide sulfide (Cd(Se,S)) | 012626-36-7 |
| Chemistry | Liquichek Immunoassay Plus Control | Bio-Rad Laboratories, Diagnostic Group | estradiol | 000050-28-2 |
| Chemistry | Liquichek Unassayed Chemistry Control (Human) | Bio-Rad Laboratories USSD Clinical Diagnostics | Ethanol | 000064-17-5 |
| Chemistry | Liquid Unassayed Multiqual | Bio-Rad Laboratories USSD Clinical Diagnostics | Ethanol | 000064-17-5 |
| Histology | Luxol Fast Blue 0.1% Alcoholic | Poly Scientific R&D Corp. | Ethanol | 000064-17-5 |
| Histology | Lymph Node Revealing Solution | Poly Scientific R&D Corp. | Formaldehyde | 000050-00-0 |
| Histology | Mayer's Mucicarmine Solution | Poly Scientific R&D Corp. | Ethanol | 000064-17-5 |
| Microbiology | Microscan Mineral Oil | Dade Behring Inc | Paraffin oils | 008012-95-1 |
| Microbiology | Microscan Reagent Q.C. Peptidate | Dade Behring Inc | Ethanol | 000064-17-5 |
| Microbiology | Modified Trichrome Stain Set | Scientific Device Laboratory | Ethanol | 000064-17-5 |
| Special Reference Laboratory (SRL) | Mumps IgG | Diamedix Corporation | Sulfuric Acid | 007664-93-9 |
| Ancillary Testing | Nickel Metal Hydride (NiMH) Battery | Harding Energy Systems, Inc. | Nickel hydroxide (Ni(OH)2) | 012054-48-7 |
| Histology | Protocol 10% Neutral Buffered Formalin | Fisher Scientific | Formaldehyde | 000050-00-0 |
| Hematology | Purell Foaming Hand Sanitizer | GOJO Industries, Inc. | Ethanol | 000064-17-5 |
| Microbiology | Purell Hand Sanitizer | GOJO Industries, Inc. | Ethyl Alcohol | 000064-17-5 |
| Ancillary Testing | Purell Instant Hand Sanitizer | GoJo Industries, Inc | Ethanol | 000064-17-5 |
| Blood Bank | Purell Instant Hand Sanitizer Foam | GOJO Industries, Inc. | Ethanol | 000064-17-5 |
| Microbiology | Purell Instant Hand Sanitizer with Aloe | GOJO Industries, Inc. | Ethanol | 000064-17-5 |
| Blood Bank | Purell Instant Hand Sanitizer with Moisturizers and Vitamin E | GOJO Industries, Inc. | Ethanol | 000064-17-5 |
| Microbiology | PVA Fixative | Scientific Device Laboratory | Ethanol | 000064-17-5 |
| Special Reference Laboratory (SRL) | R2. Wash Solution Concentrate (30X) | Bio-Rad Laboratories USSD Clinical Diagnostics | Sulfuric Acid | 007664-93-9 |
| Special Reference Laboratory (SRL) | R3. HIV-1/HIV-2 Plus O EIA Specimen Diluent | Bio-Rad Laboratories USSD Clinical Diagnostics | Sulfuric Acid | 007664-93-9 |
| Histology | Reagent Alcohol 100% | Leica MicroSystems | Ethanol | 000064-17-5 |
| Cytology | Reagent Alcohol, 95% | Leica MicroSystems | Ethanol | 000064-17-5 |
| Microbiology | Sanihol 70 | Decon Laboratories Inc. | Ethanol | 000064-17-5 |

| Location | Product Name | Manufacturer | Carcinogenic Ingredient | CAS |
| --- | --- | --- | --- | --- |
| Histology | Schiff Reagent for Fungi | Poly Scientific R&D Corp. | Benzenamine, 4-[(4-aminophenyl)(4-imino-2,5-cyclohexadien-1-ylidene)methyl]-, monohydrochloride | 000569-61-9 |
| Hematology | Semen Analysis Kit (Solution I-Semen Diluting Fluid) | ENG Scientific, Inc. | Formaldehyde | 000050-00-0 |
| Microbiology | Substrate Solutions | BD Biosciences | 2-Propanol | 000067-63-0 |
| Special Reference Laboratory (SRL) | Sulfuric Acid | Avantor Performance Materials | Sulfuric Acid | 007664-93-9 |
| Microbiology | Sulfuric Acid 1%-250 Ml | Remel Inc | Sulfuric Acid | 007664-93-9 |
| Ancillary Testing | Super Sani-Cloth Germicidal Disposable Wipes | Nice-Pak Products Inc. | 2-Propanol | 000067-63-0 |
| Microbiology | TB Carbolfuchsin KF | BD Diagnostics | Ethanol | 000064-17-5 |
| Microbiology | TB Methylene Blue | BD Diagnostics | Ethanol | 000064-17-5 |
| Phlebotomy | Three-In-One Multi-Purpose Oil | WD-40 Company | Distillates, petroleum, hydrotreated heavy naphthenic | 064742-52-5 |
| Chemistry | Tri-Flow Superior Lubricant with PTFE, 21021 21013 22025 26020 29200 | Sherwin Williams | Distillates, petroleum, solvent-refined light naphthenic | 064741-97-5 |
| Ancillary Testing | Uni-Safe/Univeral Gel Sorbent | PND CORPORATION | Cristobalite (SiO2) | 014464-46-1 |
| Blood Bank | Virex II 256 (US) | Johnson Diversey | Ethanol | 000064-17-5 |
| Microbiology | VP2 | bioMerieux, Inc. | Ethyl Alcohol | 000064-17-5 |
| Microbiology | Webcol/Curity Alcohol Prep Pads | Kendall Healthcare/ Tyco Healthcare/ Covidien | iso-Propyl alcohol | 000067-63-0 |
| Histology | Weigerts Hematoxylin Solution A | Poly Scientific R&D Corp. | Ethanol | 000064-17-5 |
| Microbiology | Xpert EV | Cepheid USA | Ethanol | 000064-17-5 |
| Microbiology | Xpert Flu | Cepheid | Ethanol | 000064-17-5 |

- IARC Group 2a

The chemical (mixture) is probably carcinogenic to humans. The exposure circumstance entails exposures that are probably carcinogenic to humans. This category is used when there is limited evidence of carcinogenicity in humans and sufficient evidence of carcinogenicity in experimental animals. Exceptionally, a chemical may be classified in this category solely on the basis of limited evidence of carcinogenicity in humans or of sufficient evidence of carcinogenicity in experimental animals strengthened by supporting evidence from other relevant data.

| Location | Product Name | Manufacturer | Probable Carcinogenic Ingredient | CAS |
| --- | --- | --- | --- | --- |
| Chemistry | Cardiac Marker Control VS | Cliniqa Corporation | Acetamide, 2,2-dichloro-N-[(1R,2R)-2-hydroxy-1-(hydroxymethyl)-2-(4-nitrophenyl)ethyl]- | 000056-75-7 |
| Blood Bank | Checkcell (weak) | Immucor Inc | Acetamide, 2,2-dichloro-N-[(1R,2R)-2-hydroxy-1-(hydroxymethyl)-2-(4-nitrophenyl)ethyl]- | 000056-75-7 |
| Blood Bank | Complement Control Cells | Immucor Gamma | Acetamide, 2,2-dichloro-N-[(1R,2R)-2-hydroxy-1-(hydroxymethyl)-2-(4-nitrophenyl)ethyl]- | 000056-75-7 |
| Histology | Congo Red 1% Aqueous | Poly Scientific R&D Corp. | 1-Naphthalenesulfonic acid, 3,3'-[[1,1'-biphenyl]-4,4'-diylbis(azo)]bis[4-amino-, disodium salt | 000573-58-0 |
| Blood Bank | CorQC Test System | Immucor | Acetamide, 2,2-dichloro-N-[(1R,2R)-2-hydroxy-1-(hydroxymethyl)-2-(4-nitrophenyl)ethyl]- | 000056-75-7 |
| Histology | Crystal Violet 1% Aqueous | Poly Scientific R&D Corp. | 1-Naphthalenesulfonic acid, 3,3'-[[1,1'-biphenyl]-4,4'-diylbis(azo)]bis[4-amino-, disodium salt | 000573-58-0 |
| Chemistry | Liquichek Cardiac Markers Plus Lt Control | Bio-Rad Laboratories USSD Clinical Diagnostics | Acetamide, 2,2-dichloro-N-[(1R,2R)-2-hydroxy-1-(hydroxymethyl)-2-(4-nitrophenyl)ethyl]- | 000056-75-7 |
| Chemistry | Liquichek Immunoassay Plus Control | Bio-Rad Laboratories, Diagnostic Group | estradiol | 000050-28-2 |
| Chemistry | Liquichek Immunoassay Plus Control | Bio-Rad Laboratories, Diagnostic Group | Pregn-4-ene-3,20-dione | 000057-83-0 |
| Chemistry | Liquichek Immunoassay Plus Control | Bio-Rad Laboratories, Diagnostic Group | Androst-4-en-3-one, 17-hydroxy-, (17b)- | 000058-22-0 |
| Blood Bank | Panoscreen I & II | Immucor | Chloraphenicol | 000056-75-7 |
| Microbiology | Remel Prepared Culture Media Containing Chloramphenicol | Remel Inc | Acetamide, 2,2-dichloro-N-[(1R,2R)-2-hydroxy-1-(hydroxymethyl)-2-(4-nitrophenyl)ethyl]- | 000056-75-7 |
| Microbiology | Remel Prepared Culture Media Containing Chloramphenicol and Cyclohexamide | Remel Inc | Acetamide, 2,2-dichloro-N-[(1R,2R)-2-hydroxy-1-(hydroxymethyl)-2-(4-nitrophenyl)ethyl]- | 000056-75-7 |

- IARC Group 2B

The chemical is possibly carcinogenic to humans. The exposure circumstance entails exposures that are possibly carcinogenic to humans. This category is generally used for chemicals for which there is limited evidence in humans in the absence of sufficient evidence in experimental animals. It may also be used when there is inadequate evidence of carcinogenicity in experimental animals. In some instances, a chemical for which there is inadequate evidence or no data in humans, but limited evidence of carcinogenicity in experimental animals together with supporting evidence from other relevant data, may be placed in this group.

| Location | Product Name | Manufacturer | Possible Carcinogenic Ingredient | CAS |
| --- | --- | --- | --- | --- |
| Ancillary Testing | Accu-Chek Inform II Battery Pack | ROCHE DIAGNOSTICS | Nickel | 007440-02-0 |
| Ancillary Testing | Accu-Chek Inform II Battery Pack | ROCHE DIAGNOSTICS | Cobalt lithium oxide (CoLiO2) | 012190-79-3 |
| General | AR-150TD/TD-J/TD-C/DC/MI/LI/CI./AR-151LI/AR-152MI (Black Developer) | Sharp Corporation | Carbon black | 001333-86-4 |
| General | B4400/B4600 Black Toner | Oki Data | Styrene | 000100-42-5 |
| General | B4400/B4600 Black Toner | Oki Data | Carbon black | 001333-86-4 |
| General | Black Toner, Type 9 | Oki Data | Benzene, ethenyl- | 000100-42-5 |
| General | Black Toner, Type 9 | Oki Data | Carbon black | 001333-86-4 |
| Histology | Carbol Fuchsin Ziehl Neelsen | Poly Scientific R&D Corp. | Basic Fuchsin | 000569-61-9 |
| Cytology | Cyto-Stain | Richard-Allan Scientific | 2-Pentanone, 4-methyl- | 000108-10-1 |
| General | Hp Color Laserjet Q6470A Black Print Cartridge | Hewlett Packard Company | Carbon black | 001333-86-4 |
| Chemistry | Liquichek Immunoassay Plus Control | Bio-Rad Laboratories, Diagnostic Group | 2,4,6(1H,3H,5H)-Pyrimidinetrione, 5-ethyl-5-phenyl- | 000050-06-6 |
| Chemistry | Liquichek Immunoassay Plus Control | Bio-Rad Laboratories, Diagnostic Group | Card-20(22)-enolide, 3-[(O-2,6-dideoxy-b-D-ribo-hexopyranosyl-(1®4)-O-2,6-dideoxy-b-D-ribo-hexopyranosyl-(1®4)-2,6-dideoxy-b-D-ribo-hexopyranosyl)oxy]-12,14-dihydroxy-, (3b,5b,12b)- | 020830-75-5 |
| Chemistry | Liquichek Unassayed Chemistry Control (Human) | Bio-Rad Laboratories USSD Clinical Diagnostics | 2,4,6(1H,3H,5H)-Pyrimidinetrione, 5-ethyl-5-phenyl- | 000050-06-6 |
| Chemistry | Liquichek Unassayed Chemistry Control (Human) | Bio-Rad Laboratories USSD Clinical Diagnostics | Card-20(22)-enolide, 3-[(O-2,6-dideoxy-b-D-ribo-hexopyranosyl-(1®4)-O-2,6-dideoxy-b-D-ribo-hexopyranosyl-(1®4)-2,6-dideoxy-b-D-ribo-hexopyranosyl)oxy]-12,14-dihydroxy-, (3b,5b,12b)- | 020830-75-5 |
| Chemistry | Liquid Unassayed Multiqual | Bio-Rad Laboratories USSD Clinical Diagnostics | Card-20(22)-enolide, 3-[(O-2,6-dideoxy-b-D-ribo-hexopyranosyl-(1®4)-O-2,6-dideoxy-b-D-ribo-hexopyranosyl-(1®4)-2,6-dideoxy-b-D-ribo-hexopyranosyl)oxy]-12,14-dihydroxy-, (3b,5b,12b)- | 020830-75-5 |
| Hemotology | Liquid Unassayed Multiqual | Bio-Rad Laboratories USSD Clinical Diagnostics | Card-20(22)-enolide, 3-[(O-2,6-dideoxy-b-D-ribo-hexopyranosyl-(1®4)-O-2,6-dideoxy-b-D-ribo-hexopyranosyl-(1®4)-2,6-dideoxy-b-D-ribo-hexopyranosyl)oxy]-12,14-dihydroxy-, (3b,5b,12b)- | 020830-75-5 |
| Histology | Micromount Mounting Medium | Leica MicroSystems | Benzene, ethyl- | 000100-41-4 |
| Ancillary Testing | Nickel Metal Hydride (NiMH) Battery | Harding Energy Systems, Inc. | Cobalt hydroxide (Co(OH)2) | 021041-93-0 |
| Hemotology | Osmometer Standard | Prec Systems | [1,1'-Biphenyl]-2-ol, sodium salt | 000132-27-4 |
| Special Reference Laboratory (SRL) | R2. Wash Solution Concentrate (30X) | Bio-Rad Laboratories USSD Clinical Diagnostics | Cobalt chloride (CoCl2) | 007646-79-9 |
| Microbiology | Remel Prepared Culture Media Containing Trypan Blue | Remel Inc | Trypan Blue | 000072-57-1 |
| Histology | Schiff Reagent for Fungi | Poly Scientific R&D Corp. | Benzenamine, 4-[(4-aminophenyl)(4-imino-2,5-cyclohexadien-1-ylidene)methyl]-, monohydrochloride | 000569-61-9 |
| General | T430 Return Program Print Cartridge | Lexmark International Inc. | Carbon black | 001333-86-4 |
| General | T640, T642, T644 Return Program Print Cartridge | Lexmark International Inc. | Carbon black | 001333-86-4 |
| Microbiology | TB Carbolfuchsin KF | BD Diagnostics | Benzenamine, 4-[(4-aminophenyl)(4-imino-2,5-cyclohexadien-1-ylidene)methyl]-, monohydrochloride | 000569-61-9 |
| Special Reference Laboratory (SRL) | TN-430 Toner, TN-460 Toner, TN-530 Toner, TN-560 Toner, TN-6300 Toner, TN-6600 Toner, TN-7300 Toner, TN-7600 Toner, TN-33J Toner and TN-36J Toner | Brother Industries, Ltd. | Carbon black | 001333-86-4 |
| General | Toner Cartridge (Black) | Minolta Corporation | Carbon black | 001333-86-4 |
| Histology | Xylenes, mixed isomers with ethylbenzene | Fisher Scientific | Benzene, ethyl- | 000100-41-4 |
| Histology | Xylenes, Purified | Leica MicroSystems | Benzene, ethyl- | 000100-41-4 |

## Standard Operating Procedures for Select Hazards

- Flammable or Compressed Gases

- Cylinders must be secured at all times so they cannot fall.

- Valve safety covers should be left on until pressure regulators are attached.  Only approved regulators may be used.

- Containers must be clearly labeled with name of contents and hazards.

- Hand trucks or dollies with a securing device installed must be used when moving cylinders.

- The use of oil, grease, or lubricants on valves is prohibited.

- Do not attempt to repair damaged cylinders or force frozen cylinder valves.

- No more than two cylinders should be manifolded together; however, several instruments or outlets are permitted for a single cylinder.

- When more than one cylinder of a highly flammable gas is to be used in one room, specific approval by the Center Director or Medical Center Safety Office must be obtained.

- Standby cylinders (full or empty) are not to be stored in the laboratory.

- Valves on all flammable gases cylinders shall be shut off when the unit is unattended.

Caustic Materials

- Specific information describing chemicals is given in the Safety Data Sheet provided by the manufacturer and on the container labels.

- If quantities of acids or alkalis are being used, use a shield or barrier so the breaks and spills can be controlled.

- Wear aprons or fluid resistant lab coats, gloves, and eye protection when handling highly corrosive materials as recommended on SDS.

- Dilution:  Use great care and add reagents S L O W L Y.  Always add acid to water.  Allow acid to run down the side of the container and mix slowly by gentle rotation.  Avoid overheating.

Toxic (includes reproductive toxicity and acute toxicity) and Reactive Chemicals

- Safety Data Sheets state the recommended limits or OSHA mandated guidelines for exposure to toxic chemicals.

- When a Threshold Limit Value (TLV) or Permissible Exposure Limit (PEL) for a chemical (i.e. glutaraldehyde) is greater than 50 ppm, the employee using the chemical shall handle the chemical in a fume hood which has been certified and approved for use.

- Whenever handling toxic substances with vapor pressures likely to exceed air concentration limits (i.e. formaldehyde, xylene), such work shall be conducted in a fume hood.

- Reactivity information can be found on a chemical SDS or container label.

- Reactive chemicals shall be handled with appropriate safety precautions, including segregation in storage and use of appropriate personal protective equipment.

Carcinogens

- A “hazardous substance" is regarded as a carcinogen or potential carcinogen if it is identified as such by: Chemicals that must be handled as potential carcinogens include those defined by OSHA as "select carcinogens."  OSHA defines select carcinogens as any substance that is:

  - Regulated as a carcinogen by OSHA, has been classified as "known to be carcinogenic" by the NTP, or listed as a group I carcinogen by the IARC

  - Has been classified as "reasonably anticipated to be carcinogenic" by the NTP or listed as a group 2A or 2B carcinogen by the IARC if it meets the toxicological criteria listed in the January 31, 1990 Fed Register, pages 3319-3320

- See Attachments F through J for a listing of carcinogens and potential carcinogens as outlined above.

- Bis-chloromethyl ether (BCME) may be a spontaneous reaction product following the mixing of hydrochloric acid and formaldehyde.  Although the decalcification of formalin fixed specimens in chlorine containing solutions is unlikely to produce significant levels of BCME, procedures allowing for BCME as a reaction product should be avoided.

- Benzidine is most likely to be found in a clinical laboratory as a chromagen.  Over 230 dyes contain Benzidine.  NIOSH Publication No.  80-109 Special Occupational Hazard Review for Benzidine-Based Dyes should be consulted for information.

- Formaldehyde is used extensively in the Anatomic section.

- Carcinogen Handling Requirements

- Do not ingest a carcinogen.

- Protective clothing and equipment, and hand washing

- Appropriate laboratory clothing should be specified in the procedure manual.  Appropriate clothing may include laboratory coats, aprons, respiratory masks or respirators, and gloves.

- Appropriate eye protective devices should be available and used in the work area.  Such devices may include close-fitting goggles.

- Personnel should wash hands and forearms after the completion of any procedure in which a chemical carcinogen has been used.  Following gross contact with carcinogenic material, thorough showering and clothes change is mandatory.

- Access to chemical carcinogens should be limited to knowledgeable individuals.  The work area should be designed to preclude casual contact by others.

- Work surfaces should be covered by dry absorbent plastic- backed paper which should be disposed of after each procedure.  Alternatively, the work surface should be constructed of impervious material and thoroughly decontaminated after each procedure.

- Working quantities should be maintained at a minimum.

- Labeling of the chemicals and of the work area where carcinogenic risk is significant is mandatory.

## Accident Reporting

- Employees who incur a work related injury or illness must notify their supervisor immediately.

- Supervisors must initiate the ASISTS accident report (VA Form 2162) immediately followed by an accident investigation within 72 hours of the notification to allow the organization to meet the OSHA recordkeeping requirements

- Corrective actions shall be initiated quickly to prevent repeating the incident and to remove the hazardous condition.

- If the employee needs to be seen by a physician, a CA-17 must be filled out by the supervisor and given to the employee to take with them to the physician.

- If the employee is filing for Worker’s Compensation, the CA 1 and 2 on ASISTS will need to be filled out.

- Printed copies of all reports will be submitted to the Laboratory Safety Officer and kept on file.

## Training

- Before working with particular laboratory chemicals, supervisors will provide employees with information and training on hazardous chemicals and chemical products in their work area at the time of initial assignment and whenever a new/modified hazardous chemical or chemical product is introduced in their work area.  Certification of training will be recorded in the Pathology Education Management Program and retained in the Service Line respective files.

Training will include the following:

- Operations or procedures in the work area where hazardous chemicals or chemical products are utilized.

- Location and availability of the written hazard communication program, SDS, and the chemical inventory.

- Methods and observations that may be used to detect the presence of released hazardous chemicals in the work area.

- Physical and health hazards of chemicals in the work area.

- Measures the employee can take to protect themselves from these hazards, including specific handling procedures and work practices that have been implemented to protect employees from exposure, and personal protective equipment to be used.

- Explanation of the SDS and chemical container labels, proper reading of SDS and chemical container labels, and how employees can obtain and use the appropriate hazard information.

- Training must be provided on required components of the Service “Chemical Hygiene Plan” where applicable.

- An annual review of the Chemical Hygiene/Hazardous Communication Plan

- The Safety Office (138 CDD) will provide generic training, upon request, to supervisors and research investigators regarding regulations and required topics for employee training.

- All employees who are assigned to workplaces where there is a health hazard from formaldehyde, must participate in a training program at the time of their initial assignment and whenever a new hazard from formaldehyde is introduced into their work area.

## Contractor Hazard Information Exchange

- A Contractor’s employees may be exposed to hazardous chemicals used or stored at this facility in the process of doing their work.  Likewise, the Contractor’s employees may use hazardous chemicals that could expose VA employees.

The Engineering Service, Planning Section delegated Project Manager will obtain SDS’ from the Contractor and make copies available to the supervisor(s) in the area where the Contractor’s employees will be working.

The delegated manager will introduce the contractor’s foreman to the VA supervisor (s) in the work area.

The VA supervisor will show the foreman where the SDS’ are located.

The VA supervisor(s) and Contractor foreman will exchange any necessary hazards and label information so both the VA and the Contractor are fully informed about hazardous chemicals to be used in the area.

These individuals will train their respective employees on the hazards involved and on any emergency procedures required.

The delegated Project Manager will submit an e-mail with an attachment documenting the completion of the above procedures to the Safety Office/SDS Coordinator (138 CDD).

## Record Maintenance

- Engineering Service, Safety Section will maintain exposure records, i.e. workplace monitoring results, for at least thirty (30) years.

Employee Health records will be maintained by the Occupational Health Office.

Service Chiefs will maintain related training records.

According to 29 CFR 1910.1020(d) (1) (ii) (B), SDS’ for chemicals no longer in use shall be retained for 30 years.  The retired SDS should also document when the chemical was used, what department it was used in and the retirement date.  The retired SDS’s will be stored in B112 in a binder titled “Retired SDS’s”.

## Chemical Spill Response

- Refer to SDS for appropriate action.

Remove any contaminated clothing or personal protective equipment (PPE)

Use emergency eyewash or shower if needed.

Notify all personnel in the immediate area of the spill

If spilled material is flammable, turn off ignition and heat sources.

Locate the spill kits in your area.

Spill control kits are provided in the laboratory.  At the Cooper Drive Division, there is a set in the Chemistry Section along the back wall.  There are three wall mounted green cases, one near the overhead shower outside of room B112, one near Histology in Rm. B-102, one in the tissue processing room (B110).  There are specific spill kits in room A606 and in the grossing room (B115).

The Spill Kits are checked annually by the Laboratory Safety Officer.  The supplies are checked for completeness and condition.  The spill agents are checked for clumping and color change.  If any component of the spill kit is unsatisfactory, the item will be removed and replaced with a satisfactory item.  If items are missing, the item will be replaced.

If spills are less than one liter, they can be handled by the staff.

If spills are greater the one liter, call the Safety Office @ 4176 or 3365.

If spill is large, notify supervisor, Security (ext. 4444) and Safety Officer (ext. 4176 at CDD, 3365 at LD, Pager 223).

- Close doors and secure affected area; and, if available, place damp cloths or towels at base of door to prevent escape of vapors from room.

- DO NOT ATTEMPT CLEANUP YOURSELF!

## Mercury disposal/spills

Based on the mercury reduction goal presented by the Environmental Protection Agency (EPA) and the American Hospital Association to eliminate mercury from hospitals by 2005, our laboratory eliminates or reduces mercury as it is found and is replaced (if possible) with an acceptable substitute.

- Mercury spill

- Evacuate everyone from the area and control access.

- Immediately notify both Environmental Management Service (ext. 4908 or pager 616) and Safety Officer (ext. 4176 at CDD, 3365 at LD) as to location and degree of spillage.

- For clean-up after normal working hours, you should page the Environmental Management Service Supervisor on duty (Pager 616).

Mercury Disposal

- Items found containing mercury should be reported to the Laboratory Safety Officer.  If possible, these items will be collected in a safe location pending disposal by the Safety Office per contract.

- Chemical waste containing mercury (stains, fixatives, etc) should be collected into containers with a hazardous waste label – date accumulation ends and complete list and percentage of all chemicals contained.  Do not store next to sink in case of accidental spill.  When full, this container should be labeled for disposal and moved to the appropriate holding area to await disposal per contract.

## Chemical Waste Disposal

- Review the Medical Center’s policy  on Hazardous Waste Management –

- MCM 138-20 (attachment E)

Waste disposal guidance may be found by calling the GEMS coordinator @ 3354 or the safety manager @ 3365.

Fill out a Request for a VA Chemical Pickup form (see attachment D)

The chemical pickup request form and procedure are available on the public drive under the “Safety Office” folder and then to the subfolder “Request for a VA Chemical Pickup (\\vhalexfpc20\Service\Public\Safety Office\Request for a VA Chemical Pickup\Request for a VA Chemical Pickup Procedure.doc)

Liquid wastes

a.   Some liquids may be flushed into the sewer system with copious amounts of water if the

chemical volume is minimal – refer to SDS

b.   Disposal through the sewer system must comply with all Federal, State, and local

regulations and ordinances.

c.   The Safety Officer should be consulted when in doubt whether a chemical can be disposed

via the sewer system.

When chemical waste is ready for disposal and pickup by contractor or GEMS coordinator, laboratory personnel will:

  - Assure that waste is in appropriate container; that the chemicals are compatible with one

- another; and that the container is labeled with the description or chemical name and

- percentage of each chemical in the waste contents (just affixing turn-in form is not

- sufficient  – container must have permanent label/notation identifying contents)

- Label should include date accumulation ends

- Fill out electronic Request for VA Chemical Pickup form located on the public drive under the      Safety Office folder. Tape a copy of form to container.

- The chemicals may be disposed of at one of the laboratory’s Hazardous Waste Satellite Accumulation Areas.  The most obvious one is in room B110.

Chemicals containing Methanol, Mercury, Ethanol, and Stain Waste

- Should be collected into separate containers labeled hazardous waste.

- Xylene and alcohol may be collected in the same container.

- Label should include date accumulation ends and complete list and percentage of all chemicals in the container.

- Do not store container near a sink in case of accidental spill.

- Complete “Request for VA Chemical Pickup Form” and attach copy.

- The chemicals may be disposed of at one of the laboratory’s Hazardous Waste Satellite Accumulation Areas.  The most obvious one is in room B110.

Unused reagents that contain Sodium Azide should be disposed of as “Non-Regulated Waste” or RCRA equivalent.

- RCRA equivalent reagents and chemicals should be disposed of in a bin marked “Non-Regulated Waste” (found on the back bench of Chemistry)  When the bin is near full, fill out a chemical pickup form  and have the GEMS coordinator pick them up.

- Label should include date accumulation ends and complete list of all chemicals in the container.

Unused acids or bases should be disposed as “Hazardous Waste”

- Label should include date accumulation ends and complete list and percentage of all chemicals in the container.

- Do not store container near a sink in case of accidental spill.

- Acid and base waste must be kept separate.

- Complete request for VA Chemical Pickup form and attach copy.

- The chemicals may be disposed of at one of the laboratory’s Hazardous Waste Satellite Accumulation Areas.  The most obvious one is in room B110.

Thiocyanate will be handled as RCRA equivalent.

- RCRA equivalent reagents and chemicals should be disposed of in a bin or container marked “Non-Regulated Waste” (found on the back bench of Chemistry)  When the bin or container  is near full, fill out a chemical pickup form  and have the GEMS coordinator pick them up.

- Label should include date accumulation ends and complete list and percentage of all chemicals in the container.

Cepheid waste cartridges containing sodium hydroxide (NaOH) should be managed as waste pH > 12.5. Additionally, waste cartridges containing chemical reagents and biological fluids should be considered “mixed waste”.

- The Cepheid cartridges will be collected in a container labeled as mixed waste.

- When the container is near full, fill out a chemical pickup form and have the GEMS coordinator pick them up.

- Label should include date accumulation ends and complete list and percentage of all chemicals in the container.

Containers in the Satellite accumulation areas must have lids closed and remain closed when not in use.

## Monitoring for Exposure

- Exposure or exposed means that an employee is subjected to a hazardous chemical in the course of employment through any route entry (inhalation, ingestion, skin contact or absorption, etc.) and includes potential (e.g. accidental or possible) exposure as referenced by the SDS.  When the employer discovers that an employee has received an exposure to any substance or agent, the employer must immediately notify the employee and take such steps that may be necessary to provide medical evaluation, monitoring, or treatment using the ASISTS accident report.  Likewise, an employee that has received a potentially hazardous exposure to a substance or agent must immediately notify the employer of such exposure.

- Employees are notified of any monitoring results within 15 working days after the receipt of the results.

- The following chemicals are routinely monitored:

- Formaldehyde

  - Signs of formaldehyde overexposure include:

    - irritation of the eyes, nose, and throat, even at low levels for short periods

    - Longer exposure or higher doses can cause coughing or choking.

    - Severe exposure can cause death from throat swelling or from chemical burns to the lungs.

    - Direct contact with the skin, eyes, or gastrointestinal tract can cause serious burns.

  - Exposure to any formaldehyde solution, gas or mixture composed of more than 0.1% formaldehyde is considered a potential carcinogen.

  - When employees’ skin could have become splashed with solutions containing 0.1% or greater formaldehyde, employees must flush the affected area with copious amounts of water.

  - The employer must assure that no employee is exposed to an airborne concentration of formaldehyde exceeding 0.75 ppm calculated as an eight hour time weighted average or 2 ppm as a 15 minute STEL (short term exposure limit).

  - The initial monitoring process shall be repeated each time there is a change in production, equipment, process, personnel, or control measures which may result in new or additional exposure to formaldehyde.

  - The employer shall periodically measure and accurately determine exposure to formaldehyde for employees shown by the initial monitoring to be exposed at or above the action level or at or above the STEL

  - Affected employees or their designated representatives have an opportunity to observe any monitoring of employee exposure to formaldehyde required by OSHA standard.

  - If the last monitoring results reveal employee exposure at or above the action level, the employer shall repeat monitoring of the employees at least every 6 months.

  - If the last monitoring results reveal employee exposure at or above the STEL, the employer shall repeat monitoring of the employees at least once a year under worst conditions.

  - The employer may discontinue periodic monitoring for employees if results from two consecutive sampling periods taken at least 7 days apart show that employee exposure is below the action level and the STEL.  The results must be statistically representative and consistent with the employer's knowledge of the job and work operation.

  - Employees at risk of potential over exposure to formaldehyde are monitored by badge on a semi-annual basis and if positive, then quarterly thereafter.

  - Exposure badges are furnished by the Safety Office and are sent to an outside laboratory for analysis.

  - The safety Office will forward the original copy of the results for review by the Medical Safety Officer.  A copy will be given to the Anatomic, Microbiology or Hematology supervisor.  The Service Chief will be notified in the event of an overexposure.

  - In the event of overexposure, the employee must be notified in writing by supervisor within 15 days of receiving results.  Employee Health must also be notified.

  - All results must be shared with the employee(s) monitored.

  - Plans must be implemented to reduce exposure.

  - Areas identified as exceeding permissible exposure limits must be identified with the following warning:

DANGER

FORMALDEHYDE

Irritant and possible cancer hazard

AUTHORIZED PERSONNEL ONLY

- Xylene

  - Signs of Xylene overexposure include:

    - dizziness, excitement, drowsiness, incoordination, staggering gait, impaired reaction time, impaired short-term memory; headache; eye, nose, throat irritation; corneal vacuolization; anorexia; nausea, vomiting, abdominal pain; dryness and scaling of the skin, dermatitis;

    - reversible liver and kidney damage after exposure to sudden high concentrations.

  - The odor threshold for xylene is 1 part per million ppm parts of air.  Because this value is below the Occupational Safety and Health Administration (OSHA) current permissible exposure limit (PEL) of 100 ppm [29 CFR 1910.1000, Table Z-1-A], xylene is considered to have adequate warning properties.

  - The current OSHA PEL for xylene is 100 ppm (435 milligrams per cubic meter (mg/m (3)) of air) as an 8-hour time-weighted average (TWA) concentration.  The OSHA limits are based on the risk of irritant, narcotic, and chronic effects associated with exposure to xylene.

  - Exposure is not to exceed 150 ppm per 15 minute short term exposure limit (STEL).

  - Employees at risk of exposure are monitored by badge on a semi-annual basis.

  - Exposure badges are furnished by the Safety Office and sent to an outside laboratory for analysis.

  - The Safety Office will forward the original copy of the results for review by the Medical Center Safety Officer.  A copy will be given to the Anatomic, Microbiology or Hematology Section Supervisor.  The Service Chief will be notified in the event of an overexposure.

  - The employee must be notified of any monitoring results in writing by supervisor within 15 days of receiving results.  Employee Health must also be notified.

  - In the event of an exposure, plans must be implemented to reduce future exposure.

- Mercury (only in the case of a large spill)

  - Signs of Mercury overexposure include:

    - Tremors; emotional changes (e.g., mood swings, irritability, nervousness, excessive shyness); insomnia; neuromuscular changes (such as weakness, muscle atrophy, twitching); headaches; disturbances in sensations; changes in nerve responses; performance deficits on tests of cognitive function.

    - At higher exposures, there may be kidney effects, respiratory failure and death.

  - The allowable airborne concentration of mercury shall be interpreted as 0.1 mg/m(3) as determined on the basis of the full shift time-weighted average.

## Medical Surveillance

- The Personnel Health Service administers the medical surveillance program for all VA employees who work with hazardous chemicals under the following circumstances:

- Whenever an employee develops signs and symptoms possibly associated with exposure to a hazardous chemical in the lab;

- Where exposure monitoring reveals an exposure level routinely above the action level, permissible exposure, or short term exposure limit for a regulated hazardous chemical;

- Whenever an employee is believed to have been exposed to a hazardous chemical during an emergency (spill, leak, etc.).

This program allows affected employees to receive medical attention, including any follow-up examinations which the examining physician determines to be necessary.  The physician will be provided the following information:

- The identity of the hazardous chemical(s) to which the employee was exposed.

- The conditions under which the exposure occurred including any quantitative exposure data.

- A description of the signs and symptoms of exposure that the employee is experiencing.

The examining physician will provide his written opinion to include:

- Any recommendations for further medical follow-up

- The results of medical examination and testing

- Any medical condition which may place the employee at increased risk as a result of exposure

- A statement that the employee has been informed of the results of examination and any medical condition that may require further follow-up

## Emergency First Aid Procedures

- Eye

- In the event of a chemical splash to the eyes, ask co-workers to help you wash the eyes thoroughly.  Lift eye lids to avoid pooling of chemicals under eyelids.  Flush with water for 15 minutes.

- Seek medical attention immediately.

- Report accident to immediate supervisor.

Cuts, punctures and needle sticks:  (MUST BE REPORTED TO EMPLOYEE HEALTH)

- If the injuries are clean cuts, punctures, or needle sticks, clean the affected area immediately with soap, then flood or soak affected area in water.  Bandage to prevent infection.

- If the injuries are dirty cuts, punctures, or needle sticks, i.e. those contaminated with patient blood or body fluids or bacterial agents, proceed as above as long as no object which caused the wound is left in the wound.

- If the object causing the injury is not easily removed, do not attempt to cleanse the area.  Doing so may cause further damage.  Consult Employee Health immediately.

- If the objects which cause the injury are contaminated with patient's specimens, identify the specimen with the patient's name for further evaluation and investigation by Employee Health.  Thoroughly cleanse wound.  Report to Employee Health at once.

- Report all accidents of any degree to your supervisor and consult Employee Health.

Chemical Burns

- Corrosives can cause second or third degree burns.  These chemicals include alkalis such as sodium hydroxide and common acids such as hydrochloric, sulfuric, and nitric.

- Chemicals should be diluted and washed off with copious amounts of water.  Minor splashes and spills can be flooded in a sink.  Larger splashes and spills require the use of the emergency drench type laboratory shower.  Enlist the help of co-workers.  Some chemical powders should be brushed off the skin, before flooding with water, to avoid further skin and tissue damage.  Always consult the chemical manufacturer's SDS for emergency first aid procedures BEFORE working with any chemical.

Work related accident, injury or illness reporting

- Employees who incur a work related injury or illness must notify their supervisor immediately.

- Supervisors must initiate the ASISTS accident report (VA Form 2162) immediately followed by an accident investigation within 72 hours of the notification to allow the organization to meet the OSHA recordkeeping requirements

- Corrective actions shall be initiated quickly to prevent repeating the incident and to remove the hazardous condition.

- If the employee needs to be seen by a physician, a CA-17 must be filled out by the supervisor and given to the employee to take with them to the physician.

- If the employee is filing for Worker’s Compensation, the CA 1 and 2 on ASISTS will need to be filled out.

- Printed copies of all reports will be submitted to the Laboratory Safety Officer and kept on file.

## Policy regarding medical evaluation of employees who habitually demonstrate unsafe work practices.

- Each laboratory employee is responsible for complying with oral and written safety rules, regulations and procedures required for the task assigned.

In the event that an employee habitually does not follow safe work practices, they will be referred to the Laboratory Chief Medical Technologist for evaluation and if necessary in turn be referred to the Employee Health Physician for subsequent evaluation.

## Personal Protective Equipment

- Personal Protective Equipment (PPE) includes gloves, goggles, face shields, aprons, fluid impervious lab coats/gowns, and masks.  The equipment must be available for situations when an unexpected exposure to chemical substances, physical agents or biological materials could have serious consequences.

- Appropriate PPE will be provided without cost to employees and the supervisor ensures that the employee wears them.

Personal Protection Equipment

- All personal protective equipment shall be removed prior to leaving the work area.

- Eye

- Chemical safety goggles and/or face shields, rather than safety glasses or prescription glasses should be used when pouring any hazardous chemicals or hazardous waste as they provide the best protection against splashes.

- Protective eyewear must be available in all areas where hazardous substances are utilized.

- Protective eyewear should be easy to clean and disinfect.

- For those employees who wear glasses, goggles must fit over the glasses.

- Gloves

- Nitrile gloves are provided for all employees.

- Employees are required to wear gloves when handling all patient specimens.

- Gloves must be replaced whenever torn or soiled with blood or body fluids.

- Special acid resistant gloves will be provided in all areas utilizing acids.

- Nitrile gloves should never be washed or decontaminated and reused.

- Information and training are provided in Reference VA Directive 7701 – Latex Sensitivity/Allergy.

- Aprons and Lab Coats

- Lab coats must be impermeable to fluids so that no blood or body fluid will soak through to the clothing or skin of the employee.

- If a safety apron is required for chemical handling, it must be rubberized and acid resistant.

# References

| Comprehensive Guide to Laboratory Safety, 2nd Edition | Terry Jo Gile, MT(ASCP), MA Ed |
| --- | --- |
| Formaldehyde | VAMC Memo 138-31 |
| Hazard Communication Program | VAMC 138-07 |
| Hazardous Product Removal | VAMC Memo 90-22 |
| Hazardous Waste Management | VAMC Memo 138-16 |
| International Agency for Research on Cancer | Group 1 |
| International Agency for Research on Cancer | Group 2A |
| International Agency for Research on Cancer | Group 2B |
| National Toxicology Program Report on Carcinogens | 12th ROC (2011) |
| NIOSH Carcinogen List |  |
| OSHA Laboratory Standard | 29 CFR 1910.1450 |
| OSHA 1910, Subpart Z, Toxic and Hazardous Substances | 29 CFR 1910, Subpart Z |
| Formaldehyde Standard | 29CFR 1910.1048 and Appendices A, C, D, and E |

### Attachment A:  The OSHA Laboratory Standard (29 CFR 1910.1450)

Part Number: 1910

• Part Title: Occupational Safety and Health Standards

• Subpart: Z

• Subpart Title: Toxic and Hazardous Substances

• Standard Number: 1910.1450 App A

• Title: National Research Council Recommendations Concerning Chemical Hygiene in Laboratories (Non-Mandatory)

--------------------------------------------------------------------------------

To assist employers in developing an appropriate laboratory Chemical Hygiene Plan (CHP), the following non-mandatory recommendations were based on the National Research Council's (NRC) 2011 edition of "Prudent Practices in the Laboratory: Handling and Management of Chemical Hazards." This reference, henceforth referred to as "Prudent Practices," is available from the National Academies Press, 500 Fifth Street NW., Washington DC 20001 (www.nap.edu). "Prudent Practices" is cited because of its wide distribution and acceptance and because of its preparation by recognized authorities in the laboratory community through the sponsorship of the NRC. However, these recommendations do not modify any requirements of the OSHA Laboratory standard. This appendix presents pertinent recommendations from "Prudent Practices," organized into a form convenient for quick reference during operation of a laboratory and during development and application of a CHP. For a detailed explanation and justification for each recommendation, consult "Prudent Practices."

"Prudent Practices" deals with both general laboratory safety and many types of chemical hazards, while the Laboratory standard is concerned primarily with chemical health hazards as a result of chemical exposures. The recommendations from "Prudent Practices" have been paraphrased, combined, or otherwise reorganized in order to adapt them for this purpose. However, their sense has not been changed.

Section F contains information from the U.S. Chemical Safety Board's (CSB) Fiscal Year 2011 Annual Performance and Accountability report and Section F contains recommendations extracted from the CSB's 2011 case study, "Texas Tech University Laboratory Explosion," available from: http://www.csb.gov/.

Culture of Safety

With the promulgation of the Occupational Safety and Health Administration (OSHA) Laboratory standard (29 CFR 1910.1450), a culture of safety consciousness, accountability, organization, and education has developed in industrial, governmental, and academic laboratories. Safety and training programs have been implemented to promote the safe handling of chemicals from ordering to disposal, and to train laboratory personnel in safe practices. Laboratory personnel must realize that the welfare and safety of each individual depends on clearly defined attitudes of teamwork and personal responsibility. Learning to participate in this culture of habitual risk assessment, experiment planning, and consideration of worst-case possibilities—for oneself and one’s fellow workers—is as much part of a scientific education as learning the theoretical background of experiments or the step-by-step protocols for doing them in a professional manner. A crucial component of chemical education for all personnel is to nurture basic attitudes and habits of prudent behavior so that safety is a valued and inseparable part of all laboratory activities throughout their career.

Over the years, special techniques have been developed for handling chemicals safely. Local, state, and federal regulations hold institutions that sponsor chemical laboratories accountable for providing safe working environments. Beyond regulation, employers and scientists also hold themselves personally responsible for their own safety, the safety of their colleagues and the safety of the general public. A sound safety organization that is respected by all requires the participation and support of laboratory administrators, workers, and students. A successful health and safety program requires a daily commitment from everyone in the organization. To be most effective, safety and health must be balanced with, and incorporated into, laboratory processes. A strong safety and health culture is the result of positive workplace attitudes—from the chief executive officer to the newest hire; involvement and buy-in of all members of the workforce; mutual, meaningful, and measurable safety and health improvement goals; and policies and procedures that serve as reference tools, rather than obscure rules.

In order to perform their work in a prudent manner, laboratory personnel must consider the health, physical, and environmental hazards of the chemicals they plan to use in an experiment. However, the ability to accurately identify and assess laboratory hazards must be taught and encouraged through training and ongoing organizational support. This training must be at the core of every good health and safety program. For management to lead, personnel to assess worksite hazards, and hazards to be eliminated or controlled, everyone involved must be trained.

A. General Principles

1. Minimize All Chemical Exposures and Risks

Because few laboratory chemicals are without hazards, general precautions for handling all laboratory chemicals should be adopted. In addition to these general guidelines, specific guidelines for chemicals that are used frequently or are particularly hazardous should be adopted.

Laboratory personnel should conduct their work under conditions that minimize the risks from both known and unknown hazardous substances. Before beginning any laboratory work, the hazards and risks associated with an experiment or activity should be determined and the necessary safety precautions implemented. Every laboratory should develop facility-specific policies and procedures for the highest-risk materials and procedures used in their laboratory. To identify these, consideration should be given to past accidents, process conditions, chemicals used in large volumes, and particularly hazardous chemicals.

Perform Risk Assessments for Hazardous Chemicals and Procedures Prior to Laboratory Work:

(a) Identify chemicals to be used, amounts required, and circumstances of use in the experiment. Consider any special employee or laboratory conditions that could create or increase a hazard. Consult sources of safety and health information and experienced scientists to ensure that those conducting the risk assessment have sufficient expertise.

(b) Evaluate the hazards posed by the chemicals and the experimental conditions. The evaluation should cover toxic, physical, reactive, flammable, explosive, radiation, and biological hazards, as well as any other potential hazards posed by the chemicals.

(c) For a variety of physical and chemical reasons, reaction scale-ups pose special risks, which merit additional prior review and precautions.

(d) Select appropriate controls to minimize risk, including use of engineering controls, administrative controls, and personal protective equipment (PPE) to protect workers from hazards. The controls must ensure that OSHA’s Permissible Exposure Limits (PELs) are not exceeded. Prepare for contingencies and be aware of the institutional procedures in the event of emergencies and accidents.

One sample approach to risk assessment is to answer these five questions:

(a) What are the hazards?

(b) What is the worst thing that could happen?

(c) What can be done to prevent this from happening?

(d) What can be done to protect from these hazards?

(e) What should be done if something goes wrong?

2. Avoid Underestimation of Risk

Even for substances of no known significant hazard, exposure should be minimized; when working with substances that present special hazards, special precautions should be taken. Reference should be made to the safety data sheet (SDS) that is provided for each chemical. Unless otherwise known, one should assume that any mixture will be more toxic than its most toxic component and that all substances of unknown toxicity are toxic.

Determine the physical and health hazards associated with chemicals before working with them. This determination may involve consulting literature references, laboratory chemical safety summaries (LCSSs), SDSs, or other reference materials. Consider how the chemicals will be processed and determine whether the changing states or forms will change the nature of the hazard. Review your plan, operating limits, chemical evaluations and detailed risk assessment with other chemists, especially those with experience with similar materials and protocols.

Before working with chemicals, know your facility’s policies and procedures for how to handle an accidental spill or fire. Emergency telephone numbers should be posted in a prominent area. Know the location of all safety equipment and the nearest fire alarm and telephone.

3. Adhere to the Hierarchy of Controls

The hierarchy of controls prioritizes intervention strategies based on the premise that the best way to control a hazard is to systematically remove it from the workplace, rather than relying on employees to reduce their exposure. The types of measures that may be used to protect employees (listed from most effective to least effective) are: engineering controls, administrative controls, work practices, and PPE. Engineering controls, such as chemical hoods, physically separate the employee from the hazard. Administrative controls, such as employee scheduling, are established by management to help minimize the employees’ exposure time to hazardous chemicals. Work practice controls are tasks that are performed in a designated way to minimize or eliminate hazards. Personal protective equipment and apparel are additional protection provided under special circumstances and when exposure is unavoidable.

Face and eye protection is necessary to prevent ingestion and skin absorption of hazardous chemicals. At a minimum, safety glasses, with side shields, should be used for all laboratory work. Chemical splash goggles are more appropriate than regular safety glasses to protect against hazards such as projectiles, as well as when working with glassware under reduced or elevated pressures (e.g., sealed tube reactions), when handling potentially explosive compounds (particularly during distillations), and when using glassware in high-temperature operations. Do not allow laboratory chemicals to come in contact with skin. Select gloves carefully to ensure that they are impervious to the chemicals being used and are of correct thickness to allow reasonable dexterity while also ensuring adequate barrier protection.

Lab coats and gloves should be worn when working with hazardous materials in a laboratory. Wear closed-toe shoes and long pants or other clothing that covers the legs when in a laboratory where hazardous chemicals are used. Additional protective clothing should be used when there is significant potential for skin-contact exposure to chemicals. The protective characteristics of this clothing must be matched to the hazard. Never wear gloves or laboratory coats outside the laboratory or into areas where food is stored and consumed.

4. Provide Laboratory Ventilation

The best way to prevent exposure to airborne substances is to prevent their escape into the working atmosphere by the use of hoods and other ventilation devices. To determine the best choice for laboratory ventilation using engineering controls for personal protection, employers are referred to Table 9.3 of the 2011 edition of ‘‘Prudent Practices.’’ Laboratory chemical hoods are the most important components used to protect laboratory personnel from exposure to hazardous chemicals.

(a) Toxic or corrosive chemicals that require vented storage should be stored in vented cabinets instead of in a chemical hood.

(b) Chemical waste should not be disposed of by evaporation in a chemical hood.

(c) Keep chemical hood areas clean and free of debris at all times.

(d) Solid objects and materials, such as paper, should be prevented from entering the exhaust ducts as they can reduce the air flow.

(e) Chemical hoods should be maintained, monitored and routinely tested for proper performance.

A laboratory ventilation system should include the following characteristics and practices:

(a) Heating and cooling should be adequate for the comfort of workers and operation of equipment. Before modification of any building HVAC, the impact on laboratory or hood ventilation should be considered, as well as how laboratory ventilation changes may affect the building HVAC.

(b) A negative pressure differential should exist between the amount of air exhausted from the laboratory and the amount supplied to the laboratory to prevent uncontrolled chemical vapors from leaving the laboratory.

(c) Local exhaust ventilation devices should be appropriate to the materials and operations in the laboratory.

(d) The air in chemical laboratories should be continuously replaced so that concentrations of odoriferous or toxic substances do not increase during the workday.

(e) Laboratory air should not be recirculated but exhausted directly outdoors.

(f) Air pressure should be negative with respect to the rest of the building. Local capture equipment and systems should be designed only by an experienced engineer or industrial hygienist.

(g) Ventilation systems should be inspected and maintained on a regular basis. There should be no areas where air remains static or areas that have unusually high airflow velocities.

Before work begins, laboratory workers should be provided with proper training that includes how to use the ventilation equipment, how to ensure that it is functioning properly, the consequences of improper use, what to do in the event of a system failure or power outage, special considerations, and the importance of signage and postings.

5. Institute a Chemical Hygiene Program

A comprehensive chemical hygiene program is required. It should be designed to minimize exposures, injuries, illnesses and incidents. There should be a regular, continuing effort that includes program oversight, safe facilities, chemical hygiene planning, training, emergency preparedness and chemical security. The chemical hygiene program must be reviewed annually and updated as necessary whenever new processes, chemicals, or equipment is implemented. Its recommendations should be followed in all laboratories.

6. Observe the PELs and TLVs

OSHA’s Permissible Exposure Limits (PELs) must not be exceeded. The American Conference of Governmental Industrial Hygienists’ Threshold Limit Values (TLVs) should also not be exceeded.

B. Responsibilities

Persons responsible for chemical hygiene include, but are not limited to, the following:

1. Chemical Hygiene Officer

(a) Establishes, maintains, and revises the chemical hygiene plan (CHP).

(b) Creates and revises safety rules and regulations.

(c) Monitors procurement, use, storage, and disposal of chemicals.

(d) Conducts regular inspections of the laboratories, preparations rooms, and chemical storage rooms, and submits detailed laboratory inspection reports to administration.

(e) Maintains inspection, personnel training, and inventory records.

(f) Assists laboratory supervisors in developing and maintaining adequate facilities.

(g) Seeks ways to improve the chemical hygiene program.

2. Department Chairperson or Director

(a) Assumes responsibility for personnel engaged in the laboratory use of hazardous chemicals.

(b) Provides the chemical hygiene officer (CHO) with the support necessary to implement and maintain the CHP.

(c) After receipt of laboratory inspection report from the CHO, meets with laboratory supervisors to discuss cited violations and to ensure timely actions to protect trained laboratory personnel and facilities and to ensure that the department remains in compliance with all applicable federal, state, university, local and departmental codes and regulations.

(d) Provides budgetary arrangements to ensure the health and safety of the departmental personnel, visitors, and students.

3. Departmental Safety Committee reviews accident reports and makes appropriate recommendations to the department chairperson regarding proposed changes in the laboratory procedures.

4. Laboratory Supervisor or Principal Investigator has overall responsibility for chemical hygiene in the laboratory, including responsibility to:

(a) Ensure that laboratory personnel comply with the departmental CHP and do not operate equipment or handle hazardous chemicals without proper training and authorization.

(b) Always wear personal protective equipment (PPE) that is compatible to the degree of hazard of the chemical.

(c) Follow all pertinent safety rules when working in the laboratory to set an example.

(d) Review laboratory procedures for potential safety problems before assigning to other laboratory personnel.

(e) Ensure that visitors follow the laboratory rules and assumes responsibility for laboratory visitors.

(f) Ensure that PPE is available and properly used by each laboratory employee and visitor.

(g) Maintain and implement safe laboratory practices.

(h) Provide regular, formal chemical hygiene and housekeeping inspections, including routine inspections of emergency equipment;

(i) Monitor the facilities and the chemical fume hoods to ensure that they are maintained and function properly. Contact the appropriate person, as designated by the department chairperson, to report problems with the facilities or the chemical fume hoods.

5. Laboratory Personnel

(a) Read, understand, and follow all safety rules and regulations that apply to the work area;

(b) Plan and conduct each operation in accordance with the institutional chemical hygiene procedures;

(c) Promote good housekeeping practices in the laboratory or work area.

(d) Notify the supervisor of any hazardous conditions or unsafe work practices in the work area.

(e) Use PPE as appropriate for each procedure that involves hazardous chemicals.

C. The Laboratory Facility

General Laboratory Design Considerations Wet chemical spaces and those with a higher degree of hazard should be separated from other spaces by a wall or protective barrier wherever possible. If the areas cannot be separated, then workers in lower hazard spaces may require additional protection from the hazards in connected spaces.

1. Laboratory Layout and Furnishing

(a) Work surfaces should be chemically resistant, smooth, and easy to clean.

(b) Hand washing sinks for hazardous materials may require elbow, foot, or electronic controls for safe operation.

(c) Wet laboratory areas should have chemically resistant, impermeable, slipresistant flooring.

(d) Walls should be finished with a material that is easy to clean and maintain.

(e) Doors should have view panels to prevent accidents and should open in the direction of egress.

(f) Operable windows should not be present in laboratories, particularly if there are chemical hoods or other local ventilation systems present.

2. Safety Equipment and Utilities

(a) An adequate number and placement of safety showers, eyewash units, and fire extinguishers should be provided for the laboratory.

(b) Use of water sprinkler systems is resisted by some laboratories because of the presence of electrical equipment or waterreactive materials, but it is still generally safer to have sprinkler systems installed. A fire large enough to trigger the sprinkler system would have the potential to cause far more destruction than the local water damage.

D. Chemical Hygiene Plan (CHP)

The OSHA Laboratory standard defines a CHP as ‘‘a written program developed and implemented by the employer which sets forth procedures, equipment, personal protective equipment and work practices that are capable of protecting employees from the health hazards presented by hazardous chemicals used in that particular workplace.’’ (29 CFR 1910.1450(b)). The Laboratory Standard requires a CHP: ‘‘Where hazardous chemicals as defined by this standard are used in the workplace, the employer shall develop and carry out the provisions of a written Chemical Hygiene Plan.’’ (29 CFR 1910.1450(e)(1)). The CHP is the foundation of the laboratory safety program and must be reviewed and updated, as needed, and at least on an annual basis to reflect changes in policies and personnel. A CHP should be facility specific and can assist in promoting a culture of safety to protect workers from exposure to hazardous materials.

1. The Laboratory’s CHP must be readily available to workers and capable of protecting workers from health hazards and minimizing exposure. Include the following topics in the CHP:

(a) Individual chemical hygiene responsibilities;

(b) Standard operating procedures;

(c) Personal protective equipment, engineering controls and apparel;

(d) Laboratory equipment;

(e) Safety equipment;

(f) Chemical management;

(g) Housekeeping;

(h) Emergency procedures for accidents and spills;

(i) Chemical waste;

(j) Training;

(k) Safety rules and regulations;

(l) Laboratory design and ventilation;

(m) Exposure monitoring;

(n) Compressed gas safety;

(o) Medical consultation and examination.

It should be noted that the nature of laboratory work may necessitate addressing biological safety, radiation safety and security issues.

2. Chemical Procurement, Distribution, and Storage

Prudent chemical management includes the following processes:

Chemical Procurement:

(a) Information on proper handling, storage, and disposal should be known to those who will be involved before a substance is received.

(b) Only containers with adequate identifying labels should be accepted.

(c) Ideally, a central location should be used for receiving all chemical shipments.

(d) Shipments with breakage or leakage should be refused or opened in a chemical hood.

(e) Only the minimum amount of the chemical needed to perform the planned work should be ordered.

(f) Purchases of high risk chemicals should be reviewed and approved by the CHO.

(g) Proper protective equipment and handling and storage procedures should be in place before receiving a shipment.

Chemical Storage:

(a) Chemicals should be separated and stored according to hazard category and compatibility.

(b) SDS and label information should be followed for storage requirements.

(c) Maintain existing labels on incoming containers of chemicals and other materials.

(d) Labels on containers used for storing hazardous chemicals must include the chemical identification and appropriate hazard warnings.

(e) The contents of all other chemical containers and transfer vessels, including, but not limited to, beakers, flasks, reaction vessels, and process equipment, should be properly identified.

(f) Chemical shipments should be dated upon receipt and stock rotated.

(g) Peroxide formers should be dated upon receipt, again dated upon opening, and stored away from heat and light with tightfitting, nonmetal lids.

(h) Open shelves used for chemical storage should be secured to the wall and contain 3/4-inch lips. Secondary containment devices should be used as necessary.

(i) Consult the SDS and keep incompatibles separate during transport, storage, use, and disposal.

(j) Oxidizers, reducing agents, and fuels should be stored separately to prevent contact in the event of an accident.

(k) Chemicals should not be stored in the chemical hood, on the floor, in areas of egress, on the benchtop, or in areas near heat or in direct sunlight.

(l) Laboratory-grade, flammable-rated refrigerators and freezers should be used to store sealed chemical containers of flammable liquids that require cool storage. Do not store food or beverages in the laboratory refrigerator.

(m) Highly hazardous chemicals should be stored in a well-ventilated and secure area designated for that purpose.

(n) Flammable chemicals should be stored in a spark-free environment and in approved flammable-liquid containers and storage cabinets. Grounding and bonding should be used to prevent static charge buildups when dispensing solvents.

(o) Chemical storage and handling rooms should be controlled-access areas. They should have proper ventilation, appropriate signage, diked floors, and fire suppression systems.

Chemical Handling:

(a) As described above, a risk assessment should be conducted prior to beginning work with any hazardous chemical for the first time.

(b) All SDS and label information should be read before using a chemical for the first time.

(c) Trained laboratory workers should ensure that proper engineering controls (ventilation) and PPE are in place.

Chemical Inventory:

(a) Prudent management of chemicals in any laboratory is greatly facilitated by keeping an accurate inventory of the chemicals stored.

(b) Unneeded items should be discarded or returned to the storeroom.

Transporting Chemicals:

(a) Secondary containment devices should be used when transporting chemicals.

(b) When transporting chemicals outside of the laboratory or between stockrooms and laboratories, the transport container should be break-resistant.

(c) High-traffic areas should be avoided.

Transferring Chemicals:

(a) Use adequate ventilation (such as a fume hood) when transferring even a small amount of a particularly hazardous substance (PHS).

(b) While drum storage is not appropriate for laboratories, chemical stockrooms may purchase drum quantities of solvents used in high volumes. Ground and bond the drum and receiving vessel when transferring flammable liquids from a drum to prevent static charge buildup.

(c) If chemicals from commercial sources are repackaged into transfer vessels, the new containers should be labeled with all essential information on the original container.

Shipping Chemicals: Outgoing chemical shipments must meet all applicable Department of Transportation (DOT) regulations and should be authorized and handled by the institutional shipper.

3. Waste Management

A waste management plan should be in place before work begins on any laboratory activity. The plan should utilize the following hierarchy of practices:

(a) Reduce waste sources. The best approach to minimize waste generation is by reducing the scale of operations, reducing its formation during operations, and, if possible, substituting less hazardous chemicals for a particular operation.

(b) Reuse surplus materials. Only the amount of material necessary for an experiment should be purchased, and, if possible, materials should be reused.

(c) Recycle waste. If waste cannot be prevented or minimized, the organization should consider recycling chemicals that can be safely recovered or used as fuel.

(d) Dispose of waste properly. Sink disposal may not be appropriate. Proper waste disposal methods include incineration, treatment, and land disposal. The organization’s environmental health and safety (EHS) office should be consulted in determining which methods are appropriate for different types of waste.

Collection and Storage of Waste:

(a) Chemical waste should be accumulated at or near the point of generation, under the control of laboratory workers.

(b) Each waste type should be stored in a compatible container pending transfer or disposal. Waste containers should be clearly labeled and kept sealed when not in use.

(c) Incompatible waste types should be kept separate to ensure that heat generation, gas evolution, or another reaction does not occur.

(d) Waste containers should be segregated by how they will be managed. Waste containers should be stored in a designated location that does not interfere with normal laboratory operations. Ventilated storage and secondary containment may be appropriate for certain waste types.

(e) Waste containers should be clearly labeled and kept sealed when not in use. Labels should include the accumulation start date and hazard warnings as appropriate.

(f) Non-explosive electrical systems, grounding and bonding between floors and containers, and non-sparking conductive floors and containers should be used in the central waste accumulation area to minimize fire and explosion hazards. Fire suppression systems, specialized ventilation systems, and dikes should be installed in the central waste accumulation area. Waste management workers should be trained in proper waste handling procedures as well as contingency planning and emergency response. Trained laboratory workers most familiar with the waste should be actively involved in waste management decisions to ensure that the waste is managed safely and efficiently. Engineering controls should be implemented as necessary, and personal protective equipment should be worn by workers involved in waste management.

4. Inspection Program

Maintenance and regular inspection of laboratory equipment are essential parts of the aboratory safety program. Management should participate in the design of a laboratory inspection program to ensure that the facility is safe and healthy, workers are adequately trained, and proper procedures are being followed.

Types of inspections: The program should include an appropriate combination of routine inspections, self-audits, program audits, peer inspections, EHS inspections, and inspections by external entities.

Elements of an inspection:

(a) Inspectors should bring a checklist to ensure that all issues are covered and a camera to document issues that require correction.

(b) Conversations with workers should occur during the inspection, as they can provide valuable information and allow inspectors an opportunity to show workers how to fix problems.

(c) Issues resolved during the inspection should be noted.

(d) An inspection report containing all findings and recommendations should be prepared for management and other appropriate workers.

(e) Management should follow-up on the inspection to ensure that all corrections are implemented.

5. Medical Consultation and Examination

The employer must provide all employees who work with hazardous chemicals an opportunity to receive medical attention, including any follow-up examinations that the examining physician determines to be necessary, whenever an employee develops signs or symptoms associated with a hazardous chemical to which the employee may have been exposed in the laboratory. If an employee encounters a spill, leak, explosion or other occurrence resulting in the likelihood of a hazardous exposure, the affected employee must be provided an opportunity for a medical consultation by a licensed physician. All medical examinations and consultations must be performed by or under the direct supervision of a licensed physician and must be provided without cost to the employee, without loss of pay and at a reasonable time and place. The identity of the hazardous chemical, a description of the incident, and any signs and symptoms that the employee may experience must be relayed to the physician.

6. Records

All accident, fatality, illness, injury, and medical records and exposure monitoring records must be retained by the institution in accordance with the requirements of state and federal regulations (see 29 CFR part 1904 and § 1910.1450(j)). Any exposure monitoring results must be provided to affected laboratory staff within 15 working days after receipt of the results (29 CFR 1910.1450(d)(4)).

7. Signs

Prominent signs of the following types should be posted:

(a) Emergency telephone numbers of emergency personnel/facilities, supervisors, and laboratory workers;

(b) Location signs for safety showers, eyewash stations, other safety and first aid equipment, and exits; and

(c) Warnings at areas or equipment where special or unusual hazards exist.

8. Spills and Accidents

Before beginning an experiment, know your facility’s policies and procedures for how to handle an accidental release of a hazardous substance, a spill or a fire. Emergency response planning and training are especially important when working with highly toxic compounds. Emergency telephone numbers should be posted in a prominent area. Know the location of all safety equipment and the nearest fire alarm and telephone. Know who to notify in the event of an emergency. Be prepared to provide basic emergency treatment. Keep your co-workers informed of your activities so they can respond appropriately. Safety equipment, including spill control kits, safety shields, fire safety equipment, PPE, safety showers and eyewash units, and emergency equipment should be available in wellmarked highly visible locations in all chemical laboratories. The laboratory supervisor or CHO is responsible for ensuring that all personnel are aware of the locations of fire extinguishers and are trained in their use. After an extinguisher has been used, designated personnel must promptly recharge or replace it (29 CFR 1910.157(c)(4)). The laboratory supervisor or CHO is also responsible for ensuring proper training and providing supplementary equipment as needed.

Special care must be used when handling solutions of chemicals in syringes with needles. Do not recap needles, especially when they have been in contact with chemicals. Remove the needle and discard it immediately after use in the appropriate sharps containers. Blunt-tip needles are available from a number of commercial sources and should be used unless a sharp needle is required to puncture rubber septa or for subcutaneous injection.

For unattended operations, laboratory lights should be left on, and signs should be posted to identify the nature of the experiment and the hazardous substances in use. Arrangements should be made, if possible, for other workers to periodically inspect the operation. Information should be clearly posted indicating who to contact in the event of an emergency. Depending on the nature of the hazard, special rules, precautions, and alert systems may be necessary.

9. Training and Information

Personnel training at all levels within the organization, is essential. Responsibility and accountability throughout the organization are key elements in a strong safety and health program. The employer is required to provide employees with information and training to ensure that they are apprised of the hazards of chemicals present in their work area (29 CFR 1910.1450(f)). This information must be provided at the time of an employee’s initial assignment to a work area where hazardous chemicals are present and prior to assignments involving new exposure situations. The frequency of refresher information and training should be determined by the employer. At a minimum, laboratory personnel should be trained on their facility’s specific CHP, methods and observations that may be used to detect the presence or release of a hazardous chemical (such as monitoring conducted by the employer, continuous monitoring devices, visual appearance or odor of hazardous chemicals when being released), the physical and health hazards of chemicals in the work area and means to protect themselves from these hazards. Trained laboratory personnel must know shut-off procedures in case of an emergency. All SDSs must be made available to the employees.

E. General Procedures for Working With Chemicals

The risk of laboratory injuries can be reduced through adequate training, improved engineering, good housekeeping, safe work practice and personal behavior.

1. General Rules for Laboratory Work With Chemicals

(a) Assigned work schedules should be followed unless a deviation is authorized by the laboratory supervisor.

(b) Unauthorized experiments should not be performed.

(c) Plan safety procedures before beginning any operation.

(d) Follow standard operating procedures at all times.

(e) Always read the SDS and label before using a chemical.

(f) Wear appropriate PPE at all times.

(g) To protect your skin from splashes, spills and drips, always wear long pants and closed-toe shoes.

(h) Use appropriate ventilation when working with hazardous chemicals.

(i) Pipetting should never be done by mouth.

(j) Hands should be washed with soap and water immediately after working with any laboratory chemicals, even if gloves have been worn.

(k) Eating, drinking, smoking, gum chewing, applying cosmetics, and taking medicine in laboratories where hazardous chemicals are used or stored should be strictly prohibited.

(l) Food, beverages, cups, and other drinking and eating utensils should not be stored in areas where hazardous chemicals are handled or stored.

(m) Laboratory refrigerators, ice chests, cold rooms, and ovens should not be used for food storage or preparation.

(n) Contact the laboratory supervisor, Principal Investigator, CHO or EHS office with all safety questions or concerns.

(o) Know the location and proper use of safety equipment.

(p) Maintain situational awareness.

(q) Make others aware of special hazards associated with your work.

(r) Notify supervisors of chemical sensitivities or allergies.

(s) Report all injuries, accidents, incidents, and near misses.

(t) Unauthorized persons should not be allowed in the laboratory.

(u) Report unsafe conditions to the laboratory supervisor or CHO.

(v) Properly dispose of chemical wastes.

Working Alone in the Laboratory

Working alone in a laboratory is dangerous and should be strictly avoided. There have been many tragic accidents that illustrate this danger. Accidents are unexpected by definition, which is why coworkers should always be present. Workers should coordinate schedules to avoid working alone.

Housekeeping

Housekeeping can help reduce or eliminate a number of laboratory hazards. Proper housekeeping includes appropriate labeling and storage of chemicals, safe and regular cleaning of the facility, and proper arrangement of laboratory equipment.

2. Nanoparticles and Nanomaterials

Nanoparticles and nanomaterials have different reactivities and interactions with biological systems than bulk materials, and understanding and exploiting these differences is an active area of research. However, these differences also mean that the risks and hazards associated with exposure to engineered nanomaterials are not well known. Because this is an area of ongoing research, consult trusted sources for the most up to date information available. Note that the higher reactivity of many nanoscale materials suggests that they should be treated as potential sources of ignition, accelerants, and fuel that could result in fire or explosion. Easily dispersed dry nanomaterials may pose the greatest health hazard because of the risk of inhalation. Operations involving these nanomaterials deserve more attention and more stringent controls than those where the nanomaterials are embedded in solid or suspended in liquid matrixes.

Consideration should be given to all possible routes of exposure to nanomaterials including inhalation, ingestion, injection, and dermal contact (including eye and mucous membranes). Avoid handling nanomaterials in the open air in a freeparticle state. Whenever possible, handle and store dispersible nanomaterials, whether suspended in liquids or in a dry particle form, in closed (tightly-sealed) containers. Unless cutting or grinding occurs, nanomaterials that are not in a free form (encapsulated in a solid or a nanocomposite) typically will not require engineering controls. If a synthesis is being performed to create nanomaterials, it is not enough to only consider the final material in the risk assessment, but consider the hazardous properties of the precursor materials as well.

To minimize laboratory personnel exposure, conduct any work that could generate engineered nanoparticles in an enclosure that operates at a negative pressure differential compared to the laboratory personnel breathing zone. Limited data exist regarding the efficacy of PPE and ventilation systems against exposure to nanoparticles. However, until further information is available, it is prudent to follow standard chemical hygiene practices. Conduct a hazard evaluation to determine PPE appropriate for the level of hazard according to the requirements set forth in OSHA’s Personal Protective Equipment standard (29 CFR 1910.132).

3. Highly Toxic and Explosive/Reactive Chemicals/Materials

The use of highly toxic and explosive/ reactive chemicals and materials has been an area of growing concern. The frequency of academic laboratory incidents in the U.S. is an area of significant concern for the Chemical Safety Board (CSB). The CSB issued a case study on an explosion at Texas Tech University in Lubbock, Texas, which severely injured a graduate student handling a high-energy metal compound. Since 2001, the CSB has gathered preliminary information on 120 different university laboratory incidents that resulted in 87 evacuations, 96 injuries, and three deaths.

It is recommended that each facility keep a detailed inventory of highly toxic chemicals and explosive/reactive materials. There should be a record of the date of receipt, amount, location, and responsible individual for all acquisitions, syntheses, and disposal of these chemicals. A physical inventory should be performed annually to verify active inventory records. There should be a procedure in place to report security breaches, inventory discrepancies, losses, diversions, or suspected thefts.

Procedures for disposal of highly toxic materials should be established before any experiments begin, possibly even before the chemicals are ordered. The procedures should address methods for decontamination of any laboratory equipment that comes into contact with highly toxic chemicals. All waste should be accumulated in clearly labeled impervious containers that are stored in unbreakable secondary containment.

Highly reactive and explosive materials that may be used in the laboratory require appropriate procedures and training. An explosion can occur when a material undergoes a rapid reaction that results in a violent release of energy. Such reactions can happen spontaneously and can produce pressures, gases, and fumes that are hazardous. Some reagents pose a risk on contact with the atmosphere. It is prudent laboratory practice to use a safer alternative whenever possible.

If at all possible, substitutes for highly acute, chronic, explosive, or reactive chemicals should be considered prior to beginning work and used whenever possible.

4. Compressed Gas

Compressed gases expose laboratory personnel to both chemical and physical hazards. It is essential that these are monitored for leaks and have the proper labeling. By monitoring compressed gas inventories and disposing of or returning gases for which there is no immediate need, the laboratory can substantially reduce these risks. Leaking gas cylinders can cause serious hazards that may require an immediate evacuation of the area and activation of the emergency response system. Only appropriately trained hazmat responders may respond to stop a leaking gas cylinder under this situation.

F. Safety Recommendations—Physical Hazards

Physical hazards in the laboratory include combustible liquids, compressed gases, reactives, explosives and flammable chemicals, as well as high pressure/energy procedures, sharp objects and moving equipment. Injuries can result from bodily contact with rotating or moving objects, including mechanical equipment, parts, and devices. Personnel should not wear loosefitting clothing, jewelry, or unrestrained long hair around machinery with moving parts.

The Chemical Safety Board has identified the following key lessons for laboratories that address both physical and other hazards:

(1) Ensure that research-specific hazards are evaluated and then controlled by developing specific written protocols and training.

(2) Expand existing laboratory safety plans to ensure that all safety hazards, including physical hazards of chemicals, are addressed.

(3) Ensure that the organization’s EHS office reports directly to an identified individual/office with organizational authority to implement safety improvements.

(4) Develop a verification program that ensures that the safety provisions of the CHP are communicated, followed, and enforced at all levels within the organization.

(5) Document and communicate all laboratory near-misses and previous incidents to track safety, provide opportunities for education and improvement to drive safety changes at the university.

(6) Manage the hazards unique to laboratory chemical research in the academic environment. Utilize available practice guidance that identifies and describes methodologies to assess and control hazards.

(7) Written safety protocols and training are necessary to manage laboratory risk.

G. Emergency Planning

In addition to laboratory safety issues, laboratory personnel should be familiar with established facility policies and procedures regarding emergency situations. Topics may include, but are not limited to:

(1) Evacuation procedures—when it is appropriate and alternate routes;

(2) Emergency shutdown procedures—equipment shutdown and materials that should be stored safely;

(3) Communications during an emergency—what to expect, how to report, where to call or look for information;

(4) How and when to use a fire extinguisher;

(5) Security issues—preventing tailgating and unauthorized access;

(6) Protocol for absences due to travel restrictions or illness;

(7) Safe practices for power outage;

(8) Shelter in place—when it is appropriate;

(9) Handling suspicious mail or phone calls;

(10) Laboratory-specific protocols relating to emergency planning and response;

(11) Handling violent behavior in the workplace; and

(12) First-aid and CPR training, including automated external defibrillator training if available.

It is prudent that laboratory personnel are also trained in how to respond to short-term, long-term and large-scale emergencies. Laboratory security can play a role in reducing the likelihood of some emergencies and assisting in preparation and response for others. Every institution, department, and individual laboratory should consider having an emergency preparedness plan. The level of detail of the plan will vary depending on the function of the group and institutional planning efforts already in place.

Emergency planning is a dynamic process. As personnel, operations, and events change, plans will need to be updated and modified. To determine the type and level of emergency planning needed, laboratory personnel need to perform a vulnerability assessment. Periodic drills to assist in training and evaluation of the emergency plan are recommended as part of the training program.

H. Emergency Procedures

(1) Fire alarm policy. Most organizations use fire alarms whenever a building needs to be evacuated—for any reason. When a fire alarm sounds in the facility, evacuate immediately after extinguishing all equipment flames. Check on and assist others who may require help evacuating.

(2) Emergency safety equipment. The following safety elements should be met:

a. A written emergency action plan has been provided to workers;

b. Fire extinguishers, eyewash units, and safety showers are available and tested on a regular basis; and

c. Fire blankets, first-aid equipment, fire alarms, and telephones are available and accessible.

(3) Chemical spills. Workers should contact the CHO or EHS office for instructions before cleaning up a chemical spill. All SDS and label instructions should be followed, and appropriate PPE should be worn during spill cleanup.

(4) Accident procedures. In the event of an accident, immediately notify appropriate personnel and local emergency responders. Provide an SDS of any chemical involved to the attending physician. Complete an accident report and submit it to the appropriate office or individual within 24 hours.

(5) Employee safety training program. New workers should attend safety training before they begin any activities. Additional training should be provided when they advance in their duties or are required to perform a task for the first time. Training documents should be recorded and maintained. Training should include hands-on instruction of how to use safety equipment appropriately.

(6) Conduct drills. Practice building evacuations, including the use of alternate routes. Practice shelter-in-place, including plans for extended stays. Walk the fastest route from your work area to the nearest fire alarm, emergency eye wash and emergency shower. Learn how each is activated. In the excitement of an actual emergency, people rely on what they learned from drills, practice and training.

(7) Contingency plans. All laboratories should have long-term contingency plans in place (e.g., for pandemics). Scheduling, workload, utilities and alternate work sites may need to be considered.

I. Laboratory Security

Laboratory security has evolved in the past decade, reducing the likelihood of some emergencies and assisting in preparation and response for others. Most security measures are based on the laboratory’s vulnerability. Risks to laboratory security include, but are not limited to:

(1) Theft or diversion of chemicals, biologicals, and radioactive or proprietary materials, mission-critical or high-value equipment;

(2) Threats from activist groups;

(3) Intentional release of, or exposure to, hazardous materials;

(4) Sabotage or vandalism of chemicals or high-value equipment;

(5) Loss or release of sensitive information; and

(6) Rogue work or unauthorized laboratory experimentation. Security systems in the laboratory are used to detect and respond to a security breach, or a potential security breach, as well as to delay criminal activity by imposing multiple layered barriers of increasing stringency. A good laboratory security system will increase overall safety for laboratory personnel and the public, improve emergency preparedness by assisting with preplanning, and lower the organization’s liability by incorporating more rigorous planning, staffing, training, and command systems and implementing emergency communications protocols, drills, background checks, card access systems, video surveillance, and other measures. The security plan should clearly delineate response to security issues, including the coordination of institution and laboratory personnel with both internal and external responders.

[76 FR 33609, June 8, 2011; 77 FR 17888, March 26, 2012; 78 FR 4325, Jan. 22, 2013]

### Attachment B:  Memorandum 138-07 Hazard Communication Program

### Attachment C:  Hazard Analysis Worksheet

| Activity/facility description: VA Medical Center | Service/Group: |  |
| --- | --- | --- |
| Location: Lexington, KY | Date: |  |
| Contact: Vicki Richardson ext 5627 | Safety Office Contact: Sally Higgins ext 4176 |  |
| INSTRUCTIONS |  |  |
| The user will complete as much information as possible and forward this worksheet to the Safety Office (138-LD). The Safety Office will review and identify additional potential hazards to be addressed. This hazard analysis worksheet shall be retained by the Safety Office as a part of the facility’s risk assessment documentation. |  |  |
| SCOPE OF OPERATIONS / ACTIVITY DESCRIPTION |  |  |
| Description of Activity: |  |  |
| New activity, or Change to an existing activity. Describe change: |  |  |
| Equipment Description: |  |  |
| Chemicals/Materials: |  |  |
| Process Parameters (temperature/pressure): |  |  |
| SAFETY & HEALTH HAZARD REVIEW Check all that apply. Write in specific information describing hazard. Items indicated with an asterisk (*) are extremely hazardous. |  |  |
| Acids | Asbestos / lead (circle) concerns |  |
| Bases | Biohazards |  |
| Carcinogen* | Compressed gas |  |
| Confined spaces | Construction/maintenance activities |  |
| Cryogen | Destructive testing |  |
| Dusty material/atmosphere | Electrical (high voltage) |  |
| Electrical (low voltage) Max 50V | Energized electrical work |  |
| Explosive* | Extremely Hazardous Chemicals |  |
| Falls from elevation | Fire |  |
| Fire protection system modification | Flammable gas |  |
| Flammable liquid | Flammable solid |  |
| Forklift operation | Glassware |  |
| Handtools | High acute toxicity* |  |
| Highly toxic* | Hot work |  |
| Hydraulic systems | Ionizing radiation-generating devices |  |
| Ladders/scaffolds | Lasers. Specify number and type. |  |
| Lighting | Manlift operation |  |
| Manual materials handling/Ergonomic concerns | Animals |  |
| Noise | Non-ionizing radiation other than laser |  |
| Operating/rotating equipment | Organic peroxide |  |
| OSHA Chemical Specific Standard material | Oxidizer |  |
| Pressure vessels/systems | Pyrophoric* |  |
| Radioactive materials | Reproductive hazard* |  |
| Sensitizer | Toxic Toxic Metals |  |
| Unstable/reactive* | Other Thermal |  |
| ADDITIONAL ACTIONS REQUIRED? |  |  |
| Do existing Service procedures, programs and/or operating manuals address the hazards? No Yes |  |  |
| Special Work Permit Required? No Yes, Specify type(s). |  |  |
| Personal Protective Equipment (PPE) requirements (note this form serves a “certificate of hazard assessment”, per OSHA 1910.132): |  |  |
| Additional comments/recommendations: |  |  |
| SIGNATURE |  |  |
| Service Contact Signature |  | Date |
| Safety Contact Signature |  | Date |

### Attachment D:  Request for a VA Chemical Pickup Procedure

Request for a VA Chemical Pickup Procedure

Go to the Public drive via "My Computer".  First, open the "Safety Office" folder and then open its subfolder "Request for a VA Chemical Pickup".  This will give you access to the "Request for a VA Chemical Pickup" template (see circled xsn file below).  Double click on this file.  Microsoft InfoPath needs to be on your computer and you must have access to Outlook to submit this form.

Below is the initial page with instructions.

There is a choice of three different types of requests depending on what you want picked up.

Commercially Available Chemical Only:

User Generated Chemical Mixtures Only:

Commercially Available Chemical and User Generated Chemical Mixtures:

After completing the appropriate section, you can use the "Submit" button.

### Attachment E:  Memorandum No. 138-16 Hazardous Waste Managament

### Attachment F:   NIOSH Carcinogen List

Last Reviewed May 18, 2012                    http://www.cdc.gov/niosh/topics/cancer/npotocca.html

The following is a list of substances NIOSH considers to be potential occupational carcinogens.

A number of the carcinogen classifications deal with groups of substances: aniline and homologs, chromates, dintrotoluenes, arsenic and inorganic arsenic compounds, beryllium and beryllium compounds, cadmium compounds, nickel compounds, and crystalline forms of silica. There are also substances of variable or unclear chemical makeup that are considered carcinogens, coal tar pitch volatiles, coke oven emissions, diesel exhaust and environmental tobacco smoke.

Some of the potential carcinogens listed in this index may be re-evaluated by NIOSH as new data become available and the NIOSH recommendations on these carcinogens either as to their status as a potential occupational carcinogen or as to the appropriate recommended exposure limit may change.

| #### A Acetaldehyde 2-Acetylaminofluorene Acrylamide Acrylonitrile Aldrin 4-Aminodiphenyl Amitrole Aniline and homologs o-Anisidine p-Anisidine Arsenic and inorganic arsenic compounds Arsine Asbestos Asphalt fumes | #### C Cadmium dust and fume Captafol Captan Carbon black (exceeding 0.1% PAHs) Carbon tetrachloride Chlordane Chlorinated camphene Chlorodiphenyl (42% chlorine); class polychlorinated biphenyls Chlorodiphenyl (54% chlorine); class polychlorinated biphenyls Chloroform Chloromethyl methyl ether bis(Chloromethyl) ether B-Chloroprene Chromium, hexavalent [Cr(VI)] Chromyl chloride; class, chromium hexavalent Chrysene Coal tar pitch volatiles; class, coal tar products Coke oven emissions |
| --- | --- |
| #### B Benzene Benzidine Benzidine-based dyes Beryllium Butadiene tert-Butyl chromate; class, chromium hexavalent |  |
| #### D DDT (dichlorodiphenyltrichloroethane) Di-2-ethylhexyl phthalate (DEHP) 2,4-Diaminoanisoleo o-Dianisidine-based dyes 1,2-Dibromo-3-chloropropane (DBCP) Dichloroacetylene p-Dichlorobenzene 3,3'-Dichlorobenzidine Dichloroethyl ether 1,3-Dichloropropene Dieldrin Diesel exhaust Diglycidyl ether (DGE); class, glycidyl ethers 4-Dimethylaminoazobenzene Dimethyl carbomoyl chloride 1,1-Dimethylhydrazine; class, hydrazines Dimethyl sulfate Dinitrotoluene Dioxane | #### M Malonaldehyde Methoxychlor Methyl bromide; class, monohalomethanes Methyl chloride Methyl iodide; class, monohalomethanes Methyl hydrazine; class, hydrazines4,4'-Methylenebis(2-chloroaniline) (MBOCA) Methylene chloride 4,4-Methylenedianiline (MDA) |
| #### E-G Environmental tobacco smoke Epichlorohydrin Ethyl acrylate Ethylene dibromide Ehtylene dichloride Ethylene oxide Ethyleneimine Ethylene thiourea Formaldehyde Gallium arsenide Gasoline | #### N a-Naphylamine B-Naphylamine Nickel, metal, soluble, insoluble, and inorganic; class, nickel, inorganic Nickel carbonyl Nickel sulfide roasting 4-Nitrobiphenyl p-Nitrochlorobenzene 2-Nitronaphthalene 2-Nitropropane N-Nitrosodimethylamine |
| #### H-K Heptachlor Hexachlorobutadiene Hexachloroethane Hexamethyl phosphoric triamide (HMPA) Hydrazine Kepone | #### P Pentachloroethane; class, chloroethanes N-Phenyl-b-naphthylamine; class, b-naphthalene Phenyl glycidyl ether; class, glycidyl ethers Phenylhydrazine; class, hydrazines Propane Sultone B-Propiolactone Propylene dichloride Proplyene imine Propylene oxide |
| #### R-S Radon Rosin core solder, pyrolysis products (containing formaldehyde) Silica, crystalline cristobalite Silica, crystalline quartz Silica, crystalline tripoli Silica, crystalline tridymite silica, fused Soapstone, total dust silicates |  |
| #### T Tremolite silicates 2,3,7,8-Tetrachlorodibenzo-p-dioxin (TCDD) (dioxin) 1,1,2,2-Tetrachloroethane Tetrachloroethylene Titanium dioxide o-Tolidine-based dyes o-Tolidine Toluene diisocyanate (TDI) Toluene diamine (TDA) o-Toluidine p-Toluidine 1,1,2-Trichloroethane; class, chloroethanes Trichloroethylene 1,2,3-Trichloropropane |  |

### Attachment G: 29 CFR Part 1910, Subpart Z, Toxic and Hazardous Substances, OSHA

| 2,4,5-T 2,4-D (Dichlorophenoxyacetic acid) Acetaldehyde Acetic acid Acetic anhydride Acetone Acetonitrile Acetylene tetrabromide Acetylsalicylic acid (Aspirin) Acrolein Acrylamide Acrylic acid Aldrin Allyl alcohol Allyl chloride Allyl glycidyl ether Allyl propyl disulfide alpha-Alumina Aluminum metal Aluminum, alkyls Aluminum, pyro powders Aluminum, soluble salts Aluminum, welding fumes 2-Aminopyridine Amitrole Ammonia Ammonium chloride fume Ammonium sulfamate sec-Amyl acetate n-Amyl acetate Aniline and homologs Anisidine (o-, p- isomers) Antimony Antimony compounds ANTU (alpha-Naphthyl thiourea) Arsenic Arsine Atrazine Azinphos-methyl | Barium Barium sulfate Barium, soluble compounds Benomyl Benzene Benzoyl peroxide Benzyl chloride Beryllium Beryllium compounds, n.o.s. Bismuth telluride (Se doped) Bismuth telluride, undoped Borates, tetra, sodium salts, anhydrous Borates, tetra, sodium salts, decahydrate Borates, tetra, sodium salts, pentahydrate Boron oxide Boron tribromide Boron trifluoride Bromacil Bromine Bromine pentafluoride Bromoform Butadiene (1,3-Butadiene) Butane 2-Butanone (Methyl ethyl ketone) 2-Butoxyethanol n-Butyl acetate tert-Butyl acetate sec-Butyl acetate Butyl acrylate tert-Butyl alcohol sec-Butyl alcohol n-Butyl alcohol tert-Butyl chromate n-Butyl glycidyl ether (BGE) n-Butyl lactate Butyl mercaptan Butylamine (n-) o-sec-Butylphenol p-tert-Butyltoluene | Cadmium Cadmium fume Calcium carbonate Calcium cyanamide Calcium hydroxide Calcium oxide Calcium silicate Calcium sulfate Camphor, synthetic Caprolactam Captafol (Difolatan) Captan Carbaryl (Sevin) Carbofuran (Furadan) Carbon black Carbon dioxide Carbon disulfide Carbon monoxide Carbon tetrabromide Carbon tetrachloride Carbonyl fluoride Catechol (pyrocatechol) Cellulose Cesium hydroxide Chlordane Chlorinated camphene Chlorinated diphenyl oxide Chlorine Chlorine dioxide Chlorine trifluoride 1-Chloro-1-nitropropane 2-Chloro-6-(trichloromethyl)pyridine Chloroacetaldehyde alpha-Chloroacetophenone (Phenacyl chloride) Chloroacetyl chloride Chlorobenzene o-Chlorobenzylidene malonitrile Chlorobromomethane Chlorodifluoromethane Chlorodiphenyl (42% chlorine) (PCB) Chlorodiphenyl (54% chlorine) (PCB) Chloroform (Trichloromethane) Chloropentafluoroethane Chloropicrin Chloropicrin/methyl chloride beta-Chloroprene o-Chlorostyrene o-Chlorotoluene Chlorpyrifos Chromates Chromic acid | Chromium Chromium (III) compounds, soluble Chromium insoluble salts Clopidol Coal dust (greater than or equal to 5% SiO2), respirable quartz fraction Coal tar pitch volatiles Cobalt carbonyl Cobalt hydrocarbonyl Cobalt metal, dust and fume Copper Copper dusts and mists Cotton dust (raw) Crag herbicide (Sesone) Cresol, all isomers Crotonaldehyde Crotonaldehyde, (E)- Crufomate Cumene Cyanamide Cyanides Cyanogen Cyanogen chloride Cyclohexane Cyclohexanol Cyclohexanone Cyclohexene Cyclohexylamine Cyclonite Cyclopentadiene Cyclopentane Cyhexatin |
| --- | --- | --- | --- |
| Decaborane Demeton (Systox) Di-sec octyl phthalate (Di-2-ethylhexyl-phthalate) 2,6-Di-tert-butyl-p-cresol Diacetone alcohol (4-Hydroxy-4-methyl-2-pentanone) Diazinon Diazomethane Diborane Dibutyl phosphate Dibutyl phthalate 2-N-Dibutylaminoethanol Dichloro diphenyl trichloroethane (DDT) 1,1-Dichloro-1-nitroethane 1,3-Dichloro-5,5-dimethyl hydantoin Dichloroacetylene o-Dichlorobenzene p-Dichlorobenzene Dichlorodifluoromethane 1,1-Dichloroethane Dichloroethyl ether 1,2-Dichloroethylene Dichlorofluoromethane 1,3-Dichloropropene 2,2-Dichloropropionic acid 1,2-Dichlorotetrafluoroethane Dichlorvos (DDVP) Dicrotophos Dicyclopentadiene Dicyclopentadienyl iron Dieldrin Diethanolamine Diethyl ketone Diethyl phthalate Diethylamine 2-Diethylaminoethanol Diethylene triamine Difluorodibromomethane Diglycidyl ether (DGE) Diisobutylketone Diisopropylamine Dimethyl 1,2-dibromo-2,2-dichloroethyl phosphate Dimethyl acetamide Dimethyl aniline (N,N-dimethylaniline) 1,1-Dimethyl hydrazine Dimethyl phthalate Dimethyl sulfate Dimethylamine Dimethylformamide Dinitolmide (3,5-Dinitro-o-toluamide) Dinitro-o-cresol Dinitrobenzene (alpha-) | Dinitrobenzene (meta-) Dinitrobenzene (para-) Dinitrobenzene, all isomers Dinitrotoluene Dioxane (Diethylene dioxide) Dioxathion (Delnav) Diphenyl (Biphenyl) Diphenylamine Dipropyl ketone Dipropylene glycol, methyl ether Diquat Disulfiram Disulfoton Diuron Divinyl benzene Emery Endosulfan Endrin Epichlorohydrin EPN Ethanolamine Ethion 2-Ethoxyethanol 2-Ethoxyethyl acetate (Cellosolve acetate) Ethyl acrylate Ethyl alcohol (Ethanol) Ethyl amyl ketone (5-Methyl-3-heptanone) Ethyl benzene Ethyl bromide Ethyl butyl ketone (3-Heptanone) Ethyl chloride Ethyl ether Ethyl formate Ethyl mercaptan Ethyl silicate Ethylacetate Ethylamine Ethylene chlorohydrin Ethylene diamine Ethylene dibromide (1,2-Dibromoethane) Ethylene dichloride Ethylene glycol Ethylene glycol, dinitrate Ethylidene norbornene N-Ethylmorpholine | Fenaminphos Fensulfothion (Dasanit) Fenthion Ferbam Ferrovanadium dust Fluorides Fluorine Fluorotrichloromethane (Trichlorofluoromethane) Fonofos Formaldehyde Formamide Formic acid Furfural Furfuryl alcohol Gasoline Germanium tetrahydride Glutaraldehyde Glycerin mist Glycidol Grain dust (oat, wheat, barley) Graphite, natural Graphite, synthetic Gypsum Hafnium Heptachlor Heptane (n-Heptane) Hexachlorobutadiene Hexachlorocyclo-pentadiene Hexachloroethane Hexachloronaphthalene Hexafluoroacetone n-Hexane Hexane isomers 2-Hexanone (Methyl n-butyl ketone) Hexone (Methyl isobutyl ketone) sec-Hexyl acetate Hexylene glycol Hydrazine Hydrogen bromide Hydrogen chloride Hydrogen cyanide Hydrogen fluoride Hydrogen peroxide Hydrogen selenide Hydrogen sulfide Hydrogenated terphenyls Hydroquinone 2-Hydroxypropyl acrylate | Indene Indium Indium compounds, n.o.s. Iodine Iodoform Iron oxide fume Iron salts (soluble) Iron, pentacarbonyl- Isoamyl acetate Isoamyl alcohol (primary and secondary) Isobutyl acetate Isobutyl alcohol Isooctyl alcohol Isophorone Isophoronediisocyanate 2-Isopropoxyethanol Isopropyl acetate Isopropyl alcohol Isopropyl ether Isopropyl glycidyl ether (IGE) Isopropylamine N-Isopropylaniline |
| Magnesite Magnesium oxide fume Malathion Maleic anhydride Manganese Manganese cyclopentadienyl tricarbonyl Manganese fume Manganese tetroxide Mercury Mercury (organo) alkyl compounds Mesityl oxide Methacrylic acid Methomyl (Lannate) Methoxychlor 4-Methoxyphenol Methyl 2-cyanoacrylate Methyl acetate Methyl acetylene (Propyne) Methyl acetylene - Propadiene mixture (MAPP) Methyl acrylate Methyl acrylonitrile Methyl alcohol Methyl bromide (Bromomethane) Methyl cellosolve (2-methoxyethanol) Methyl cellosolve acetate (2-Methoxyethyl acetate) Methyl chloride Methyl chloroform (1,1,1-Trichloroethane) Methyl cyclopentadienyl manganese tricarbonyl Methyl demeton Methyl ethyl ketone peroxide (MEKP) Methyl formate Methyl hydrazine (Monomethyl hydrazine) Methyl iodide Methyl isoamyl ketone Methyl isocyanate Methyl isopropyl ketone Methyl methacrylate Methyl n-amyl ketone Methyl parathion Methyl silicate alpha-Methyl styrene Methylal (Dimethoxymethane) Methylamine Methylcyclohexane Methylcyclohexanol o-Methylcyclohexanone Methylene bis (4-cyclohexylisocyanate) | Methylene bisphenol isocyanate (MDI) Methylene chloride 4,4'-Methylenebis (2-chloroaniline) (MBOCA) Methylisobutyl carbinol Methylmercaptan Metribuzin Mica Molybdenum Molybdenum insoluble compounds Molybdenum soluble compounds Monocrotophos (Azodrin) Monomethylaniline Morpholine Naphtha (coal tar) Naphthalene Nickel Nickel carbonyl Nickel insoluble compounds Nickel soluble compounds Nicotine Nitric acid Nitric oxide p-Nitroaniline Nitrobenzene p-Nitrochlorobenzene Nitroethane Nitrogen dioxide Nitrogen trifluoride Nitroglycerin Nitromethane 2-Nitropropane 1-Nitropropane o-Nitrotoluene m-Nitrotoluene p-Nitrotoluene Nonane Octachloronaphthalene Octane Oil mist, mineral Osmium tetroxide Oxalic acid Oxygen difluoride Ozone | Paraffin wax fume Paraquat Paraquat Paraquat methosulfate Parathion Particulates not otherwise regulated Pentaborane Pentachloronaphthalene Pentachlorophenol Pentaerythritol Pentane 2-Pentanone (Methyl propyl ketone) Perchloroethylene (Tetrachloroethylene) Perchloryl fluoride Perlite Petroleum distillates (naphtha) (rubber solvent) Phenol Phenothiazine Phenyl ether Phenyl ether-Biphenyl mixture vapor Phenyl glycidyl ether (PGE) Phenyl mercaptan p-Phenylene diamine Phenylhydrazine Phenylphosphine Phorate Phosdrin (Mevinphos) Phosgene (Carbonyl chloride) Phosphine Phosphoric acid Phosphorus (yellow) Phosphorus oxychloride Phosphorus pentachloride Phosphorus pentasulfide Phosphorus trichloride Phthalic anhydride m-Phthalodinitrile Picloram Picric acid Pindone (2-pivalyl-1,3-indandione) Piperazine dihydrochloride Plaster of paris Platinum Platinum soluble salts Portland cement Potassium hydroxide Propane Propargyl alcohol Propionic acid Propoxur (Baygon) n-Propyl acetate | n-Propyl alcohol n-Propyl nitrate Propylene dichloride Propylene glycol dinitrate Propylene glycol monomethyl ether Propylene imine Propylene oxide Pyrethrum Pyridine Quinone Resorcinol Rhodium Rhodium soluble compounds Rhodium, insoluble compounds Ronnel Rosin core solder pyrolysis products, as formaldehyde Rotenone Rouge |
| Selenium Selenium compounds Selenium hexafluoride Silica, amorphous, diatomaceous earth, containing less than1% crystalline silica Silica, amorphous, precipitated and gel Silica, crystalline, tridymite Silica, fused Silica-crystalline, cristobalite Silica-crystalline, quartz Silica-crystalline, tripoli Silicon Silicon carbide Silicon tetrahydride Silver soluble compounds Silver, metal Soapstone Sodium azide Sodium bisulfite Sodium fluoroacetate Sodium hydroxide Sodium metabisulfite Starch Stibine Stoddard solvent Strychnine Styrene Subtilisins (proteolytic enzymes) Sucrose Sulfur dioxide Sulfur hexafluoride Sulfur monochloride Sulfur pentafluoride Sulfur tetrafluoride Sulfuric acid Sulfuryl fluoride Sulprofos | Talc (containing no asbestos) Tantalum metal Tantalum, oxide dusts TEDP (Sulfotep) Tellurium Tellurium compounds, n.o.s. Tellurium hexafluoride Temephos TEPP Terphenyls 1,1,2,2-Tetrachloro-1,2-difluoroethane 1,1,1,2-Tetrachloro-2,2-difluoroethane 1,1,2,2-Tetrachloroethane Tetrachloronaphthalene Tetraethyllead Tetrahydrofuran Tetramethyl lead Tetramethyl succinonitrile Tetranitromethane Tetrasodium pyrophosphate Tetryl (2,4,6-Trinitro-phenylmethylnitramine) Thallium soluble compounds Thallium soluble compounds 4,4'-Thiobis (6-tert-butyl-m-cresol) Thioglycolic acid Thionyl chloride Thiram Tin Tin inorganic compounds Tin organic compounds Tin oxide Titanium dioxide Toluene Toluene 2,4-diisocyanate (TDI) p-Toluidine o-Toluidine m-Toluidine Tributyl phosphate 1,1,2-Trichloro-1,2,2-trifluoroethane Trichloroacetic acid 1,2,4-Trichlorobenzene 1,1,2-Trichloroethane Trichloroethylene Trichloromethanesulphenyl chloride Trichloronaphthalene 1,2,3-Trichloropropane Triethylamine Trifluorobromomethane Trimellitic anhydride Trimethyl benzene Trimethyl phosphite Trimethylamine | 2,4,6-Trinitrotoluene (TNT) Triorthocresyl phosphate Triphenyl amine Triphenyl phosphate Tungsten Tungsten, insoluble compounds Tungsten, soluble compounds Turpentine Uranium Uranium insoluble compounds Uranium soluble compounds n-Valeraldehyde Vanadium Vegetable oil mist Vinyl acetate Vinyl bromide Vinyl cyclohexene dioxide Vinyl toluene Vinylidene chloride (1,1-Dichloroethylene) VM&P Naphtha Warfarin Welding fumes (total particulate) Wood dust, all soft and hard woods, except western red cedar Wood dust, western red cedar m-Xylene-alpha, alpha'-diamine Xylenes (o-, m-, p- isomers) Xylidine Yttrium Zinc chloride fume Zinc chromate Zinc oxide Zinc stearate Zirconium Zirconium compounds, n.o.s. |  |

### Attachment H:  International Agency for Research on Cancer (Groups 1, 2a, and 2b)

Last update: 30 October 2013

http://monographs.iarc.fr/ENG/Classification/ClassificationsGroupOrder.pdf

### Attachment I:  National Toxicology Program Report on Carcinogens (RoC)-12th RoC (2011)

http://ntp.niehs.nih.gov/ntp/roc/twelfth/ListedSubstancesKnown.pdf

Known To Be Human Carcinogens

### Attachment J:  National Toxicology Program 12th Report on Carcinogens "Reasonably anticipated to be human carcinogens"

http://ntp.niehs.nih.gov/ntp/roc/twelfth/ListedSubstancesReasonablyAnticipated.pdf

Acetaldehyde

2 Acetylaminofluorene

Acrylamide

Acrylonitrile

Adriamycin

2 Aminoanthraquinone

o Aminoazotoluene

1-Amino-2,4-dibromoanthraquinone

2 Amino-3,4-dimethylimidazo[4,5 f]quinoline (see Heterocyclic Amines [Selected])

2 Amino-3,8-dimethylimidazo[4,5 f]quinoxaline (see Heterocyclic Amines [Selected])

1 Amino-2-methylanthraquinone

2 Amino-3-methylimidazo[4,5 f]quinoline (see Heterocyclic Amines [Selected])

2 Amino-1-methyl-6-phenylimidazo[4,5 b]pyridine (see Heterocyclic Amines [Selected])

Amitrole

o-Anisidine and Its Hydrochloride

Azacitidine

Basic Red 9 Monohydrochloride

Benz[a]anthracene (see Polycyclic Aromatic Hydrocarbons: 15 Listings)

Benzo[b]fluoranthene (see Polycyclic Aromatic Hydrocarbons: 15 Listings)

Benzo[j]fluoranthene (see Polycyclic Aromatic Hydrocarbons: 15 Listings)

Benzo[k]fluoranthene (see Polycyclic Aromatic Hydrocarbons: 15 Listings)

Benzo[a]pyrene (see Polycyclic Aromatic Hydrocarbons: 15 Listings)

Benzotrichloride

2,2-Bis(bromomethyl)-1,3-propanediol (Technical Grade)

Bis(chloroethyl) Nitrosourea (see Nitrosourea Chemotherapeutic Agents)

Bromodichloromethane

1,4-Butanediol Dimethanesulfonate

Butylated Hydroxyanisole

Captafol

Carbon Tetrachloride

Ceramic Fibers (Respirable Size)

Chloramphenicol

Chlorendic Acid

Chlorinated Paraffins (C12, 60% Chlorine)

Chloroform

1-(2-Chloroethyl)-3-cyclohexyl-1-nitrosourea (see Nitrosourea Chemotherapeutic Agents)

3-Chloro-2-methylpropene

4-Chloro-o-phenylenediamine

Chloroprene

p-Chloro-o-toluidine and Its Hydrochloride

Chlorozotocin (see Nitrosourea Chemotherapeutic Agents)

Cisplatin

Cobalt Sulfate

Cobalt–Tungsten Carbide: Powders and Hard Metals

p Cresidine

Cupferron

Dacarbazine

Danthron

2,4 Diaminoanisole Sulfate

2,4 Diaminotoluene

Diazoaminobenzene

Dibenz[a,h]acridine (see Polycyclic Aromatic Hydrocarbons: 15 Listings)

Dibenz[a,j]acridine (see Polycyclic Aromatic Hydrocarbons: 15 Listings)

Dibenz[a,h]anthracene (see Polycyclic Aromatic Hydrocarbons: 15 Listings)

7H-Dibenzo[c,g]carbazole (see Polycyclic Aromatic Hydrocarbons: 15 Listings)

Dibenzo[a,e]pyrene (see Polycyclic Aromatic Hydrocarbons: 15 Listings)

Dibenzo[a,h]pyrene (see Polycyclic Aromatic Hydrocarbons: 15 Listings)

Dibenzo[a,i]pyrene (see Polycyclic Aromatic Hydrocarbons: 15 Listings)

Dibenzo[a,l]pyrene (see Polycyclic Aromatic Hydrocarbons: 15 Listings)

1,2-Dibromo-3-chloropropane

1,2-Dibromoethane

2,3-Dibromo-1-propanol

1,4-Dichlorobenzene

3,3′-Dichlorobenzidine and Its Dihydrochloride

Dichlorodiphenyltrichloroethane

1,2-Dichloroethane

Dichloromethane

1,3-Dichloropropene (Technical Grade)

Diepoxybutane

Diesel Exhaust Particulates

Di(2-ethylhexyl) Phthalate

Diethyl Sulfate

Diglycidyl Resorcinol Ether

3,3′-Dimethoxybenzidine (see 3,3′-Dimethoxybenzidine and Dyes Metabolized to 3,3′-Dimethoxybenzidine)

4 Dimethylaminoazobenzene

3,3′-Dimethylbenzidine (see 3,3′-Dimethylbenzidine and Dyes Metabolized to 3,3′-Dimethylbenzidine)

Dimethylcarbamoyl Chloride

1,1 Dimethylhydrazine

Dimethyl Sulfate

Dimethylvinyl Chloride

1,6-Dinitropyrene (see Nitroarenes [Selected])

1,8-Dinitropyrene (see Nitroarenes [Selected])

1,4-Dioxane

Disperse Blue 1

Dyes Metabolized to 3,3′-Dimethoxybenzidine (3,3′-Dimethoxybenzidine Dye Class) (see 3,3′-Dimethoxybenzidine and Dyes Metabolized to 3,3′-Dimethoxybenzidine)

Dyes Metabolized to 3,3′-Dimethylbenzidine (3,3′-Dimethylbenzidine Dye Class) (see 3,3′-Dimethylbenzidine and Dyes Metabolized to 3,3′-Dimethylbenzidine)

Epichlorohydrin

Ethylene Thiourea

Ethyl Methanesulfonate

Furan

Glass Wool Fibers (Inhalable), Certain

Glycidol

Hexachlorobenzene

Hexachloroethane

Hexamethylphosphoramide

Hydrazine and Hydrazine Sulfate

Hydrazobenzene

Indeno[1,2,3-cd]pyrene (see Polycyclic Aromatic Hydrocarbons: 15 Listings)

Iron Dextran Complex

Isoprene

Kepone

Lead and Lead Compounds

Lindane, Hexachlorocyclohexane (Technical Grade), and Other Hexachlorocyclohexane Isomers

2-Methylaziridine

5-Methylchrysene (see Polycyclic Aromatic Hydrocarbons: 15 Listings)

4,4′-Methylenebis(2-chloroaniline)

4,4′-Methylenebis(N,N-dimethyl)benzenamine

4,4′ Methylenedianiline and Its Dihydrochloride

Methyleugenol

Methyl Methanesulfonate

N-Methyl-N′-Nitro-N-Nitrosoguanidine (see N-Nitrosamines: 15 Listings)

Metronidazole

Michler’s Ketone

Mirex

Naphthalene

Nickel, Metallic (see Nickel Compounds and Metallic Nickel)

Nitrilotriacetic Acid

o-Nitroanisole

Nitrobenzene

6-Nitrochrysene (see Nitroarenes [Selected])

Nitrofen

Nitrogen Mustard Hydrochloride

Nitromethane

2-Nitropropane

1-Nitropyrene (see Nitroarenes [Selected])

4-Nitropyrene (see Nitroarenes [Selected])

N-Nitrosodi-n-butylamine (see N-Nitrosamines: 15 Listings)

N-Nitrosodiethanolamine (see N-Nitrosamines: 15 Listings)

N-Nitrosodiethylamine (see N-Nitrosamines: 15 Listings)

N-Nitrosodimethylamine (see N-Nitrosamines: 15 Listings)

N-Nitrosodi-n-propylamine (see N-Nitrosamines: 15 Listings)

N-Nitroso-N-ethylurea (see N-Nitrosamines: 15 Listings)

4-(N-Nitrosomethylamino)-1-(3-pyridyl)-1-butanone (see N-Nitrosamines: 15 Listings)

N-Nitroso-N-methylurea (see N-Nitrosamines: 15 Listings)

N-Nitrosomethylvinylamine (see N-Nitrosamines: 15 Listings)

N-Nitrosomorpholine (see N-Nitrosamines: 15 Listings)

N-Nitrosonornicotine (see N-Nitrosamines: 15 Listings)

N-Nitrosopiperidine (see N-Nitrosamines: 15 Listings)

N-Nitrosopyrrolidine (see N-Nitrosamines: 15 Listings)

N-Nitrososarcosine (see N-Nitrosamines: 15 Listings)

o-Nitrotoluene

Norethisterone

Ochratoxin A

4,4′ Oxydianiline

Oxymetholone

Phenacetin (see Phenacetin and Analgesic Mixtures Containing Phenacetin)

Phenazopyridine Hydrochloride

Phenolphthalein

Phenoxybenzamine Hydrochloride

Phenytoin and Phenytoin Sodium

Polybrominated Biphenyls

Polychlorinated Biphenyls

Procarbazine and Its Hydrochloride

Progesterone

1,3 Propane Sultone

β Propiolactone

Propylene Oxide

Propylthiouracil

Reserpine

Riddelliine

Safrole

Selenium Sulfide

Streptozotocin (see Nitrosourea Chemotherapeutic Agents)

Styrene

Styrene-7,8-oxide

Sulfallate

Tetrachloroethylene

Tetrafluoroethylene

Tetranitromethane

Thioacetamide

4,4′-Thiodianiline

Thiourea

Toluene Diisocyanates

o Toluidine and Its Hydrochloride

Toxaphene

Trichloroethylene

2,4,6-Trichlorophenol

1,2,3-Trichloropropane

Tris(2,3-dibromopropyl) Phosphate

Ultraviolet Radiation A (see Ultraviolet Radiation Related Exposures)

Ultraviolet Radiation B (see Ultraviolet Radiation Related Exposures)

Ultraviolet Radiation C (see Ultraviolet Radiation Related Exposures)

Urethane

Vinyl Bromide (see Vinyl Halides [Selected])

4-Vinyl-1-cyclohexene Diepoxide

Vinyl Fluoride (see Vinyl Halides [Selected])

### Attachment K:  Target Organ Poster

### Attachment L:  Chemical Incompatibility Chart

### Attachment M:  Using CEOSH to pull up SDS’s

4.  When this screen opens, select “SDS Search”

5.  When this screen opens, click on “+My Locations”

6.  Click on VISN 09

7.  Click on Lexington, KY –

8.  Click on Pathology & Laboratory Medicine

a.    At this time, you may choose an individual department by clicking on

it; however, just selecting Pathology & Laboratory Medicine  will

allow you to bring up all chemicals in our service

9.  Type in the chemical name you would like to search for in the rectangle next to

the word “contains”

10. Click on “Search”

11.  After clicking on “Search”, a screen with your choices will come up.

12.  Click on “Action” next to the chemical you would like to select

13.  At this time, a pop up box appears with 6 choices.

14.  If you would like to view the SDS for the chemical you selected,

Click on “View (M)SDS & Attachments.

15.  Another pop up box will appear.  At this time you will have the option of

viewing, emailing or faxing the SDS.  Click on “View” if you wish to view

the SDS

16.  The SDS appears on the screen.  At this time you may view it, save it or

print it.

### Attachment N:  Printing Chemical labels using the 3E SDS Library

1.  Go through steps 1-13 in attachment O (previous attachment)

2.  If you would like to print labels for the chemical, click on “Labels”

3.  Select United States

4.  Select NFPA/HMIS

5.  Click on “Process Label”

6.  1 – Review the label data, gives you the option to edit the information

7.  2-  Choose label placement, tells you what kind of labels to load and gives you

the opportunity to print only the amount of labels you need.

8.  The number of labels default will be 10 labels.  If you don’t need 10 labels,

uncheck the amount not needed.

9.  Click on “View Label” for a print preview.

10. If everything is okay, load the required labels and print the labels using the

color printer (LEX-PTPLM02_COLOR) in B112.

### Attachment O:  Formaldehyde VA Medical Center Lexington, KY  SOP-ENG-138-12 (Formerly Memorandum 138-31)

### Attachment P:  Standard, 29CFR 1910.1048 and Appendices A, C, D, and E

• Part Number: 1910

• Part Title: Occupational Safety and Health Standards

• Subpart: Z

• Subpart Title: Toxic and Hazardous Substances

• Standard Number: 1910.1048

• Title: Formaldehyde.

• Appendix: A ,   B ,   C ,   D ,   E

--------------------------------------------------------------------------------

Note: The following standard has been updated to reflect the final rule that was issued on March 26, 2012 and became effective on May 25, 2012. See the e-CFR 1910.1048 and the Federal Register* references.

1910.1048(a)

Scope and application. This standard applies to all occupational exposures to formaldehyde, i.e. from formaldehyde gas, its solutions, and materials that release formaldehyde.

1910.1048(b)

Definitions. For purposes of this standard, the following definitions shall apply:

Action level means a concentration of 0.5 part formaldehyde per million parts of air (0.5 ppm) calculated as an eight (8)-hour time-weighted average (TWA) concentration.

Assistant Secretary means the Assistant Secretary of Labor for the Occupational Safety and Health Administration, U.S. Department of Labor, or designee.

Authorized Person means any person required by work duties to be present in regulated areas, or authorized to do so by the employer, by this section, or by the OSH Act of 1970.

Director means the Director of the National Institute for Occupational Safety and Health, U.S. Department of Health and Human Services, or designee.

Emergency is any occurrence, such as but not limited to equipment failure, rupture of containers, or failure of control equipment that results in an uncontrolled release of a significant amount of formaldehyde.

Employee exposure means the exposure to airborne formaldehyde which would occur without corrections for protection provided by any respirator that is in use.

Formaldehyde means the chemical substance, HCHO, Chemical Abstracts Service Registry No. 50-00-0.

1910.1048(c)

Permissible Exposure Limit (PEL) -

1910.1048(c)(1)

TWA: The employer shall assure that no employee is exposed to an airborne concentration of formaldehyde which exceeds 0.75 parts formaldehyde per million parts of air (0.75 ppm) as an 8-hour TWA.

1910.1048(c)(2)

Short Term Exposure Limit (STEL): The employer shall assure that no employee is exposed to an airborne concentration of formaldehyde which exceeds two parts formaldehyde per million parts of air (2 ppm) as a 15-minute STEL.

1910.1048(d)

Exposure monitoring -

1910.1048(d)(1)

General.

1910.1048(d)(1)(i)

Each employer who has a workplace covered by this standard shall monitor employees to determine their exposure to formaldehyde.

1910.1048(d)(1)(ii)

Exception. Where the employer documents, using objective data, that the presence of formaldehyde or formaldehyde-releasing products in the workplace cannot result in airborne concentrations of formaldehyde that would cause any employee to be exposed at or above the action level or the STEL under foreseeable conditions of use, the employer will not be required to measure employee exposure to formaldehyde.

1910.1048(d)(1)(iii)

When an employee's exposure is determined from representative sampling, the measurements used shall be representative of the employee's full shift or short-term exposure to formaldehyde, as appropriate.

1910.1048(d)(1)(iv)

Representative samples for each job classification in each work area shall be taken for each shift unless the employer can document with objective data that exposure levels for a given job classification are equivalent for different work shifts.

1910.1048(d)(2)

Initial monitoring. The employer shall identify all employees who may be exposed at or above the action level or at or above the STEL and accurately determine the exposure of each employee so identified.

1910.1048(d)(2)(i)

Unless the employer chooses to measure the exposure of each employee potentially exposed to formaldehyde, the employer shall develop a representative sampling strategy and measure sufficient exposures within each job classification for each workshift to correctly characterize and not underestimate the exposure of any employee within each exposure group.

1910.1048(d)(2)(ii)

The initial monitoring process shall be repeated each time there is a change in production, equipment, process, personnel, or control measures which may result in new or additional exposure to formaldehyde.

1910.1048(d)(2)(iii)

If the employer receives reports of signs or symptoms of respiratory or dermal conditions associated with formaldehyde exposure, the employer shall promptly monitor the affected employee's exposure.

1910.1048(d)(3)

Periodic monitoring.

1910.1048(d)(3)(i)

The employer shall periodically measure and accurately determine exposure to formaldehyde for employees shown by the initial monitoring to be exposed at or above the action level or at or above the STEL.

1910.1048(d)(3)(ii)

If the last monitoring results reveal employee exposure at or above the action level, the employer shall repeat monitoring of the employees at least every 6 months.

1910.1048(d)(3)(iii)

If the last monitoring results reveal employee exposure at or above the STEL, the employer shall repeat monitoring of the employees at least once a year under worst conditions.

1910.1048(d)(4)

Termination of monitoring. The employer may discontinue periodic monitoring for employees if results from two consecutive sampling periods taken at least 7 days apart show that employee exposure is below the action level and the STEL. The results must be statistically representative and consistent with the employer's knowledge of the job and work operation.

1910.1048(d)(5)

Accuracy of monitoring. Monitoring shall be accurate, at the 95 percent confidence level, to within plus or minus 25 percent for airborne concentrations of formaldehyde at the TWA and the STEL and to within plus or minus 35 percent for airborne concentrations of formaldehyde at the action level.

1910.1048(d)(6)

Employee notification of monitoring results. The employer must, within 15 working days after the receipt of the results of any monitoring performed under this section, notify each affected employee of these results either individually in writing or by posting the results in an appropriate location that is accessible to employees. If employee exposure is above the PEL, affected employees shall be provided with a description of the corrective actions being taken by the employer to decrease exposure.

1910.1048(d)(7)

Observation of monitoring.

1910.1048(d)(7)(i)

The employer shall provide affected employees or their designated representatives an opportunity to observe any monitoring of employee exposure to formaldehyde required by this standard.

1910.1048(d)(7)(ii)

When observation of the monitoring of employee exposure to formaldehyde requires entry into an area where the use of protective clothing or equipment is required, the employer shall provide the clothing and equipment to the observer, require the observer to use such clothing and equipment, and assure that the observer complies with all other applicable safety and health procedures.

1910.1048(e)

Regulated areas.

1910.1048(e)(1)

Signs.

1910.1048(e)(1)(i)

The employer shall establish regulated areas where the concentration of airborne formaldehyde exceeds either the TWA or the STEL and post all entrances and access ways with signs bearing the following legend:

DANGER

FORMALDEHYDE

MAY CAUSE CANCER

CAUSES SKIN, EYE, AND RESPIRATORY IRRITATION

AUTHORIZED PERSONNEL ONLY

1910.1048(e)(1)(ii)

Prior to June 1, 2016, employers may use the following legend in lieu of that specified in paragraph (e)(1)(i) of this section:

DANGER

FORMALDEHYDE

IRRITANT AND POTENTIAL CANCER HAZARD

AUTHORIZED PERSONNEL ONLY

1910.1048(e)(2)

The employer shall limit access to regulated areas to authorized persons who have been trained to recognize the hazards of formaldehyde.

1910.1048(e)(3)

An employer at a multiemployer worksite who establishes a regulated area shall communicate the access restrictions and locations of these areas to other employers with work operations at that worksite.

1910.1048(f)

Methods of compliance -

1910.1048(f)(1)

Engineering controls and work practices. The employer shall institute engineering and work practice controls to reduce and maintain employee exposures to formaldehyde at or below the TWA and the STEL.

1910.1048(f)(2)

Exception. Whenever the employer has established that feasible engineering and work practice controls cannot reduce employee exposure to or below either of the PELs, the employer shall apply these controls to reduce employee exposures to the extent feasible and shall supplement them with respirators which satisfy this standard.

1910.1048(g)

Respiratory protection.

1910.1048(g)(1)

General. For employees who use respirators required by this section, the employer must provide each employee an appropriate respirator that complies with the requirements of this paragraph. Respirators must be used during:

1910.1048(g)(1)(i)

Periods necessary to install or implement feasible engineering and work-practice controls.

1910.1048(g)(1)(ii)

Work operations, such as maintenance and repair activities or vessel cleaning, for which the employer establishes that engineering and work-practice controls are not feasible.

1910.1048(g)(1)(iii)

Work operations for which feasible engineering and work- practice controls are not yet sufficient to reduce employee exposure to or below the PELs.

1910.1048(g)(1)(iv)

Emergencies.

1910.1048(g)(2)

Respirator program.

1910.1048(g)(2)(i)

The employer must implement a respiratory protection program in accordance with § 1910.134(b) through (d) (except (d)(1)(iii), (d)(3)(iii)(b)(1), and (2)), and (f) through (m), which covers each employee required by this section to use a respirator.

1910.1048(g)(2)(ii)

When employees use air-purifying respirators with chemical cartridges or canisters that do not contain end-of-service-life indicators approved by the National Institute for Occupational Safety and Health, employers must replace these cartridges or canisters as specified by paragraphs (d)(3)(iii)(B)(1) and (B)(2) of 29 CFR 1910.134, or at the end of the workshift, whichever condition occurs first.

1910.1048(g)(2)(ii)(A)

Replace the cartridge after three (3) hours of use or at the end of the workshift, whichever occurs first, unless the cartridge contains a NIOSH-approved end-of-service-life indicator (ESLI) to show when breakthrough occurs.

1910.1048(g)(2)(ii)(B)

Unless the canister contains a NIOSH-approved ESLI to show when breakthrough occurs, replace canisters used in atmospheres up to 7.5 ppm (10xPEL) every four (4) hours and industrial-sized canisters used in atmospheres up to 75 ppm (100xPEL) every two (2) hours, or at the end of the workshift, whichever occurs first.

1910.1048(g)(3)

Respirator selection.

1910.1048(g)(3)(i)

Employers must:

1910.1048(g)(3)(i)(A)

Select, and provide to employees, the appropriate respirators specified in paragraph (d)(3)(i)(A) of 29 CFR 1910.134.

1910.1048(g)(3)(i)(B)

Equip each air-purifying, full facepiece respirator with a canister or cartridge approved for protection against formaldehyde.

1910.1048(g)(3)(i)(C)

For escape, provide employees with one of the following respirator options: A self-contained breathing apparatus operated in the demand or pressure-demand mode; or a full facepiece respirator having a chin-style, or a front-or back-mounted industrial-size, canister or cartridge approved for protection against formaldehyde.

1910.1048(g)(3)(ii)

Employers may substitute an air-purifying, half mask respirator for an air-purifying, full facepiece respirator when they equip the half mask respirator with a cartridge approved for protection against formaldehyde and provide the affected employee with effective gas-proof goggles.

1910.1048(g)(3)(iii)

Employers must provide employees who have difficulty using negative pressure respirators with powered air-purifying respirators permitted for use under paragraph (g)(3)(i)(A) of this standard and that affords adequate protection against formaldehyde exposures.

1910.1048(h)

Protective equipment and clothing. Employers shall comply with the provisions of 29 CFR 1910.132 and 29 CFR 1910.133. When protective equipment or clothing is provided under these provisions, the employer shall provide these protective devices at no cost to the employee and assure that the employee wears them.

1910.1048(h)(1)

Selection. The employer shall select protective clothing and equipment based upon the form of formaldehyde to be encountered, the conditions of use, and the hazard to be prevented.

1910.1048(h)(1)(i)

All contact of the eyes and skin with liquids containing 1 percent or more formaldehyde shall be prevented by the use of chemical protective clothing made of material impervious to formaldehyde and the use of other personal protective equipment, such as goggles and face shields, as appropriate to the operation.

1910.1048(h)(1)(ii)

Contact with irritating or sensitizing materials shall be prevented to the extent necessary to eliminate the hazard.

1910.1048(h)(1)(iii)

Where a face shield is worn, chemical safety goggles are also required if there is a danger of formaldehyde reaching the area of the eye.

1910.1048(h)(1)(iv)

Full body protection shall be worn for entry into areas where concentrations exceed 100 ppm and for emergency reentry into areas of unknown concentration.

1910.1048(h)(2)

Maintenance of protective equipment and clothing.

1910.1048(h)(2)(i)

The employer shall assure that protective equipment and clothing that has become contaminated with formaldehyde is cleaned or laundered before its reuse.

1910.1048(h)(2)(ii)

When formaldehyde-contaminated clothing and equipment is ventilated, the employer shall establish storage areas so that employee exposure is minimized.

1910.1048(h)(2)(ii)(A)

Signs. Storage areas for contaminated clothing and equipment shall have signs bearing the following legend:

DANGER

FORMALDEHYDE-CONTAMINATED [CLOTHING] EQUIPMENT

MAY CAUSE CANCER

CAUSES SKIN, EYE AND RESPIRATORY IRRITATION

DO NOT BREATHE VAPOR

DO NOT GET ON SKIN

1910.1048(h)(2)(ii)(B)

Labels. The employer shall ensure containers for contaminated clothing and equipment are labeled consistent with the Hazard Communication Standard, Sec. 1910.1200, and shall, as a minimum, include the following:

DANGER

FORMALDEHYDE-CONTAMINATED [CLOTHING] EQUIPMENT

MAY CAUSE CANCER

CAUSES SKIN, EYE, AND RESPIRATORY IRRITATION

DO NOT BREATHE VAPOR

DO NOT GET ON SKIN

1910.1048(h)(2)(ii)(C)

Prior to June 1, 2016, employers may use the following legend in lieu of that specified in paragraph (h)(2)(ii)(A) of this section:

DANGER

FORMALDEHYDE-CONTAMINATED [CLOTHING] EQUIPMENT

AVOID INHALATION AND SKIN CONTACT

1910.1048(h)(2)(ii)(D)

Prior to June 1, 2015, employers may include the following information on containers of protective clothing and equipment in lieu of the labeling requirements in paragraphs (h)(2)(ii)(B) of this section:

DANGER

FORMALDEHYDE-CONTAMINATED [CLOTHING] EQUIPMENT

AVOID INHALATION AND SKIN CONTACT

1910.1048(h)(2)(iii)

The employer shall assure that only persons trained to recognize the hazards of formaldehyde remove the contaminated material from the storage area for purposes of cleaning, laundering, or disposal.

1910.1048(h)(2)(iv)

The employer shall assure that no employee takes home equipment or clothing that is contaminated with formaldehyde.

1910.1048(h)(2)(v)

The employer shall repair or replace all required protective clothing and equipment for each affected employee as necessary to assure its effectiveness.

1910.1048(h)(2)(vi)

The employer shall inform any person who launders, cleans, or repairs such clothing or equipment of formaldehyde's potentially harmful effects and of procedures to safely handle the clothing and equipment.

1910.1048(i)

Hygiene protection.

1910.1048(i)(1)

The employer shall provide change rooms, as described in 29 CFR 1910.141 for employees who are required to change from work clothing into protective clothing to prevent skin contact with formaldehyde.

1910.1048(i)(2)

If employees' skin may become splashed with solutions containing 1 percent or greater formaldehyde, for example, because of equipment failure or improper work practices, the employer shall provide conveniently located quick drench showers and assure that affected employees use these facilities immediately.

1910.1048(i)(3)

If there is any possibility that an employee's eyes may be splashed with solutions containing 0.1 percent or greater formaldehyde, the employer shall provide acceptable eyewash facilities within the immediate work area for emergency use.

1910.1048(j)

Housekeeping. For operations involving formaldehyde liquids or gas, the employer shall conduct a program to detect leaks and spills, including regular visual inspections.

1910.1048(j)(1)

Preventative maintenance of equipment, including surveys for leaks, shall be undertaken at regular intervals.

1910.1048(j)(2)

In work areas where spillage may occur, the employer shall make provisions to contain the spill, to decontaminate the work area, and to dispose of the waste.

1910.1048(j)(3)

The employer shall assure that all leaks are repaired and spills are cleaned promptly by employees wearing suitable protective equipment and trained in proper methods for cleanup and decontamination.

1910.1048(j)(4)

Formaldehyde-contaminated waste and debris resulting from leaks or spills shall be placed for disposal in sealed containers bearing a label warning of formaldehyde's presence and of the hazards associated with formaldehyde. The employer shall ensure that the labels are in accordance with paragraph (m) of this section.

1910.1048(k)

Emergencies. For each workplace where there is the possibility of an emergency involving formaldehyde, the employer shall assure appropriate procedures are adopted to minimize injury and loss of life. Appropriate procedures shall be implemented in the event of an emergency.

1910.1048(l)

Medical surveillance -

1910.1048(l)(1)

Employees covered.

1910.1048(l)(1)(i)

The employer shall institute medical surveillance programs for all employees exposed to formaldehyde at concentrations at or exceeding the action level or exceeding the STEL.

1910.1048(l)(1)(ii)

The employer shall make medical surveillance available for employees who develop signs and symptoms of overexposure to formaldehyde and for all employees exposed to formaldehyde in emergencies. When determining whether an employee may be experiencing signs and symptoms of possible overexposure to formaldehyde, the employer may rely on the evidence that signs and symptoms associated with formaldehyde exposure will occur only in exceptional circumstances when airborne exposure is less than 0.1 ppm and when formaldehyde is present in material in concentrations less than 0.1 percent.

1910.1048(l)(2)

Examination by a physician. All medical procedures, including administration of medical disease questionnaires, shall be performed by or under the supervision of a licensed physician and shall be provided without cost to the employee, without loss of pay, and at a reasonable time and place.

1910.1048(l)(3)

Medical disease questionnaire. The employer shall make the following medical surveillance available to employees prior to assignment to a job where formaldehyde exposure is at or above the action level or above the STEL and annually thereafter. The employer shall also make the following medical surveillance available promptly upon determining that an employee is experiencing signs and symptoms indicative of possible overexposure to formaldehyde.

1910.1048(l)(3)(i)

Administration of a medical disease questionnaire, such as in Appendix D, which is designed to elicit information on work history, smoking history, any evidence of eye, nose, or throat irritation; chronic airway problems or hyperreactive airway disease: allergic skin conditions or dermatitis; and upper or lower respiratory problems.

1910.1048(l)(3)(ii)

A determination by the physician, based on evaluation of the medical disease questionnaire, of whether a medical examination is necessary for employees not required to wear respirators to reduce exposure to formaldehyde.

1910.1048(l)(4)

Medical examinations. Medical examinations shall be given to any employee who the physician feels, based on information in the medical disease questionnaire, may be at increased risk from exposure to formaldehyde and at the time of initial assignment and at least annually thereafter to all employees required to wear a respirator to reduce exposure to formaldehyde. The medical examination shall include:

1910.1048(l)(4)(i)

A physical examination with emphasis on evidence of irritation or sensitization of the skin and respiratory system, shortness of breath, or irritation of the eyes.

1910.1048(l)(4)(ii)

Laboratory examinations for respirator wearers consisting of baseline and annual pulmonary function tests. As a minimum, these tests shall consist of forced vital capacity (FVC), forced expiratory volume in one second (FEV(1)), and forced expiratory flow (FEF).

1910.1048(l)(4)(iii)

Any other test which the examining physician deems necessary to complete the written opinion.

1910.1048(l)(4)(iv)

Counseling of employees having medical conditions that would be directly or indirectly aggravated by exposure to formaldehyde on the increased risk of impairment of their health.

1910.1048(l)(5)

Examinations for employees exposed in an emergency. The employer shall make medical examinations available as soon as possible to all employees who have been exposed to formaldehyde in an emergency.

1910.1048(l)(5)(i)

The examination shall include a medical and work history with emphasis on any evidence of upper or lower respiratory problems, allergic conditions, skin reaction or hypersensitivity, and any evidence of eye, nose, or throat irritation.

1910.1048(l)(5)(ii)

Other examinations shall consist of those elements considered appropriate by the examining physician.

1910.1048(l)(6)

Information provided to the physician. The employer shall provide the following information to the examining physician:

1910.1048(l)(6)(i)

A copy of this standard and Appendix A, C, D, and E;

1910.1048(l)(6)(ii)

A description of the affected employee's job duties as they relate to the employee's exposure to formaldehyde;

1910.1048(l)(6)(iii)

The representative exposure level for the employee's job assignment;

1910.1048(l)(6)(iv)

Information concerning any personal protective equipment and respiratory protection used or to be used by the employee; and

1910.1048(l)(6)(v)

Information from previous medical examinations of the affected employee within the control of the employer.

1910.1048(l)(6)(vi)

In the event of a nonroutine examination because of an emergency, the employer shall provide to the physician as soon as possible: a description of how the emergency occurred and the exposure the victim may have received.

1910.1048(l)(7)

Physician's written opinion.

1910.1048(l)(7)(i)

For each examination required under this standard, the employer shall obtain a written opinion from the examining physician. This written opinion shall contain the results of the medical examination except that it shall not reveal specific findings or diagnoses unrelated to occupational exposure to formaldehyde. The written opinion shall include:

1910.1048(l)(7)(i)(A)

The physician's opinion as to whether the employee has any medical condition that would place the employee at an increased risk of material impairment of health from exposure to formaldehyde;

1910.1048(l)(7)(i)(B)

Any recommended limitations on the employee's exposure or changes in the use of personal protective equipment, including respirators;

1910.1048(l)(7)(i)(C)

A statement that the employee has been informed by the physician of any medical conditions which would be aggravated by exposure to formaldehyde, whether these conditions may have resulted from past formaldehyde exposure or from exposure in an emergency, and whether there is a need for further examination or treatment.

1910.1048(l)(7)(ii)

The employer shall provide for retention of the results of the medical examination and tests conducted by the physician.

1910.1048(l)(7)(iii)

The employer shall provide a copy of the physician's written opinion to the affected employee within 15 days of its receipt.

1910.1048(l)(8)

Medical removal.

1910.1048(l)(8)(i)

The provisions of paragraph (l)(8) apply when an employee reports significant irritation of the mucosa of the eyes or of the upper airways, respiratory sensitization, dermal irritation, or dermal sensitization attributed to workplace formaldehyde exposure. Medical removal provisions do not apply in the case of dermal irritation or dermal sensitization when the product suspected of causing the dermal condition contains less than 0.05 percent formaldehyde.

1910.1048(l)(8)(ii)

An employee's report of signs or symptoms of possible overexposure to formaldehyde shall be evaluated by a physician selected by the employer pursuant to paragraph (l)(3). If the physician determines that a medical examination is not necessary under paragraph (l)(3)(ii), there shall be a two-week evaluation and remediation period to permit the employer to ascertain whether the signs or symptoms subside untreated or with the use of creams, gloves, first aid treatment or personal protective equipment. Industrial hygiene measures that limit the employee's exposure to formaldehyde may also be implemented during this period. The employee shall be referred immediately to a physician prior to expiration of the two-week period if the signs or symptoms worsen. Earnings, seniority and benefits may not be altered during the two-week period by virtue of the report.

1910.1048(l)(8)(iii)

If the signs or symptoms have not subsided or been remedied by the end of the two-week period, or earlier if signs or symptoms warrant, the employee shall be examined by a physician selected by the employer. The physician shall presume, absent contrary evidence, that observed dermal irritation or dermal sensitization are not attributable to formaldehyde when products to which the affected employee is exposed contain less than 0.1 percent formaldehyde.

1910.1048(l)(8)(iv)

Medical examinations shall be conducted in compliance with the requirements of paragraph (l)(5)(i) and (ii). Additional guidelines for conducting medical exams are contained in Appendix C.

1910.1048(l)(8)(v)

If the physician finds that significant irritation of the mucosa of the eyes or of the upper airways, respiratory sensitization, dermal irritation, or dermal sensitization result from workplace formaldehyde exposure and recommends restrictions or removal, the employer shall promptly comply with the restrictions or recommendation of removal. In the event of a recommendation of removal, the employer shall remove the affected employee from the current formaldehyde exposure and if possible, transfer the employee to work having no or significantly less exposure to formaldehyde.

1910.1048(l)(8)(vi)

When an employee is removed pursuant to paragraph (l)(8)(v), the employer shall transfer the employee to comparable work for which the employee is qualified or can be trained in a short period (up to 6 months), where the formaldehyde exposures are as low as possible, but not higher than the action level. The employer shall maintain the employee's current earnings, seniority, and other benefits. If there is no such work available, the employer shall maintain the employee's current earnings, seniority and other benefits until such work becomes available, until the employee is determined to be unable to return to workplace formaldehyde exposure, until the employee is determined to be able to return to the original job status, or for six months, whichever comes first.

1910.1048(l)(8)(vii)

The employer shall arrange for a follow-up medical examination to take place within six months after the employee is removed pursuant to this paragraph. This examination shall determine if the employee can return to the original job status, or if the removal is to be permanent. The physician shall make a decision within six months of the date the employee was removed as to whether the employee can be returned to the original job status, or if the removal is to be permanent.

1910.1048(l)(8)(viii)

An employer's obligation to provide earnings, seniority and other benefits to a removed employee may be reduced to the extent that the employee receives compensation for earnings lost during the period of removal either from a publicly or employer-funded compensation program or from employment with another employer made possible by virtue of the employee's removal.

1910.1048(l)(8)(ix)

In making determinations of the formaldehyde content of materials under this paragraph the employer may rely on objective data.

1910.1048(l)(9)

Multiple physician review.

1910.1048(l)(9)(i)

After the employer selects the initial physician who conducts any medical examination or consultation to determine whether medical removal or restriction is appropriate, the employee may designate a second physician to review any findings, determinations or recommendations of the initial physician and to conduct such examinations, consultations,and laboratory tests as the second physician deems necessary and appropriate to evaluate the effects of formaldehyde exposure and to facilitate this review.

1910.1048(l)(9)(ii)

The employer shall promptly notify an employee of the right to seek a second medical opinion after each occasion that an initial physician conducts a medical examination or consultation for the purpose of medical removal or restriction.

1910.1048(l)(9)(iii)

The employer may condition its participation in, and payment for, the multiple physician review mechanism upon the employee doing the following within fifteen (15) days after receipt of the notification of the right to seek a second medical opinion, or receipt of the initial physician's written opinion, whichever is later;

1910.1048(l)(9)(iii)(A)

The employee informs the employer of the intention to seek a second medical opinion, and

1910.1048(l)(9)(iii)(B)

The employee initiates steps to make an appointment with a second physician.

1910.1048(l)(9)(iv)

If the findings, determinations or recommendations of the second physician differ from those of the initial physician, then the employer and the employee shall assure that efforts are made for the two physicians to resolve the disagreement. If the two physicians are unable to quickly resolve their disagreement, then the employer and the employee through their respective physicians shall designate a third physician who shall be a specialist in the field at issue:

1910.1048(l)(9)(iv)(A)

To review the findings, determinations or recommendations of the prior physicians; and

1910.1048(l)(9)(iv)(B)

To conduct such examinations, consultations, laboratory tests and discussions with the prior physicians as the third physician deems necessary to resolve the disagreement of the prior physicians.

1910.1048(l)(9)(v)

In the alternative, the employer and the employee or authorized employee representative may jointly designate such third physician.

1910.1048(l)(9)(vi)

The employer shall act consistent with the findings, determinations and recommendations of the third physician, unless the employer and the employee reach an agreement which is otherwise consistent with the recommendations of at least one of the three physicians.

1910.1048(m)

Communication of hazards.

1910.1048(m)(1)

Hazard communication—General.

1910.1048(m)(1)(i)

Chemical manufacturers, importers, distributors and employers shall comply with all requirements of the Hazard Communication Standard (HCS) (§ 1910.1200) for formaldehyde.

1910.1048(m)(1)(ii)

In classifying the hazards of formaldehyde at least the following hazards are to be addressed: Cancer; skin and respiratory sensitization; eye, skin and respiratory tract irritation; acute toxicity effects; and flammability.

1910.1048(m)(1)(iii)

Employers shall include formaldehyde in the hazard communication program established to comply with the HCS (§ 1910.1200). Employers shall ensure that each employee has access to labels on containers of formaldehyde and to safety data sheets, and is trained in accordance with the requirements of HCS and paragraph (n) of this section.

1910.1048(m)(1)(iv)

Paragraphs (m)(1)(i), (m)(1)(ii), and (m)(1)(iii) of this section apply to chemicals associated with formaldehyde gas, all mixtures or solutions composed of greater than 0.1 percent formaldehyde, and materials capable of releasing formaldehyde into the air at concentrations reaching or exceeding 0.1 ppm.

1910.1048(m)(1)(v)

In making the determinations of anticipated levels of formaldehyde release, the employer may rely on objective data indicating the extent of potential formaldehyde release under reasonably foreseeable conditions of use.

1910.1048(m)(2)(i)

In addition to the requirements in paragraphs (m)(1) through (m)(1)(iv) of this section, for materials listed in paragraph (m)(1)(iv) capable of releasing formaldehyde at levels above 0.5 ppm, labels shall appropriately address all hazards as defined in paragraph (d) of § 1910.1200 and Appendices A and B to § 1910.1200, including cancer and respiratory sensitization, and shall contain the hazard statement "May Cause Cancer."

1910.1048(m)(2)(ii)

As a minimum, for all materials listed in paragraph (m)(1)(i) and (iv) of this section capable of releasing formaldehyde at levels of 0.1 ppm to 0.5 ppm, labels shall identify that the product contains formaldehyde; list the name and address of the responsible party; and state that physical and health hazard information is readily available from the employer and from safety data sheets.

1910.1048(m)(2)(iii)

Prior to June 1, 2015, employers may include the phrase "Potential Cancer Hazard" in lieu of "May Cause Cancer" as specified in paragraph (m)(2)(i) of this section.

1910.1048(m)(3)

Labels.

1910.1048(m)(3)(i)

The employer shall assure that hazard warning labels complying with the requirements of 29 CFR 1910.1200(f) are affixed to all containers of materials listed in paragraph (m)(1)(i), except to the extent that 29 CFR 1910.1200(f) is inconsistent with this paragraph.

1910.1048(m)(3)(ii)

Information on labels. As a minimum, for all materials listed in paragraph (m)(1)(i) capable of releasing formaldehyde at levels of 0.1 ppm to 0.5 ppm, labels shall identify that the product contains formaldehyde; list the name and address of the responsible party; and state that physical and health hazard information is readily available from the employer and from material safety data sheets.

1910.1048(m)(3)(iii)

For materials listed in paragraph (m)(1)(i) capable of releasing formaldehyde at levels above 0.5 ppm, labels shall appropriately address all hazards as defined in 29 CFR 1910.1200 (d) and 29 CFR 1910.1200 Appendices A and B, including respiratory sensitization, and shall contain the words "Potential Cancer Hazard."

1910.1048(m)(3)(iv)

In making the determinations of anticipated levels of formaldehyde release, the employer may rely on objective data indicating the extent of potential formaldehyde release under reasonably foreseeable conditions of use.

1910.1048(m)(3)(v)

Substitute warning labels. The employer may use warning labels required by other statutes, regulations, or ordinances which impart the same information as the warning statements required by this paragraph.

1910.1048(m)(4)

Material safety data sheets.

1910.1048(m)(4)(i)

Any employer who uses formaldehyde-containing materials listed in paragraph (m)(1)(i) shall comply with the requirements of 29 CFR 1910.1200(g) with regard to the development and updating of material safety data sheets.

1910.1048(m)(4)(ii)

Manufacturers, importers, and distributors of formaldehyde-containing materials listed in paragraph (m)(1)(i) shall assure that material safety data sheets and updated information are provided to all employers purchasing such materials at the time of the initial shipment and at the time of the first shipment after a material safety data sheet is updated.

1910.1048(m)(5)

Written hazard communication program. The employer shall develop, implement, and maintain at the workplace, a written hazard communication program for formaldehyde exposures in the workplace, which at a minimum describes how the requirements specified in this paragraph for labels and other forms of warning and material safety data sheets, and paragraph (n) for employee information and training, will be met. Employers in multi-employer workplaces shall comply with the requirements of 29 CFR 1910.1200(e)(2).

1910.1048(n)

Employee information and training -

1910.1048(n)(1)

Participation. The employer shall assure that all employees who are assigned to workplaces where there is exposure to formaldehyde participate in a training program, except that where the employer can show, using objective data, that employees are not exposed to formaldehyde at or above 0.1 ppm, the employer is not required to provide training.

1910.1048(n)(2)

Frequency. Employers shall provide such information and training to employees at the time of initial assignment, and whenever a new exposure to formaldehyde is introduced into the work area. The training shall be repeated at least annually.

1910.1048(n)(3)

Training program. The training program shall be conducted in a manner which the employee is able to understand and shall include:

1910.1048(n)(3)(i)

A discussion of the contents of this regulation and the contents of the Material Safety Data Sheet.

1910.1048(n)(3)(ii)

The purpose for and a description of the medical surveillance program required by this standard, including:

1910.1048(n)(3)(ii)(A)

A description of the potential health hazards associated with exposure to formaldehyde and a description of the signs and symptoms of exposure to formaldehyde.

1910.1048(n)(3)(ii)(B)

Instructions to immediately report to the employer the development of any adverse signs or symptoms that the employee suspects is attributable to formaldehyde exposure.

1910.1048(n)(3)(iii)

Description of operations in the work area where formaldehyde is present and an explanation of the safe work practices appropriate for limiting exposure to formaldehyde in each job;

1910.1048(n)(3)(iv)

The purpose for, proper use of, and limitations of personal protective clothing and equipment;

1910.1048(n)(3)(v)

Instructions for the handling of spills, emergencies, and clean-up procedures;

1910.1048(n)(3)(vi)

An explanation of the importance of engineering and work practice controls for employee protection and any necessary instruction in the use of these controls; and

1910.1048(n)(3)(vii)

A review of emergency procedures including the specific duties or assignments of each employee in the event of an emergency.

1910.1048(n)(4)

Access to training materials.

1910.1048(n)(4)(i)

The employer shall inform all affected employees of the location of written training materials and shall make these materials readily available, without cost, to the affected employees.

1910.1048(n)(4)(ii)

The employer shall provide, upon request, all training materials relating to the employee training program to the Assistant Secretary and the Director.

1910.1048(o)

Recordkeeping -

1910.1048(o)(1)

Exposure measurements. The employer shall establish and maintain an accurate record of all measurements taken to monitor employee exposure to formaldehyde. This record shall include:

1910.1048(o)(1)(i)

The date of measurement;

1910.1048(o)(1)(ii)

The operation being monitored;

1910.1048(o)(1)(iii)

The methods of sampling and analysis and evidence of their accuracy and precision;

1910.1048(o)(1)(iv)

The number, durations, time, and results of samples taken;

1910.1048(o)(1)(v)

The types of protective devices worn; and

1910.1048(o)(1)(vi)

The names, job classifications, social security numbers, and exposure estimates of the employees whose exposures are represented by the actual monitoring results.

1910.1048(o)(2)

Exposure determinations. Where the employer has determined that no monitoring is required under this standard, the employer shall maintain a record of the objective data relied upon to support the determination that no employee is exposed to formaldehyde at or above the action level.

1910.1048(o)(3)

Medical surveillance. The employer shall establish and maintain an accurate record for each employee subject to medical surveillance under this standard. This record shall include:

1910.1048(o)(3)(i)

The name and social security number of the employee;

1910.1048(o)(3)(ii)

The physician's written opinion;

1910.1048(o)(3)(iii)

A list of any employee health complaints that may be related to exposure to formaldehyde; and

1910.1048(o)(3)(iv)

A copy of the medical examination results, including medical disease questionnaires and results of any medical tests required by the standard or mandated by the examining physician.

1910.1048(o)(4)

Respirator fit testing.

1910.1048(o)(4)(i)

The employer shall establish and maintain accurate records for employees subject to negative pressure respirator fit testing required by this standard.

1910.1048(o)(4)(ii)

This record shall include:

1910.1048(o)(4)(ii)(A)

A copy of the protocol selected for respirator fit testing.

1910.1048(o)(4)(ii)(B)

A copy of the results of any fit testing performed.

1910.1048(o)(4)(ii)(C)

The size and manufacturer of the types of respirators available for selection.

1910.1048(o)(4)(ii)(D)

The date of the most recent fit testing, the name and social security number of each tested employee, and the respirator type and facepiece selected.

1910.1048(o)(5)

Record retention. The employer shall retain records required by this standard for at least the following periods:

1910.1048(o)(5)(i)

Exposure records and determinations shall be kept for at least 30 years.

1910.1048(o)(5)(ii)

Medical records shall be kept for the duration of employment plus 30 years.

1910.1048(o)(5)(iii)

Respirator fit testing records shall be kept until replaced by a more recent record.

1910.1048(o)(6)

Availability of records.

1910.1048(o)(6)(i)

Upon request, the employer shall make all records maintained as a requirement of this standard available for examination and copying to the Assistant Secretary and the Director.

1910.1048(o)(6)(ii)

The employer shall make employee exposure records, including estimates made from representative monitoring and available upon request for examination, and copying to the subject employee, or former employee, and employee representatives in accordance with 29 CFR 1910.1020 (a)-(e) and (g)-(i).

1910.1048(o)(6)(iii)

Employee medical records required by this standard shall be provided upon request for examination and copying, to the subject employee or former employee or to anyone having the specific written consent of the subject employee or former employee in accordance with 29 CFR 1910.1020 (a)-(e) and (g)-(i).

[56 FR 10378, Mar. 12, 1991; 56 FR 26909, June 12, 1991; 56 FR 37651, Aug. 8, 1991; 56 FR 57593, Nov. 13, 1991; 57 FR 2682, Jan. 23, 1992; 57 FR 19262, May 5, 1992; 57 FR 22290, May 27, 1992; 61 FR 5507, Feb. 13, 1996; 63 FR 1152, Jan. 8, 1998; 63 FR 20098, April 23, 1998; 70 FR 1143, Jan. 5, 2005; 71 FR 16672 and 16673, April 3, 2006; 71 FR 50190, August 24, 2006; 73 FR 75586, Dec. 12, 2008; 77 FR 17784, March 26, 2012]

### Attachment Q:  Formaldehyde OSHA Fact Sheet

Training Completion Certificate

By entering my signature below I confirm that I have read, understand, and accept the policy and procedure requirements detailed in the “Chemical Hygiene-Hazardous Communication” and have completed the “Chemical Hygiene-Hazardous Communication Training”

____________________________		_________________

SIGNATURE						DATE

_____________________________

Printed Name