# GAS Reagent and Process Control Preparation

PURPOSE

- This procedure provides instructions for preparation of reagents and procedural controls

#### MATERIALS REQUIRED

| Equipment | Reagents | Supplies |
| --- | --- | --- |
| Room 1: - Laminar-flow hood, Clean rm 1 - Freezer, -10 to -30⁰ C - Refrigerator, 2 to 8⁰ C - Microcentrifuge - Nalgene cooling block - Vortex - Eppendorf Repeater pipette - Dedicated set of pipettes in each working area: 2 µl, 10 µl, 20 l, 100 l, 200 l, and 1000 l pipettes - Pipet-Aid Room 2 - BSC, Process rm 2 - Refrigerator, 2 to 8⁰ C - Freezer, ≥ - 70⁰C - Nalgene cooling block - Vortex - Microcentrifuge Dedicated set of pipettes in each working area: 2 µl, 10 µl, 20 l, 100 l, 200 l, and 1000 l pipettes Micro - McFarland densitometer (micro) | TE buffer | Micro tube racks |
|  | Nuclease Free Water (NFW) | 2 ml cryovials |
|  | SEAC - Internal control pp - Internal control DNA | Sterile filtered pipette tips for 10 µl, 20 µl, 100 l, 200 µl, 1000 µl pipettes |
|  | GAS pp | Micro tubes 1.5 ml, RNase/DNase free |
|  | GAS positive control (POSC) | Nitrile gloves (powder-free) |
|  | GAS process control (PCTL) | Sharps disposal container |
|  | TA MasterMix | Gripper rack, rm 2 |
|  | Sani-Cloth Bleach wipes | Orange barrier wipes |
|  | 70% alcohol | BBL™CultureSwab™ |
|  | 5% Extran | 12X75 sterile plastic test tubes |
|  | Streptococcus pyogenes ATCC 19615 | Sterile Q – Tipped applicator swabs |
|  |  | 50 ml sterile conical tube |
|  |  | Eppendorf 5 ml tips |
|  |  | Serological pipettes, 5 and 10 ml |

ABBREVIATIONS

- BSC: biosafety cabinet

- Ct: crossing threshold

- F/T: freeze/thaw

- GAS: Group A Strep

- IC: internal control

- MM: master mix

- NEGC: negative control

- NFW: nuclease free water

- PP: primer – probe

- PPE: personal protective equipment

- TE buffer: Tris – EDTA buffer

## SAFETY CONSIDERATIONS

- Standard precautions

PROCEDURE A:	Follow the activities in the table below for Process Control preparation

Preparing Process Control Suspension

| Activity | Step | Action | Related Doc |
| --- | --- | --- | --- |
| Prepare | 1 | Pool approximately 10 ml of nasal wash matrix in a 50 ml sterile conical tube |  |
| Matrix | 2 | Dilute matrix in NFW to achieve a 30 – 35 ml suspension |  |
| Room 2 | 3 | Vortex well |  |
|  | 4 | Test suspension in duplicate according to the GAS assay procedure to ensure that it is target free | GAS 005 Group A Strep Assay |
| GAS 0.5 McFarland Micro | 5 | Prepare a 0.5 McFarland suspension of Streptococcus pyogenes ATCC 19615 Step; Action. a; Pick isolated colonies (18 – 24 h growth) with sterile CultureSwab. b; Suspend in saline; vortex. c; Adjust suspension to 0.5 McFarland (~1.5 X 108 CFU/mL) using densitometer. d; Dilution 1: make a 1:10 dilution of this suspension in NFW (~1.5 X 107). | 0.5 McFarland Standard turbidity range = 0.5 – 0.63 |
| Working suspension Room 2 | 6 | Dilution 2: dilute suspension 1:100 in prepared matrix (final concentration ~1.5 X 105 CFU/mL) Step; Action. 1; Pipette 300 µl from GAS 1:10 suspension into 30 ml of matrix. 2; Mix well by inversion/vortexing. 3; Absorb 100 µl unto 2 swabs; test prior to freezing suspension. 4; Target control range: Ct values 29-31. 5; I f necessary, adjust suspension to obtain specified range with NFW; retest. | 1 log = ~ 3 Ct |
| Aliquot | 7 | Label 2 ml cryovials with contents and date of preparation (approx. 29 tubes) |  |
|  | 8 | Dispense 1 ml of working suspension into tubes |  |
| Freeze | 9 | Freeze aliquots at –70° C |  |
| Decontaminate Hood | 10 | Wipe down BSC with 5% bleach followed by water and 70% alcohol |  |
| Room 2 | 11 | UV hood for 15 min |  |
| Test aliquots | 12 | Before use, thaw one PCTL aliquot, prepare swabs and test 5 times to determine average Ct value | Refer to GAS 004 Proc. B |
| Before use | 13 | Document Ct values on GAS PCTL New Reagent Worksheet MB005.2.F8 |  |
|  | 14 | Place worksheet and GAS Segment report including graphs in New Lot Inventory and QC manual |  |
| Stability | 15 | Once thawed, process control is stable for 7 days at refrigerated temperature |  |
|  | 16 | Do not refreeze (only 1 F/T cycle) |  |

PROCEDURE B:	Follow the activities in the table below for preparation of process control swabs

Preparing Process Control Swabs

| Activity | Step | Action | Related Doc |
| --- | --- | --- | --- |
| Thaw | 1 | Thaw GAS working suspension; vortex |  |
| Room 2 | 2 | Each working suspension is enough for approximately 10 control swabs |  |
| Swabs Prep | 3 | Label CultureSwab transport container with contents and date of preparation |  |
|  | 4 | Dip CultureSwab into suspension for 10 sec. to absorb approx. 100 µl (final concentration per swab ~ 1.5 x 105 CFU/mL) |  |
| Swab Prep cont. | 5 | Rotate swab against the wall of the tube above the liquid to remove excess inoculum. |  |
|  | 6 | Place swab into transport container allowing contact with medium at the bottom of the tube. |  |
|  | 7 | Wipe down workbench with Bleach Sani – Cloth followed by water and 70% alcohol after preparation. |  |
|  | 8 | Let swabs stand at room temp for 5 minutes before use. |  |
|  | 9 | Test the same as a clinical specimen. |  |
| Stability | 10 | Control swabs are stable for 5 days at 2 – 8⁰ C. |  |

PROCEDURE C:	Follow the activities in the table below for aliquoting TE buffer (sample buffer tubes) and Nuclease Free Water (NFW) used for NEGC and MM

Preparing TE buffer and NFW

| Activity | Step | Action | Related Doc |
| --- | --- | --- | --- |
| PPE | 1 | Wear lab coat and gloves dedicated to the Clean room 1 |  |
| TE buffer and NFW | 2 | Label 1.5 ml micro-centrifuge tubes with contents and date of preparation |  |
| Room 1 | 3 | Aliquot the following amounts into appropriately labeled tubes - 250 µl of TE buffer into each tube - 500 µl of NFW into each tube |  |
|  | 4 | Place in cryoboxes labeled with contents and date of preparation |  |
|  | 5 | Refrigerate aliquots in room 1 |  |
|  | 6 | Keep working supply in room 2 |  |

PROCEDURE D:	Follow the activity below for preparing master mix (MM)

Preparing RT-PCR Master Mix (MM)

| ###### Activity | ###### Step | Action |
| --- | --- | --- |
|  | 1 | MM must be used within 30 min of preparation. |
|  | 2 | Wear lab coat and gloves dedicated to the Clean room 1 |
| Warm reagents to RmTemp | 3 | Thaw Primer Probe mix, IC and the Master Mix at room temperature - Protect from light - Use within 1 hour |
| Room 1 | 4 | Gently mix each component Component; Mixing action. TA mm; Vortex 1 – 2 sec, setting 7 - 8. GAS pp; Gently flick or pipette 4 – 5 times. IC DNA; Vortex 1 – 2 sec, setting 7 - 8. IC pp; Gently flick or pipette 4 – 5 times. |
|  | 5 | Quick spin |
|  | 6 | Prepare MM in a 1.5 microcentrifuge tube by combining the reagents according to Table 1 |
|  | 7 | Gently vortex MM 1 – 3 sec to mix; vortex setting 7 Note: Adjust mixing time according to volume. |
|  | 8 | Quick spin |
| Refrigerate reagents | 9 | Do not refreeze reagents; store in refrigerator up to 30 days Note: Refer to procedure GAS 003 for storage conditions and expiry dates |
| Transport | 10 | Transport to room 2 |
| Room 2 | 11 | Keep the MM in refrigerator or cooling block protected from light until PCR reaction set-up. |

Table 1: GAS Master Mix Table

| No. of samples | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TA Master Mix (µl) | 6 | 10 | 14 | 18 | 24 | 28 | 32 | 36 | 40 | 44 | 48 | 52 |
| GAS Primer Mix (µl) | 0.6 | 1 | 1.4 | 1.8 | 2.4 | 2.8 | 3.2 | 3.6 | 4 | 4.4 | 4.8 | 5.2 |
| IC DNA (µl) | 0.6 | 1 | 1.4 | 1.8 | 2.4 | 2.8 | 3.2 | 3.6 | 4 | 4.4 | 4.8 | 5.2 |
| IC Primer Mix (µl) | 0.23 | 0.38 | 0.53 | 0.68 | 0.9 | 1.05 | 1.2 | 1.35 | 1.5 | 1.65 | 1.8 | 1.95 |
| NFW (µl) | 4.58 | 7.63 | 10.68 | 13.73 | 18.3 | 21.35 | 24.4 | 27.45 | 30.5 | 33.55 | 36.6 | 39.65 |
| Total volume (µl) | 12 | 20 | 28 | 36 | 48 | 56 | 64 | 72 | 80 | 88 | 96 | 104 |

| No. of samples | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TA Master Mix (µl) | 58 | 62 | 66 | 70 | 76 | 80 | 84 | 88 | 92 | 96 | 100 | 104 |
| GAS Primer Mix (µl) | 5.8 | 6.2 | 6.6 | 7.0 | 7.6 | 8 | 8.4 | 8.8 | 9.2 | 9.6 | 10 | 10.4 |
| IC DNA (µl) | 5.8 | 6.2 | 6.6 | 7.0 | 7.6 | 8 | 8.4 | 8.8 | 9.2 | 9.6 | 10 | 10.4 |
| IC Primer Mix (µl) | 2.18 | 2.33 | 2.48 | 2.63 | 2.85 | 3 | 3.15 | 3.3 | 3.45 | 3.6 | 3.75 | 3.9 |
| NFW (µl) | 44.23 | 47.28 | 50.33 | 53.38 | 57.95 | 61 | 64.05 | 67.1 | 70.15 | 73.2 | 76.25 | 79.3 |
| Total volume (µl) | 116 | 124 | 132 | 140 | 152 | 160 | 168 | 176 | 184 | 192 | 200 | 208 |

| No. of samples | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TA Master Mix (µl) | 110 | 114 | 118 | 122 | 128 | 132 | 136 | 140 | 144 | 148 | 152 | 156 |
| GAS Primer Mix (µl) | 11 | 11.4 | 11.8 | 12.2 | 12.8 | 13.2 | 13.6 | 14 | 14.4 | 14.8 | 15.2 | 15.6 |
| IC DNA (µl) | 11 | 11.4 | 11.8 | 12.2 | 12.8 | 13.2 | 13.6 | 14 | 14.4 | 14.8 | 15.2 | 15.6 |
| IC Primer Mix (µl) | 4.13 | 4.28 | 4.43 | 4.58 | 4.8 | 4.95 | 5.1 | 5.3 | 5.4 | 5.55 | 5.7 | 5.85 |
| NFW (µl) | 84 | 87 | 90 | 93 | 97.6 | 100.7 | 103.7 | 106.8 | 109.8 | 112.9 | 115.9 | 119 |
| Total volume (µl) | 220 | 228 | 236 | 244 | 256 | 264 | 272 | 280 | 288 | 296 | 304 | 312 |

PROCEDURE E:	Follow the activity below for preparing miscellaneous reagents

Preparing miscellaneous reagents

| ###### Reagent | ###### Step | Action |
| --- | --- | --- |
| 5% Extran Working solution | 1 | Prepare in amplification room. Caution: Protective eyewear must be worn when working with concentrated Extran |
| Room 3 | 2 | Make working solution as follows: Working Volume; Conc. Extran; Water. 2000 ml; 100 ml; 1900 ml. 3000 ml; 150 ml; 2850 ml. 4000 ml; 200 ml; 3800 ml. |
| 70% alcohol | 1 | Prepare from 100% Dehydrant alcohol located in the Flammable cabinet in the Recycling room. |
| Room 3 or Recycling room | 2 | Make working solution as follows: Working Volume; 100% Dehydrant; Water. 1000 ml; 700 ml; 300 ml. |

REFERENCES

- Simplexa™ Bordetella Universal Direct, PI.MOL2700.IVD EN CE Marked, REV F, July 18, 2012, Focus Diagnostics, Cypress, CA 90630

- Clinical Verification and Validation Study performed at Children’s Hospitals and Clinics of  MN August 2014

| Approval |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ### Approved by |  |  |  |  | Signature |  |  |  | Date |  |  |
|  | Tim Casey, MD |  |  |  |  | Tim Casey MD |  |  |  | 9.29.14 |  |  |
|  | Patricia Ackerman, Technical Specialist |  |  |  |  | P. Ackerman, TS |  |  |  | 9.5.14 |  |  |
| Annual Review |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ### Reviewed by |  | Signature |  |  | Date |  | ### Reviewed by | Signature |  | Date |  |
|  | P. Ackerman |  | PA |  |  | 9.5.14 |  |  |  |  |  |  |
| Historical Record |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Version | Written/Revised by: |  | Effective Date: |  |  | Summary of Revisions |  |  |  |  |  |
|  | 1 | P. Ackerman |  | 9.5.14 |  |  | Initial Version |  |  |  |  |  |
| Distribution |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Location |  |  |  | # Copies |  | Location |  |  |  |  | # Copies |
|  | Molecular Diagnostics rm B422 |  |  |  | 1 |  | G: Molecular Biology\Mol Proc Manual\MB005.3 RIPST\PF 004 |  |  |  |  | 1 |