| Splitting Blood Products using the Sterile Connecting Device SCD |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Purpose | This procedure provides instructions to provide partial units of red blood cells, thawed plasma or platelets for transfusion in a method that will decrease donor exposure, to avoid excessive wastage, and for patient identified being at risk of Transfusion-Associated Circulatory Overload (TACO). |  |  |  |  |  |
| Policy Statements | - Use of the Sterile Connecting Device (SCD) is not required in splitting products under the following conditions: - Expiration of the primary unit is < 24 hours (red cells, plasma), < 4 hours (platelets). - Volume that would remain in the primary unit after the unit is split will not allow for and equal volume for transfusion at a later date. - Tubing on primary unit will not conform to use with the SCD - The newly created split product shall be assigned the following expiration: - Red cells maintain the same expiration as the parent unit. - Thawed plasma maintains the same expiration as the parent unit. - Platelets shall be given a 4 hour expiration, or the expiration of the parent unit if <4 hours. - The transfusion request reflects the transfusion volume. Blood administration set volume equals 48 mL. |  |  |  |  |  |
| Related Documents | TSf 17.2.1 SCD Quality Control logsheet TS 17.2 Operating the TSCD TS 11.1 BPC of Products. TS 11.6 Labeling of Products Using Hematrax Labels |  |  |  |  |  |
| Materials | Equipment |  |  |  | Supplies |  |
|  | Terumo Sterile Connecting Device |  |  |  | - Transfer bags-300mL, 400mL or 600mL - Metal clips, sealer - Scissor - Plasma expressor |  |
| Procedure |  |  |  |  |  |  |
|  | Step | Action |  |  |  |  |
|  | 1 | Re-suspend the selected blood product by gently mixing. |  |  |  |  |
|  | 2 | Connect the tubing of the transfer bag to the tubing of the unit using the Sterile Connecting Device. |  |  |  |  |
|  | 3 | Slowly dispense the required the amount of blood component into the transfer bag. A plasma expresser or gravity may be used. |  |  |  |  |
|  | 4 | Release the pressure on the plasma expressor. |  |  |  |  |
|  | 5 | Allow the air in the transfer bag to go into the tubing to clear the tubing of product. |  |  |  |  |
|  | 6 | Seal the tubing using metal sealers and the handsealer, at approximately 2 inches from the transfer bag and 5 inches from the primary unit. |  |  |  |  |
|  | 7 | Cut the tubing between the clips to separate the original bag from the transfer bag. |  |  |  |  |
|  | 8 | Discard tubing as biohazard waste. |  |  |  |  |
|  | 9 | Record component preparation on TSf 17.2.1 SCD Quality Control logsheet. |  |  |  |  |
|  | 10 | Record creation of the syringe in Sunquest using D or DI prefix to product number. |  |  |  |  |
|  | 11 | Apply product over-labels. |  |  |  |  |
| References | Standards for Blood Banks and Transfusion Services, current edition. |  |  |  |  |  |
| Approval Workflow | Transfusion Service/Medical Director |  |  |  |  |  |
| Historical Record | Version |  | Written/Revised by: | Effective Date: |  | Summary of Revisions |
|  | 1 |  | K. Hartley | 9/1982 |  | Initial Version |
|  | 2 |  | J. Wenzel | 1/1991 |  |  |
|  | 3 |  | J. Wenzel | 3/1993 |  |  |
|  | 4 |  | J. Wenzel | 4/1995 |  |  |
|  | 5 |  | D. Hansen | 1/8/1997 |  | Merger |
|  | 6 |  | J. Bjorklund | 3/2000 |  |  |
|  | 7 |  | J. Wenzel | 9/4/2001 |  |  |
|  | 8 |  | J. Wenzel | 4/27/2004 |  |  |
|  | 9 |  | J. Wenzel | 5/02/2007 |  | Sunquest BCP Gui format |
|  | 10 |  | J. Wenzel | 8/6/2009 |  | New format. Separate procedures for labeling and BCP steps. |
|  | 11 |  | S. Cassidy | 04/10/2012 |  | Added Related Documents TS11.1 and TS 11.6 |
|  | 12 |  | S. Cassidy | 08/22/16 |  | Added statement about TACO |