| Entering Cardiac Tissue into Inventory and Storage using TrackCore |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Purpose | This procedure provides instructions for entering cardiac tissue into inventory and storage into the liquid nitrogen freezer in CVCC OR in Minneapolis using TrackCore. |  |  |  |  |  |
| Policy Statements | - At the time of receipt the package integrity and transport temperature range was controlled and acceptable for cardiac tissues are documented. |  |  |  |  |  |
| Materials | - Cryo laminating tape - Jumpsuit - Surgical Cap - Cryogenic gloves |  |  |  |  |  |
| Before You Begin | - Verify from the white board in Blood Bank or surgery that a cardiac tissue was ordered. |  |  |  |  |  |
| Procedure |  |  |  |  |  |  |
|  | Step |  | Action |  |  |  |
| Entering Tissue into TrackCore | 1 |  | Log onto Tissue Track Manager icon on the paperwork desk PC in Minneapolis. |  |  |  |
|  | 2 |  | Sign-in using your Children’s credentials (CE # and password) |  |  |  |
|  | 3 |  | Dashboard screen appears. |  |  |  |
|  | 4 |  | Click on Receive menu, select Vendor Items. |  |  |  |
|  | 5 |  | Select a Vendor for the drop-down. |  |  |  |
|  | 6 |  | Input Item Details If; Then. Manual Input; - Click Add Blank Row link. - Enter Serial number of Tissue. Scan Vendor Barcode; - Move Cursor to Quick Add Field.. |
|  | 7 |  | Click on the drop-down box to choose product type |  |  |  |
|  | 8 |  | Type in the expiration date MMDDYYYY or click on the drop-down box to choose date from calendar. |  |  |  |
|  | 9 |  | Click on “…” field to add more demographics. |  |  |  |
|  | 10 |  | Type in diameter and length of cardiac tissue into the dimension field. Note: Add a zero in front of any dimension that is a single digit. Example 09mm x 1.5cm |  |  |  |
|  | 11 |  | Click Save. |  |  |  |
|  | 12 |  | Click “Save and Print Labels” |  |  |  |
|  | 13 |  | Select the barcode Printer ZD420-203dpi and print two labels. |  |  |  |
|  | 14 |  | Place one label on tissue paperwork and save the other for the tissue box. |  |  |  |
|  | 15 |  | Print an Item Search Report to find a location to place the graft. |  |  |  |
|  | Step | Action |  |  |  |  |
| Placing Tissue into Liquid N2 Freezer using TrackCore **Note this is a 2 person job | 1 | Take tissue in shipping container and paperwork to surgery. |  |  |  |  |
|  | 2 | Put on jumpsuit and surgical cap prior to entering CVCC OR. |  |  |  |  |
|  | 3 | Log onto Tissue Track Manager icon on the computer in the freezer room |  |  |  |  |
|  | 4 | Click on the general Tab and select Search Cart. |  |  |  |  |
|  | 5 | Scan the TrackCore barcode or check item if already in cart. |  |  |  |  |
|  | 6 | Click Place in Storage. Warning: Only place one graft into storage at a time. |  |  |  |  |
|  | 7 | Click the drop-down box for storage location. |  |  |  |  |
|  | 8 | Check Verify All to document QA checks |  |  |  |  |
|  | 9 | Click Save. |  |  |  |  |
|  | 10 | Attach Trackcore barcode with thermal tape to tissue box. (Place label flat on the front of the box, toward the bottom making sure not to cover any important information.) Note: Tissue must be placed in freezer within 3 minutes after removal from shipper. |  |  |  |  |
|  | 11 | If; Then. Cryolife tissue; Note: CryoLife tissues must be placed in the liquid in vapors for a minimum of 2 hours then moved to liquid storage. - Place tissue in upper slots of sleeves (vapors). - Record temporary slot locations on a post-it note. - Wait at least two hours before moving tissue into permanent storage (liquid).. Lifenet tissue; Note: Lifenet tissues must never be placed in the liquid nitrogen. Store in the vapors only. - Place tissue in the slot selected.. |
|  | 12 | Place Tissue paperwork in mailbox outside the freezer room.. |  |  |  |  |
|  | Step | Action |  |  |  |  |
| Shipping back Cryoshippers containers | 1 | Empty shipping container. |  |  |  |  |
|  | 2 | Replace all packing material inside the box. |  |  |  |  |
|  | 3 | If; Then. Lifenet; - Secure the lid of the cryoshipper with a plastic tie and fit the cryoshipper securely within the shipping container if it has been removed. - Close the lid of the shipping container and tighten down the metal latches. Secure the latches with two plastic ties. - Attach the LifeNet Health return address label to the top of the shipping container being careful not obscure or cover any other labels. - Place cryoshipper in supermarket; place a label that says “empty, ready for return’.. |
|  | 4 | If; Then. Cryolife; - Tape box shut with packing tape. - Place FedEx shipping label on the top right corner of the box. - Place cryoshipper in supermarket; place a label that says “empty, ready for return”.. |
| Approval Workflow | Transfusion Service/Medical Director |  |  |  |  |  |
| Historical Record | Version |  |  | Written/Revised by: | Effective Date: | Summary of Revisions |
|  | 1 |  |  | S. Cassidy | 03/15/2017 | Initial Version |