| SECTION: GENERAL INFORMATION |  |
| --- | --- |
| DOCUMENT TYPE: Procedure |  |
| TITLE: Laboratory Daily Operational Tasks: Convenience Care locations |  |
| APPROVED BY: Laboratory Director, or Laboratory Supervisor | DOCUMENT #: 1.0157-a.7 |
| WRITTEN BY: Charlotte Yuza, MLT (ASCP) | EFFECTIVE: 11/8/12 |
| SCOPE: FastCare, and Skyway Clinics | COMPLEXITY: Waived |

Purpose:

To outline tasks pertaining to the daily operation of convenience care (waived level) laboratories.

Policy:

All operational tasks must be performed daily and documented.

Startup:

- Turn on Clinitek(s) if applicable:

- Allow the following to stabilize to room temperature (approximately 20 minutes):

- Quality Control

  - Do not allow controls to remain at room temperature for more than 40 minutes

  - Urine Quality Control Bio-rad Quantify (UA)

- Media Plates: enough for approximately one day’s use.

- Quality Control and maintenance:

- Perform and document daily urine quality control, and document temperatures.

- Troubleshoot and document out of range controls and instrumentation error codes.

- Refer to Recording and Reviewing Daily Quality Control Procedure for details.

Throughout the Day:

- Accurately perform and document patient specimen collection, testing, processing and resulting.

  - Review and check for errors by following the Manual Result verification process.

- Maintain patient confidentiality:

- Minimize screens, close sessions of IC chart, when not at computer terminals.

- Keep Patient information out of general view, discard labels, close or cover test worksheets

- Upon opening of any reagent, control, kit, etc.

- Document open date and initial on the bottles /kits, and QC sheets.

- Communications- Read correspondence and sign if necessary:

- Policies, procedures, memos, e-mails, and communication tools (calendar, inter-lab notes).

- Quality information

- Use and comply with safety policies and usage of safety devices, which includes but not limited to:

- Fluid resistance lab coats/jackets

- Gloves

- Other phlebotomy safety devices

- Lab supplies as necessary:

- Restock

- Order

Shutdown:

- Turn off Clinitek(s) as applicable.

- Review QC

- Ensure all QC has been documented for the day

- Document as appropriate any testing that wasn’t performed during the day on patients.

- Put away any:

- Unused media plates

- Manual patient testing worksheets, or any visible patient information

- Clean or disinfect all counters, the sink, and phone.

- Document any communication that needs to be communicated to the staff that works tomorrow.

CAP/COLA References:

PER 4  Laboratory Personnel fulfill responsibilities

ORG 7	Patient Confidentiality

| Approved By | Date | Review/Revised by | Date | Revision |
| --- | --- | --- | --- | --- |
| Michael Blue | 7/2007 | C Yuza | 7/2007 | New document |
| Michael Blue | 12/2007 | C Yuza | 12/2007 | Changed Clinitek verbiage |
| Michael Blue, Dale Loeffler | 4/08 | C Yuza | 4/08 | Added #2 Maintain patient confidentiality and COLA standard ORG 7 |
| Dr. Dale Loeffler | 9/3/09 | C Yuza | 8/27/09 | Updated format |
| Medical Director | 10/25/10 | C Yuza | 10/14/10 | Added University of Minnesota to scope and Title |
| Medical Director | 11/14/11 | C Yuza | 11/14/11 | Annual review |
| C Yuza | 5/18/12 | C Yuza | 5/9/12 | Revised section Throughout the day 1, 3,4 |
| C Yuza | 11/8/12 | C Yuza | 11/8/12 | Updated scope to include Skyway Clinic and level of complexity as waived. |

Similar Procedures (Specify title of procedure and other OMC location(s) used if applicable)

Department Responsibilities- Main Clinic1.0155

Laboratory Daily Operational Tasks1.0157

Attachments: