Clinical Laboratory

| SECTION: Specimen Collection and Processing |  |
| --- | --- |
| DOCUMENT TYPE: Procedure |  |
| TITLE: Skin Puncture-Heelsticks and Fingersticks |  |
| APPROVED BY: Medical Director | DOCUMENT #: 5.0805.6 |
| WRITTEN BY: Judy Vaughan, MT(ASCP) | EFFECTIVE: 11/20/2012 |
| SCOPE: All Laboratories | COMPLEXITY: NA |

Purpose:

To describe the procedure for specimen collection by dermal puncture (finger or heel).

Policy:

Staff with documented training and competency may perform this task.

Principle:

Capillary specimens contain a mixture of blood from capillaries, venules, arterioles, and tissue fluid. Skin punctures are especially important in pediatrics because small amounts of blood can be obtained with this technique. The technique used to perform the skin puncture and obtain the specimen is crucial to maintaining its integrity.

Reagents and Materials:

1.	Micro-collection tubes

2.	70% alcohol prep pads

3.	Gauze and Band-aids (if needed)

4.	Heel warmer (if needed)

5.	Gloves

6.	Sharps container

7. 	Automated skin puncture device: select appropriate device for the type of specimen and patient.

| DEVICE | USE | DEPTH | WIDTH |
| --- | --- | --- | --- |
| BD Quikheel – Preemie | Low birth weight infants | 0.85 mm | 1.75 mm |
| BD Quikheel | Infants less than 1 year of age (heel only) | 1.0 mm | 2.5 mm |
| BD Microtainer Contact Activated Lancet-Purple | Finger stick (low flow) Whole blood glucose | 1.5 mm | 30g |
| ICT Tenderlett- Red | Finger stick | 1.75 mm | 0.94 mm |
| Surgilance by Medipurpose-Green | Finger stick (child) | 1.8 mm | 19 g |
| Surgilance by Medipurpose- Blue | Finger stick (adult) | 2.3 mm | 18 g |

Procedural Notes:

1.	HEELSTICK:

- The heel is the preferred site for dermal punctures in infants less than one year old or under 20 pounds. The heel contains more tissue than the fingers and has not yet become callused from walking.

- Acceptable sites for heel punctures are the medial and lateral areas of the plantar (bottom) surface of the heel:

- DO NOT puncture other areas of the foot where the heel bone is closest to the surface or the arch where nerves and tendons may be injured.

- Lancets should not penetrate more than 2.0 mm to prevent accidental puncture of the calcaneus which could increase the risk of bone infection.

- Recommendation is 0.65-0.85mm heel stick depth for premature neonates and <=1.0 mm depth for larger infants.

2.	FINGERSTICK:

- Finger punctures are performed on adults and children over one year of age or over 20 pounds.

- The puncture must be on the palmar surface of the finger. Fleshy areas of the third and fourth fingers are the sites of choice.

- Incision should be perpendicular to the finger whorls.

- DO NOT use the tip or sides of the finger. This increases the possibility of bone injury.

- Recommendation is <=1.8 mm finger stick depth for toddlers/children.

3.	The difference in the concentration of several analytes in capillary specimens versus venipuncture is clinically and/or statistically significant:

- Glucose (higher)

- Potassium (lower)

- Total protein (lower)

- Calcium (lower)

4. 	Recommended Depth for Capillary Draws:

|  | Heel Stick | Finger Stick |
| --- | --- | --- |
| Premature Neonate | 0.65-0.85 mm | NA |
| Neonate | <=1.0 mm | NA |
| Child / Toddler | NA | <=1.8 mm |
| Adult | NA | <=2.3 mm |

Procedure:

1.	Introduce yourself to the patient.

2.	Identify patient using two unique identifiers.

3.	Verify patient information on bar-coded labels, requisition forms, and/or patient ID band.

4.	Determine/verify any diet restrictions or last dose times for therapeutic drugs.

5.	Determine the appropriate micro-collection tubes, any special specimen requirements, and assemble supplies.

6.	Select appropriate site (see procedural notes).

7.	Apply warmer for 3-5 minutes prior to specimen collection

8.	Put on gloves.

9.	Cleanse the puncture site and allow to air dry. Do not blow on or fan the site to speed drying.

10.	Remove selected capillary puncture device from packaging.

11.	Puncture the site by activating device.

a.	Heel

- Grasp the infant’s heel with your forefinger at the arch of the foot and your thumb at the ankle.

- Align puncture device so lancet will cut across the grooves of the footprint.

- Press appropriate device firmly against the puncture site and activate.

b.	Finger

- Grasp finger between thumb and index finger with palmar surface facing up.

- Align puncture device so the lancet will cut across the grooves of the fingerprint.

- Press appropriate device firmly against the puncture site and activate.

12.	Remove the device from the surface of the skin and discard in sharps container.

13.	Wipe the first drop of blood away with a gauze pad. The first drop of blood is most likely to contain excess tissue fluid.

14.	Hold the tip of the collection tube to the drop of blood, letting the blood flow into the tube by capillary action. Fill micro-collection tubes following the order of draw:

Capillary Specimen Order of Draw

| TOP COLOR | ADDITIVE-SPECIMEN | COMMENTS |
| --- | --- | --- |
| Lavender | EDTA-whole blood/plasma | CBC, hgb |
| Slides | None | Used for CBC and to verify platelet counts |
| Green with gel | Lithium heparin-plasma | Most chemistry tests can be done on serum or plasma-plasma used because of better specimen yield after centrifugation |
| Yellow with gel | No anticoagulant Serum separator tube-serum |  |

a.	Micro-collection tubes

- Blood must be free-flowing as a result of firm pressure. Do not “milk” surrounding tissue, which will release tissue fluid. Do not tightly squeeze area without relaxation, cutting off blood flow to the puncture site.

- Apply and release pressure to area to obtain satisfactory blood flow.

- Hold tube at a slant during the collection. Do not touch puncture site with microtainer and do not scrape microtainer over skin, as this will cause specimen contamination and hemolysis.

- Allow blood to flow by capillary action through collection top and down side of tube.

- Gently tap the bottom of tube to force blood to bottom.

- When tube is filled, remove collection top and attach color-coded top.

- Tubes with anticoagulants must be gently inverted 8-10 times.

15.	After blood has been collected, apply gauze and pressure to site. Apply bandage as needed.

16.	Label tubes in the presence of the patient. Record time of collection and initials of collector on label and requisition form.

17. Handle specimen according to protocol and deliver to specimen processing or testing area.

Limitations:

1.	When multiple specimens are needed from a dermal puncture, it is essential that tubes be filled in the correct order due to the tendency of platelets to accumulate at the site of the wound.

2.	Specimen for tests that cannot be collected by dermal puncture:

- Coagulation (non-point of care)

- Tests requiring anaerobic conditions: ammonia & lactic acid.

- Blood cultures

- Blood Gases

3.	Bandaging neonates and infants less than 2 years of age is not recommended for two reasons:

- Young skin is more sensitive to adhesives.

- Pose a risk of ingestion or airway obstruction if swallowed.

References:

NCCLS “Procedures for the Collection of Diagnostic Blood Specimens by Skin Puncture; Approved Standard-Fifth Edition, June 2004.

Phlebotomy Workbook for the Multi-Skilled Healthcare Professional, Susan King Strasinger & Marjorie A Di Lorenzo, 1996.

Applied Phlebotomy, Dennis Ernst, 2005.

Phlebotomy Handbook, 8th Edition, Diana Garza, 2010

CAP Standards:

GEN.40100		Instructions for specimen collection

COLA Standards:

PRE 12             Specimen Handling

PRE 16             Patient Identification

PRE 17             Specimen Identification- two identifiers

| # Approved By | # Date | # Reviewed/Revised By | # Date | # Revision |
| --- | --- | --- | --- | --- |
| Dr Bradley Karon | 8/2006 | J Vaughan | 8/2006 | New format; More comprehensive information |
| Dr Bradley Karon | 8/2007 | J Vaughan | 8/2007 | Added caveat for site selection; Age limitation for bandage use |
| Dr Durga Vege | 11/2007 | J Vaughan | 11/2007 | Added BD Genie Lancet-toddler |
| Dr Durga Vege | 11/2008 | J Vaughan | 11/2008 | Annual review |
| Dr Durga Vege | 2/2009 | C Yuza | 2/2009 | Updated COLA standards |
| Dr Durga Vege | 2/15/2010 | J Vaughan | 2/2/2010 | Updated format; deleted Genie Lancets (discontinued); added BD Microtainer Contact Activated lancets; edited microtainer mix times |
| Dr Durga Vege | 2/16/2011 | J Vaughan | 2/1/2011 | Updated lancets & procedural notes |
| Dr Durga Vege | 2/13/2012 | J Vaughan | 2/1/2012 | Annual review |
|  |  | J Vaughan | 11/20/2012 | Updated #9; Updated Reagents & Materials #7 and Procedure Notes 1 & 2. Added Procedure Note #4 |

Similar Procedures (Specify title of procedure and other OMC location(s) used if applicable)

Attachments: