Purpose:  To describe the steps that are involve in adding a new operator to the Cobas IT system

Procedure:

1- Go to Organization > User management screen

2- Click on Add button.

3- Fill out the information required. The fields flagged with a blue dot are mandatory. Put “WELCOME1” as the password.  Remember to assign the profile if Cobas IT access is required!

4- Click on Additional Info tab, and add information if desired.

5- Click on Assign button in other to define the location(s) where the new operator will be affiliated. If operator is not affiliated to any location, he/she won’t be able to use any instrument. The locations on the right hand side indicate the affiliations of the operator.

6- Once the operator is created, highlight him/her, and click on Certificates> Add Certif.

Select the certification and phase (pending, interim or certified) desired and click on Save.

7- Press the double back arrow, to confirm that the right certification was added to the new operator.

8- The operator should now be on the operator’s list. Test one instrument, making sure it has communicated with the system after adding the new user.

References:  Cobas IT 1000 ver. 2.0