Purpose

The purpose of this document is to provide a procedure to demonstrate how to embed skin biopsies for Alopecia diagnosis.   It is important that the proper orientation be maintained in order for the pathologist to make an accurate diagnosis.

Principle

Embedding, also referred to as casting or blocking, involves enclosing the tissue in the infiltration medium used for processing and then allowing the medium to solidify, providing a hardened block of tissue for microtomy.  When a skin is submitted for Alopecia diagnosis it is generally dissected into 3 portions, each needing specific orientation.  The PA inks the surface to be embedded “down” with a red ink in order to assist the technician/technologist with orientation during the embedding process.  When embedded properly the pathologist is able to microscopically view 3 different angles of the biopsy allowing them to make an accurate diagnosis.

Equipment	The following equipment is needed for this procedure:

- Heated forceps

- Embedding Center

Safety  		 Follow standard safety procedures when embedding tissue.

Supplies and	 The following supplies and reagents are needed for this procedure:

Reagents

PARAFFIN

Quality control

- It is very important that light pressure be placed over the entire specimen during the orientation of the specimen and beginning stages of solidification so that the tissue will be embedded flat; otherwise a complete section cannot be obtained.  Sections must be held down with heated forceps longer than traditional tissue to keep the section flat.

Printed copies of this document are not considered up-to-date. Please verify current version date with online document.

- The side of the tissue that is face down in the cassette is the side to be embedded down.

Procedure	Follow the steps in the table(s) below:

| Step | Action | Note |
| --- | --- | --- |
| 1 | Remove cassetted tissue from tissue processor. |  |
| 2 | Place cassetted tissue in embedding center paraffin wells. Remove one cassette at a time for embedding. | Never open more that one cassette at a time. |
| 3 | Scan 2-D barcode to EPIC Beaker tracking, reason Embedded. | Check for any embedding instructions before embedding tissue. Document any quality issues in case info |
| 4 | Carefully open the cassette. |  |
| 5 | There should be 3 cassettes, each contains one of the 3 pieces in the diagram below. Embed each with the red surface down. The blue lines= The red lines= cut surfaces. 1 2 surface to be embedded down in cassette. 3 | If they are not marked on one surface with red ink, find the PA who dictated the case for their assistance with orientation. |
| 6 | Fill mold approximately half full with molten paraffin. |  |
| 7 | Using warm forceps, pick up tissue and place it in the bottom of the mold placing the red surface face down in the mold. |  |
| 8 | Place mold on cold plate and hold the tissue down with the warm forceps until the mold begins to solidify. |  |
| 9 | After tissue is secure, place the cassette on top of the mold and fill to the top with molten paraffin. |  |
| 10 | The blocks are cooled by placing them on the cryoplate. |  |
| 11 | After the paraffin is hard, the blocks can be removed from the molds. |  |

Results

- Tissue is embedded with proper orientation allowing appropriate sections of tissue to be cut for Alopecia diagnosis.

Authorized Reviewers

Medical Director, Anatomic Pathology

Chief, Surgical Pathology

Printed copies of this document are not considered up-to-date. Please verify current version date with online document.

##### Document Control

##### Location of Master: Master electronic file stored on the Beaumont Laboratory server under

##### S:\Anatomic Pathology\Royal Oak\Histology\Document Control\Master Documents\Histology Procedures\Microtomy

Number of Controlled Copies posted for educational purposes:  0

Number of circulating Controlled Copies:  1, Located in the Histology Procedure manual bench copy

##### Document History

| Signature | Date | Revision # |  | Related Documents Reviewed/ Updated |
| --- | --- | --- | --- | --- |
| Prepared by: S. Scalise | 10/28/13 | 00 |  |  |
| Approved by: A. Armin, MD | 10/28/13 | 00 |  |  |
| Reviewed by: (Signature) | Date | Revision # | Modification | Related Documents Reviewed/ Updated |
| M. Amin, MD | 2/14/15 | 00 |  |  |
| Dr. Qu |  | 01 | Updated to include Vantage bar code scanning step. Changed step 5 to “3 cassettes” as all 3 pieces are each in separate cassette. Removed step about placing blocks in fridge to harden. Removed scraping step. |  |
| Dr. Bernacki | 10/31/2017 | 01 |  |  |
| Dr. Bernacki | 10/22/2019 | 01 |  |  |
|  |  | 02 | Added Epic Beaker tracking Added note about quality documentation |  |

Printed copies of this document are not considered up-to-date. Please verify current version date with online document.