Purpose	To provide a procedure for the gross examination of an endocervical biopsy specimen.

Principle	To process all material submitted so that a diagnosis can be made microscopically by a Pathologist.

_____________________________________________________

Equipment	The following equipment is needed for this procedure:

- Ruler

- Forceps

- Small biopsy bags

Safety

- PPE should be worn.

- FORMALIN is a known carcinogen.

Supplies and	The following supplies and reagents are needed for this

Reagents	procedure:

- 10% NEUTRAL BUFFERED FORMALIN (pH range 6.9 – 7.2) - BBC Biochemical, Item # 0151 (5 Gallon Cube)

Quality control

- All material should be submitted.

Limitations/		The following may influence the validity of test results:

Notes

- The specimen should be fixed in formalin.

_________________

Procedure	Follow the steps listed below:

- The specimen will arrive in a jar of formalin.

- The specimen willl either be suspended in the formalin or adherent to a telfa pad within the formalin.  If the specimen is adherent to a telfa pad, gently scrape the material on the pad into the formalin with the forceps.

- Strain the specimen into one or more filter bags.  Each filter bag should be submitted in a separate cassette.

- Measure the specimen as an aggregate in 3 dimensions (cm.).

- Describe the components that are present within the aggregate (tissue fragments, mucous, and/or hemorrhagic material).

- The entire specimen should be submitted.

- If one block is submitted, step sections (X3) should be dictated under the special procedures section of the pathology report.

- If more than one block is submitted, it is not necessary to order step sections.

- The cassette or cassettes should be loaded on the appropriate processor.

References		The following items were used as sources of information.

Hruban RH, Westra, WH, Phelps, PH, & Isacson, C:  Surgical Pathology Dissection An Illustrated Guide, New York, NY, Springer-Verlag Inc., 1996.

Lester, SC:  Manual of Surgical Pathology, New York, NY, Churchill

Livingstone, 2001.

##### Document Control

##### Location of Master: Master electronic file stored on the Beaumont Laboratory server under S:\ AP_Grossing_Manual

Number of Controlled Copies posted for educational purposes:  0

Number of circulating Controlled Copies:  1

##### Location of circulating Controlled Copies: Master Grossing Manual located in Surgical Pathology

_________

##### Document 		The table below is used to capture staff signatures and dates

##### history 	associated with initial document preparation, annual review, and revisions.

| Signature | Date | Revision # |  | Related Documents Reviewed/ Updated |
| --- | --- | --- | --- | --- |
| Prepared by: Anne Tranchida PA(ASCP) | 8/2/2007 | r00 |  |  |
| Approved by: Ali-Reza Armin, MD | 12/5/2007 | r00 |  |  |
| Reviewed by: (Signature) | Date | Revision # | Modification | Related Documents Reviewed/ Updated |
| Ali-Reza Armin,MD | 12/10/2008 | r00 |  |  |
| Anne R. Tranchida,PA(ASCP) | 10/20/2009 | r00 |  |  |
| Ali-Reza Armin,MD | 10/20/2011 | r00 |  |  |
| Ali-Reza Armin,MD | 4/2/2013 | r00 |  |  |
| Mitual B. Amin,MD | 2/14/2015 | r00 |  |  |
| Zhenhong H. Qu, MD | 3/11/2015 | r00 |  |  |
| Kurt Bernacki, MD | 10/27/2017 | r00 |  |  |
| Kurt Bernacki, MD | 10/22/2019 | r00 |  |  |
| Kurt Bernacki, MD | 10/20/2021 | r00 |  |  |
| Jawwad Arshad, MD | 3/20/2023 | r00 |  |  |