Purpose	To provide a procedure for the gross examination of an adrenal gland.

Principle	To take histologic sections to demonstrate the pathologic process so that a diagnosis can be made microscopically by a Pathologist.

_____________________________________________________

Equipment	The following equipment is needed for this procedure:

- Ruler

- Forceps

- Scalpel

- Scissors

- Large Knife

Safety

- PPE should be worn.

- 10% FORMALIN is a known carcinogen.

Supplies and	The following supplies and reagents are needed for this

Reagents	procedure:

- 10% NEUTRAL BUFFERED FORMALIN (pH range 6.9 – 7.2) - BBC Biochemical, Item # 0151 (5 Gallon Cube)

- BLACK INK – Cancer Diagnostics, Inc. Tissue Marking Dye System, Black #0728-1

- WHITE DISTILLED VINEGAR – Reliance Sysco

Quality control

- All remaining tissue should be retained.

Limitations/		The following may influence the validity of test results:

Notes

- The specimen should be fixed in formalin.

- If any special studies are going to be performed, tissue must be collected while the adrenal gland is fresh.

Procedure	Follow the steps listed below:

- Weigh and measure (3 dimensions – cm.) the specimen.

- If possible, orient the specimen.  The concave aspect is inferolateral.  The adrenal consists of a head, body, and tail.  The head is the most medial, the body is in the center, and the tail is the most lateral.  The adrenal vein may be identified between the head and the body.

- Describe the outer surface.  Note whether or not a capsule is present.  If a capsule is present, determine if it is intact.

- Ink the external surface of the specimen.

- Use vinegar as a mordant for the ink.

- Serially section the adrenal gland perpendicular to its long axis.

- Examine the cut surfaces.

- Locate the pathologic process that is present and describe the size, shape, color, consistency, location (cortex or medulla), region (head, body, or tail), and the relationship to the external surface of the adrenal gland and capsule (if present).

- Describe the remaining cut surfaces of the specimen that are not involved by the pathologic process.

- If fresh tissue is required for special studies, it should be collected.

- Measure the thickness of the cortex and the medulla in all three regions of the gland (if possible).

- If the lesion is smaller than 2 cm, submit the lesion entirely.

- If the lesion is larger than 2 cm, submit three sections of the lesion plus one section per centimeter of it.

- Submit 1-2 sections of the black-inked external surface that is closest to the pathologic process.

- Submit representative sections of the grossly uninvolved adrenal gland.  If possible, include all regions.

- If no areas of abnormality are identified, grossly, generously sample each region of the adrenal gland.

- If identified, submit a representative section of the renal vein.

- Any identifiable lymph nodes should be submitted.

References		The following items were used as sources of information.

Hruban RH, Westra, WH, Phelps, PH, & Isacson, C:  Surgical Pathology Dissection An Illustrated Guide, New York, NY, Springer-Verlag Inc., 1996.

Lester, SC:  Manual of Surgical Pathology, New York, NY, Churchill

Livingstone, 2001.

##### Document Control

##### Location of Master: Master electronic file stored on the Beaumont Laboratory server under S:\ AP_Grossing_Manual

Number of Controlled Copies posted for educational purposes:  0

Number of circulating Controlled Copies:  1

##### Location of circulating Controlled Copies: Master Grossing Manual located in Surgical Pathology

_________

##### Document 		The table below is used to capture staff signatures and dates

##### history 	associated with initial document preparation, annual review, and revisions.

| Signature | Date | Revision # |  | Related Documents Reviewed/ Updated |
| --- | --- | --- | --- | --- |
| Prepared by: Anne Tranchida PA(ASCP) | 9/13/2007 | r00 |  |  |
| Approved by: Ali-Reza Armin, MD | 10/15/2007 | r00 |  |  |
| Reviewed by: (Signature) | Date | Revision # | Modification | Related Documents Reviewed/ Updated |
| Ali-Reza Armin,MD | 12/10/2008 | r00 |  |  |
| Anne R. Tranchida,PA(ASCP) | 10/20/2009 | r00 |  |  |
| Ali-Reza Armin,MD | 10/20/2011 | r00 |  |  |
| Ali-Reza Armin,MD | 4/3/2013 | r00 |  |  |
| Mitual B. Amin,MD | 2/14/2015 | r00 |  |  |
| Zhenhong H. Qu, MD | 3/14/2015 | r00 |  |  |
| Kurt Bernacki, MD | 10/27/2017 | r00 |  |  |
| Kurt Bernacki, MD | 10/22/2019 | r00 |  |  |
| Kurt Bernacki, MD | 10/20/2021 | r00 |  |  |