Purpose	To provide a procedure for the gross examination of a ciliary body biopsy.

Principle	To submit the specimen for routine histology so that a diagnosis can be made microscopically by a pathologist.

_____________________________________________________

Equipment	The following equipment is needed for this procedure:

- Ruler

- Forceps

- Scalpel

Safety

- PPE should be worn.

- FORMALIN is a known carcinogen.

Supplies and	The following supplies and reagents are needed for this

Reagents	procedure:

- 10% NEUTRAL BUFFERED FORMALIN (pH range 6.9 – 7.2) - BBC Biochemical, Item # 0151 (5 Gallon Cube)

- HEMATOXYLIN I, Richard-Allen Scientific, Reorder Number 7221.

Quality control

- All tissue should be submitted.

Limitations/		The following may influence the validity of test results:

Notes

- The specimen should be fixed in formalin.

_________________

Procedure	Follow the steps listed below:

- The specimen will arrive in a jar of formalin, fresh, or in opthamology fixative.

- If received in ocular fluid, determine if cytology is indicated for the accompanying fluid.

- If cytology is needed, the fluid should be separated from the tissue specimen for routine histology.  The fluid should then be sent to cytology for further processing.

- Measure the tissue specimen in 3 dimensions (cm. or mm.).

- Submit the specimen entirely for routine histology.

- All tissue should be inked with hematoxylin.

- If the specimen is less than 5 mm., “serial sections X6” should be dictated under the special procedures section of the pathology report.  Sections 1 & 6 should be stained with H&E while sections 2-5 should be left unstained on plus slides for possible immuno studies.

- If the specimen is larger than 5mm, one H&E stained slide should be cut as a “scout” slide.

- If there are questions related to the processing, please consult the pathologist assigned the case.

- The cassette should be loaded on the next available processor.

References		The following items were used as sources of information.

Hruban RH, Westra, WH, Phelps, PH, & Isacson, C:  Surgical Pathology Dissection An Illustrated Guide, New York, NY, Springer-Verlag Inc., 1996.

Lester, SC:  Manual of Surgical Pathology, New York, NY, Churchill

Livingstone, 2001.

##### Document Control

##### Location of Master: Master electronic file stored on the Beaumont Laboratory server under S:\ AP_Grossing_Manual

Number of Controlled Copies posted for educational purposes:  0

Number of circulating Controlled Copies:  1

##### Location of circulating Controlled Copies: Master Grossing Manual located in Surgical Pathology

_________

##### Document 		The table below is used to capture staff signatures and dates

##### history 	associated with initial document preparation, annual review, and revisions.

| Signature | Date | Revision # |  | Related Documents Reviewed/ Updated |
| --- | --- | --- | --- | --- |
| Prepared by: Anne Tranchida PA(ASCP) | 1/29/2009 | r00 |  |  |
| Approved by: Ali-Reza Armin, MD | 10/24/2009 | r00 |  |  |
| Reviewed by: (Signature) | Date | Revision # | Modification | Related Documents Reviewed/ Updated |
| Ali-Reza Armin,MD | 10/20/2011 | r00 |  |  |
| Ali-Reza Armin,MD | 4/3/2013 | r00 |  |  |
| Mitual B. Amin,MD | 2/14/2015 | r00 |  |  |
| Zhenhong H. Qu, MD | 3/14/2015 | r00 |  |  |
| Kurt Bernacki, MD | 10/27/2017 | r00 |  |  |
| Kurt Bernacki, MD | 10/22/2019 | r00 |  |  |
| Kurt Bernacki, MD | 10/20/2021 | r00 |  |  |