Purpose	To provide a procedure for the dissection of a colon segment or total colectomy for a non-neoplastic condition.

Principle	To take histologic sections to demonstrate the pathologic process so that a diagnosis can be made microscopically by a Pathologist.

_____________________________________________________

Equipment	The following equipment is needed for this procedure:

- Ruler

- Forceps

- Scalpel

- Scissors

- Large Knife

Safety

- PPE should be worn.

- 10% FORMALIN is a known carcinogen.

Supplies and	The following supplies and reagents are needed for this

Reagents	procedure:

- 10% NEUTRAL BUFFERED FORMALIN (pH range 6.9 – 7.2) - BBC Biochemical, Item # 0151 (5 Gallon Cube)

Quality control

- All remaining tissue should be retained.

Limitations/		The following may influence the validity of test results:

Notes

- The specimen should be fixed in formalin.

Procedure	Follow the steps listed below:

- If possible, orient the specimen.

- Identify all components of the specimen and measure each.  Include a length and a diameter or an internal circumference.

- Describe the specimen using a systematic approach (ex: outside to inside).  Carefully inspect the serosa and the mucosa.

- Describe the pathologic process or abnormality (size, shape, color, & consistency) and its relationship to the margins.

- If an inflammatory condition is suspected or identified, a full thickness section should be taken every 10 cm.

- If diverticulosis is present, a few diverticula should be sampled and the specimen should be inspected for abscess cavities that may be present within the pericolonic adipose tissue or possible rupture sites that may be present within the diverticula.

- If a rupture site is present, the rupture site should be sampled.

- Measure the thickness of the uninvolved wall of the specimen.

- Other attached structures should be sampled (ex: appendix or omentum).

- Submit shave sections of all margins.  If the pathologic process approaches the margin, take perpendicular sections at that margin which may also include the pathologic process.

- Sample the pathologic process.

- Submit full thickness representative sections of the uninvolved wall.

- Sample any other areas of abnormality and/or any other attached structures.

- If the pathologic process warrants a lymph node dissection, all identifiable lymph nodes should be submitted.

References		The following items were used as sources of information.

Hruban RH, Westra, WH, Phelps, PH, & Isacson, C:  Surgical Pathology Dissection An Illustrated Guide, New York, NY, Springer-Verlag Inc., 1996.

Lester, SC:  Manual of Surgical Pathology, New York, NY, Churchill

Livingstone, 2001.

##### Document Control

##### Location of Master: Master electronic file stored on the Beaumont Laboratory server under S:\ AP_Grossing_Manual

Number of Controlled Copies posted for educational purposes:  0

Number of circulating Controlled Copies:  1

##### Location of circulating Controlled Copies: Master Grossing Manual located in Surgical Pathology

_________

##### Document 		The table below is used to capture staff signatures and dates

##### history 	associated with initial document preparation, annual review, and revisions.

| Signature | Date | Revision # |  | Related Documents Reviewed/ Updated |
| --- | --- | --- | --- | --- |
| Prepared by: Anne Tranchida PA(ASCP) | 7/31/2007 | r00 |  |  |
| Approved by: Ali-Reza Armin, MD | 9/4/2007 | r00 |  |  |
| Reviewed by: (Signature) | Date | Revision # | Modification | Related Documents Reviewed/ Updated |
| Ali-Reza Armin,MD | 12/10/2008 | r00 |  |  |
| Anne R. Tranchida,PA(ASCP) | 10/20/2009 | r00 |  |  |
| Ali-Reza Armin,MD | 10/20/2011 | r00 |  |  |
| Ali-Reza Armin,MD | 4/3/2013 | r00 |  |  |
| Mitual B. Amin,MD | 2/14/2015 | r00 |  |  |
| Zhenhong H. Qu, MD | 3/19/2015 | r00 |  |  |
| Kurt Bernacki, MD | 10/27/2017 | r00 |  |  |
| Kurt Bernacki, MD | 10/22/2019 | r00 |  |  |
| Kurt Bernacki, MD | 10/20/2021 | r00 |  |  |