Purpose	To provide a procedure for the dissection of a tongue resection.

Principle	To take histologic sections to demonstrate the pathologic process so that a diagnosis can be made microscopically by a Pathologist.  All margins should be examined microscopically.

_____________________________________________________

Equipment	The following equipment is needed for this procedure:

- Ruler

- Forceps

- Scalpel

- Scissors

- Large Knife

Safety

- PPE should be worn.

- FORMALIN is a known carcinogen.

Supplies and	The following supplies and reagents are needed for this

Reagents	procedure:

- 10% NEUTRAL BUFFERED FORMALIN (pH range 6.9 – 7.2) - BBC Biochemical, Item # 0151 (5 Gallon Cube)

- BLACK INK – Cancer Diagnostics, Inc. Tissue Marking Dye System, Black #0728-1

- BLUE INK – Cancer Diagnostics, Inc. Tissue Marking Dye System, Blue #0728-2

- GREEN INK – Cancer Diagnostics, Inc. Tissue Marking Dye System, Green #0728-3

- ORANGE INK – Cancer Diagnostics, Inc. Tissue Marking Dye Sytem, Orange #0728-6

- RED INK – Cancer Diagnostics, Inc. Tissue Marking Dye System, Red #0728-4

- YELLOW INK – Cancer Diagnostics, Inc. Tissue Marking Dye System, Yellow #0728-5

- WHITE DISTILLED VINEGAR – Reliance Sysco

Quality control

- All remaining tissue should be retained.

Limitations/		The following may influence the validity of test results:

Notes

- The specimen should be fixed in formalin.

_________________

Procedure	Follow the steps listed below:

- Orient the specimen.  If orientation has been provided by the surgeon discuss how the specimen has been oriented (ex:  using sutures or clips).

- Measure the specimen (3 dimensions – cm.).

- Ink the external surface up to six colors.  At least one side of the specimen will have attached mucosa.

- Follow the x margin color code guide and specify in the dictation that the specimen is inked according to the x margin color code.

- The x margin color code is as follows:

- Anterior/Superficial – red

- Posterior/Deep – black

- Superior – Blue

- Inferior – Green

- Lateral – Orange

- Medial – Yellow.

- Apply vinegar to act as a mordant for the ink.

- Serially section the specimen perpendicular to its long axis.

- Locate the pathologic process and describe the size (3 dimensions – cm.), the shape, the color, and the consistency.

- Measure the distance from the pathologic process to all resection margins.

- Further sections the two ends perpendicularly.

- If more than one mass, lesion, or area of abnormality is present, describe and measure the relationship.

- Describe the uninvolved cut surfaces.

- Submit entire specimen in a continuous fashion (unless very large).

- If the specimen is large, submit representative sections of the pathologic process (one section per centimeter) and submit a perpendicular section of each resection margin (include the mass if possible).

- Provide a detailed cassette summary.

- Dictate which direction the specimen is submitted (ex:  lateral to medial).

- A diagram of sections is helpful to the Pathologist assigned to the case.

References		The following items were used as sources of information.

Hruban RH, Westra, WH, Phelps, PH, & Isacson, C:  Surgical Pathology Dissection An Illustrated Guide, New York, NY, Springer-Verlag Inc., 1996.

Lester, SC:  Manual of Surgical Pathology, New York, NY, Churchill

Livingstone, 2001.

##### Document Control

##### Location of Master: Master electronic file stored on the Beaumont Laboratory server under S:\ AP_Grossing_Manual

Number of Controlled Copies posted for educational purposes:  0

Number of circulating Controlled Copies:  1

Location of circulating Controlled Copies: Master Grossing Manual located in Surgical Pathology

_________

##### Document 		The table below is used to capture staff signatures and dates

##### history 	associated with initial document preparation, annual review, and revisions.

| Signature | Date | Revision # |  | Related Documents Reviewed/ Updated |
| --- | --- | --- | --- | --- |
| Prepared by: Anne Tranchida PA(ASCP) | 9/18/2007 | r00 |  |  |
| Approved by: Ali-Reza Armin, MD | 10/23/2007 |  |  |  |
| Reviewed by: (Signature) | Date | Revision # | Modification | Related Documents Reviewed/ Updated |
| Ali-Reza Armin,MD | 10/10/2008 | r00 |  |  |
| Anne R. Tranchida,PA(ASCP) | 10/20/2009 | r00 |  |  |
| Ali-Reza Armin,MD | 10/20/2011 | r00 |  |  |
| Ali-Reza Armin,MD | 10/3/2013 | r00 |  |  |
| Mitual B. Amin,MD | 2/18/2015 | r00 |  |  |
| Zhenhong H. Qu, MD | 4/22/2015 | r00 |  |  |
| Kurt Bernacki, MD | 10/27/2017 | r00 |  |  |
| Kurt Bernacki, MD | 10/21/2019 | r00 |  |  |
| Kurt Bernacki, MD | 10/20/2021 | r00 |  |  |