Purpose	To provide a procedure for the gross examination of a vas deferen(s)  specimen.

Principle	To take a representative histologic section that demonstrates an intact lumen to the Pathologist microscopically.

_____________________________________________________

Equipment	The following equipment is needed for this procedure:

- Ruler

- Forceps

- Scalpel

- Scissors

Safety

- PPE should be worn.

- FORMALIN is a known carcinogen.

Supplies and	The following supplies and reagents are needed for this

Reagents	procedure:

- 10% NEUTRAL BUFFERED FORMALIN (pH range 6.9 – 7.2) - BBC Biochemical, Item # 0151 (5 Gallon Cube)

- BLACK INK – Cancer Diagnostics, Inc. Tissue Marking Dye System, Black #0728-1

- RED INK – Cancer Diagnostics, Inc. Tissue Marking Dye System, Red #0728-4

- WHITE DISTILLED VINEGAR – Reliance Sysco

Quality control

- All remaining tissue should be retained.

Limitations/		The following may influence the validity of test results:

Notes

- The specimen should be fixed in formalin.

_________________

Procedure	Follow the steps listed below:

- The specimen will arrive in a jar of formalin.

- The specimen should be measured in 2 dimensions (cm.).

- The external surface should be described.

- If two segments of tissue are present, the outer surface of one should be inked black.

- Use vinegar as a mordant for the ink.

- The specimen should be serially sectioned.

- The presence of a lumen should be documented and one cross section of each segment should be submitted in one cassette.

- The luminal end of each piece that is submitted should be inked red as a histology embedding aid.  The red inked side of each piece should be embedded down.

- The cassette should be loaded on the next available tissue processor.

References		The following items were used as sources of information.

Hruban RH, Westra, WH, Phelps, PH, & Isacson, C:  Surgical Pathology Dissection An Illustrated Guide, New York, NY, Springer-Verlag Inc., 1996.

Lester, SC:  Manual of Surgical Pathology, New York, NY, Churchill

Livingstone, 2001.

##### Document Control

##### Location of Master: Master electronic file stored on the Beaumont Laboratory server under S:\ AP_Grossing_Manual

Number of Controlled Copies posted for educational purposes:  0

Number of circulating Controlled Copies:  1

##### Location of circulating Controlled Copies: Master Grossing Manual located in Surgical Pathology

_________

##### Document 		The table below is used to capture staff signatures and dates

##### history 	associated with initial document preparation, annual review, and revisions.

| Signature | Date | Revision # |  | Related Documents Reviewed/ Updated |
| --- | --- | --- | --- | --- |
| Prepared by: Anne Tranchida PA(ASCP) | 1/20/2009 | r00 |  |  |
| Approved by: Ali-Reza Armin, MD | 10/24/2009 | r00 |  |  |
| Reviewed by: (Signature) | Date | Revision # | Modification | Related Documents Reviewed/ Updated |
| Ali-Reza Armin,MD | 10/20/2011 | r00 |  |  |
| Ali-Reza Armin,MD | 10/3/2013 | r00 |  |  |
| Mitual B. Amin,MD | 2/18/2015 | r00 |  |  |
| Zhenhong H. Qu, MD | 3/19/2015 | r00 |  |  |
| Kurt Bernacki, MD | 10/27/2017 | r00 |  |  |
| Kurt Bernacki, MD | 10/22/2019 | r00 |  |  |
| Kurt Bernacki, MD | 10/20/2021 | r00 |  |  |