Purpose	To provide a procedure for the dissection of a tonsil or tonsils for a pathologic process.

Principle	To take histologic sections to demonstrate the pathologic process so that a diagnosis can be made microscopically by a Pathologist.

_____________________________________________________

Equipment	The following equipment is needed for this procedure:

- Ruler

- Forceps

- Scalpel

Safety

- PPE should be worn.

- 10% FORMALIN is a known carcinogen.

Supplies and	The following supplies and reagents are needed for this

Reagents	procedure:

- 10% NEUTRAL BUFFERED FORMALIN (pH range 6.9 – 7.2) - BBC Biochemical, Item # 0151 (5 Gallon Cube)

Quality control

- All remaining tissue should be retained.

Limitations/		The following may influence the validity of test results:

Notes

- The specimen should be fixed in formalin unless lymphoma is suspected.

- If lymphoma is suspected, the tonsil should be submitted to the lab either fresh or in saline.

Procedure	Follow the steps listed below:

- Measure the tonsil(s) (3 dimensions- cm).

- Describe the outer mucosal surface(s).

- Serially section the tonsil(s).

- Describe the cut surfaces including the crypts and any material within the crypts.

- Submit a representative section from each tonsil received.

References		The following items were used as sources of information.

Hruban RH, Westra, WH, Phelps, PH, & Isacson, C:  Surgical Pathology Dissection An Illustrated Guide, New York, NY, Springer-Verlag Inc., 1996.

Lester, SC:  Manual of Surgical Pathology, New York, NY, Churchill

Livingstone, 2001.

##### Document Control

##### Location of Master: Master electronic file stored on the Beaumont Laboratory server under S:\ AP_Grossing_Manual

Number of Controlled Copies posted for educational purposes:  0

Number of circulating Controlled Copies:  1

Location of circulating Controlled Copies: Master Grossing Manual located in Surgical Pathology

_________

##### Document 		The table below is used to capture staff signatures and dates

##### history 	associated with initial document preparation, annual review, and revisions.

| Signature | Date | Revision # |  | Related Documents Reviewed/ Updated |
| --- | --- | --- | --- | --- |
| Prepared by: Anne Tranchida PA(ASCP) | 6/18/2007 | r00 |  |  |
| Approved by: Ali-Reza Armin, MD | 10/16/2007 |  |  |  |
| Reviewed by: (Signature) | Date | Revision # | Modification | Related Documents Reviewed/ Updated |
| Ali-Reza Armin,MD | 10/10/2008 | r00 |  |  |
| Anne R. Tranchida,PA(ASCP) | 10/20/2009 | r00 |  |  |
| Ali-Reza Armin,MD | 10/20/2011 | r00 |  |  |
| Ali-Reza Armin,MD | 10/04/2013 | r00 |  |  |
| Mitual B. Amin,MD | 02/18/2015 | r00 |  |  |
| Zhenhong H. Qu, MD | 03/25/2015 | r00 |  |  |
| Kurt Bernacki, MD | 10/27/2017 | r00 |  |  |
| Kurt Bernacki, MD | 10/21/2019 | r00 |  |  |
| Kurt Bernacki, MD | 10/20/2021 | r00 |  |  |