Purpose	To provide a procedure for the processing of a fat pad biopsy for amyloid

Principle	To submit fresh tissue for possible immunofluorescent studies and submit the remainder for permanent section so that a diagnosis can be made microscopically by a Pathologist.

_____________________________________________________

Equipment	The following equipment is needed for this procedure:

- Ruler

- Forceps

- Scalpel

Safety

- PPE should be worn.

- FORMALIN is a known carcinogen.

Supplies and	The following supplies and reagents are needed for this

Reagents	procedure:

- MICHEL’S FLUID – provided by immuno lab

- SMALL CLEAR PLASTIC JARS – ordered by Surgical Pathology

- 10% NEUTRAL BUFFERED FORMALIN (pH range 6.9 – 7.2) - BBC Biochemical, Item # 0151 (5 Gallon Cube)

Quality control

- All tissue should be submitted

Limitations/		The following may influence the validity of test results:

Notes

- The specimen should be received fresh so that half can be placed in Michel’s Fluid for possible immunofluorescent studies

_________________

Procedure	Follow the steps listed below:

- The specimen will be received fresh (preferably 1 cm cube, but can perform on core biopsies).

- Measure the specimen (3 dimensions – cm.).

- Place half of the specimen in Michel’s Fluid to be held for possible immunofluorescent studies.

- Place the other half in a cassette in formalin (for routine H&E, congo red, and crystal violet stains). Include in the dictation to insert the fat pad series under special procedures.

- Load on the appropriate end of day processor.

##### Document Control

##### Location of Master: Master electronic file stored on the Beaumont Laboratory server under S:\ AP_Grossing_Manual

Number of Controlled Copies posted for educational purposes:  0

Number of circulating Controlled Copies:  1

Location of circulating Controlled Copies: Master Grossing Manual located in Surgical Pathology

_________

##### Document 		The table below is used to capture staff signatures and dates

##### history 	associated with initial document preparation, annual review, and revisions.

| Signature | Date | Revision # |  | Related Documents Reviewed/ Updated |
| --- | --- | --- | --- | --- |
| Prepared by: Heather Genson, HTL(ASCP)CMPACM | 6/20/2018 | r00 |  |  |
| Approved by: Mitual B. Amin,MD | 4/7/2019 | r00 |  |  |
| Reviewed by: (Signature) | Date | Revision # | Modification | Related Documents Reviewed/ Updated |
| Kurt Bernacki, MD | 4/8/2019 | r00 |  |  |
| Kurt Bernacki, MD | 10/21/2019 | r00 |  |  |
| Kurt Bernacki, MD | 10/20/2021 | r00 |  |  |