Purpose	To provide a procedure for the dissection of a female cystectomy specimen with or without an attached hysterectomy.

Principle	To take histologic sections to demonstrate the pathologic process so that a diagnosis can be made microscopically by a Pathologist.   All margins and all lymph nodes should be examined.  Any other attached organs should be sampled.

_____________________________________________________

Equipment	The following equipment is needed for this procedure:

- Ruler

- Forceps

- Scalpel

- Scissors

- Probes

- Large Knife

- Pins

- Styrofoam

Safety

- PPE should be worn.

- 10% FORMALIN is a known carcinogen.

Supplies and	The following supplies and reagents are needed for this

Reagents	procedure:

- 10% NEUTRAL BUFFERED FORMALIN (pH range 6.9 – 7.2) - BBC Biochemical, Item # 0151 (5 Gallon Cube)

- BLACK INK – Cancer Diagnostics, Inc. Tissue Marking Dye System, Black #0728-1

- BLUE INK – Cancer Diagnostics, Inc. Tissue Marking Dye System, Blue #0728-2

- WHITE DISTILLED VINEGAR – Reliance Sysco

Quality control

- All remaining tissue should be retained.

Limitations/		The following may influence the validity of test results:

Notes

- The specimen should be fixed in formalin.

- For optimal results, the bladder should be opened, both ureters should be probed, and the specimen should be fixed overnight in formalin.

Procedure	Follow the steps listed below:

- Orient the specimen.  The peritoneum extends further along the posterior aspect.  If other organs are present, they can be used to help orient the specimen.

- Identify all components of the specimen that are present and measure each (3 dimensions - cm.).  For cylindrical structures, use two dimensions (cm.).

- Locate both ureters in the right and left perivesicular adipose tissue, shave each ureter margin, and probe both ureters.

- Ink the outside of the bladder one or two colors (ex: anterior – blue and posterior – black).

- Use vinegar as a mordant for the ink.

- Locate the urethra and shave the surgical margin.

- Open the bladder anteriorly with scissors by making a y-incision from the urethra toward the dome.

- For optimal results, the bladder should be fixed overnight in formalin.  The specimen can be pinned to styrofoam.

- Describe the specimen using a systematic approach (ex: outside to inside).  Carefully, inspect the external surface (attached perivesicular adipose tissue) and the mucosa.

- Locate and describe the pathologic process (size, shape, color, consistency, and location).  Note structures that are involved by the pathologic process.

- Measure the distance from the pathologic process to the surgical margin of the urethra (cm.) and to each ureteral orifice.

- Describe the cut surface of the pathologic process and the layers of the bladder wall that are involved.  If the pathologic process extends through the bladder wall and into an adjacent organ, describe the relationship.

- Measure the thickness of the pathologic process (cm.).

- Measure the uninvolved wall of the specimen.

- Describe any other abnormalities that are present.

- Search all attached adipose tissue for lymph nodes.  Measure all lymph nodes by giving a range in size (from smallest to largest).  If a lymph node is larger than 0.5 cm, bisect, trisect, or serially section it and describe the cut surfaces.

- Describe any other attached organs.

- Submit shave sections of the urethra and both attached ureters (right and left) to be embedded en face.

- Submit sections of the pathologic process including sections that demonstrate the deepest point of invasion and the adjacent structures.

- Sample any other grossly identifiable abnormalities.

- Submit full thickness representative sections of the uninvolved walls away from the pathologic process.  Sample each bladder wall (anterior, posterior, dome, trigone, right lateral, and left lateral).

- Submit representative sections of any other attached organs (ex:  hysterectomy).

- Submit all identifiable lymph nodes.  If no lymph nodes are able to be identified, submit representative sections of attached perivesicular adipose tissue.

- Provide a detailed cassette summary.

- The cassettes should be loaded on the end of day tissue processor.

References		The following items were used as sources of information.

Hruban RH, Westra, WH, Phelps, PH, & Isacson, C:  Surgical Pathology Dissection An Illustrated Guide, New York, NY, Springer-Verlag Inc., 1996.

Lester, SC:  Manual of Surgical Pathology, New York, NY, Churchill

Livingstone, 2001.

##### Document Control

##### Location of Master: Master electronic file stored on the Beaumont Laboratory server under

S:\AP_Grossing_Manual

Number of Controlled Copies posted for educational purposes:  0

Number of circulating Controlled Copies:  1

Location of circulating Controlled Copies: Master Grossing Manual located in Surgical Pathology

_________

##### Document 		The table below is used to capture staff signatures and dates

##### history 	associated with initial document preparation, annual review, and revisions.

| Signature | Date | Revision # |  | Related Documents Reviewed/ Updated |
| --- | --- | --- | --- | --- |
| Prepared by: Anne Tranchida PA(ASCP) | 9/26/2007 | r00 |  |  |
| Approved by: Ali-Reza Armin, MD | 10/16/2007 | r00 |  |  |
| Reviewed by: (Signature) | Date | Revision # | Modification | Related Documents Reviewed/ Updated |
| Ali-Reza Armin,MD | 10/10/2008 | r00 |  |  |
| Anne R. Tranchida,PA(ASCP) | 10/20/2009 | r00 |  |  |
| Ali-Reza Armin,MD | 10/20/2011 | r00 |  |  |
| Ali-Reza Armin,MD | 10/4/2013 | r00 |  |  |
| Mitual B. Amin,MD | 2/18/2015 | r00 |  |  |
| Zhenhong H. Qu, MD | 3/25/2015 | r00 |  |  |
| Kurt Bernacki, MD | 10/27/2017 | r00 |  |  |
| Kurt Bernacki, MD | 10/22/2019 | r00 |  |  |
| Kurt Bernacki, MD | 10/20/2021 | r00 |  |  |