| Methodist Health Services Corporation & UnityPoint Health Methodist |  | Page # 1 of 3 | Section: UPM BBPOL | Policy #: 01.008 |
| --- | --- | --- | --- | --- |
|  | Laboratory BLOOD BANK | Approved by: see signature block at end of policy |  | Date: 06/24/16 |
|  |  | Date Revised: Supersedes 09/09/04,3/07/11, 6/28/12, 7/24/14, 8/27/14, 6/24/16 Date Reviewed: |  |  |
|  |  | Policy/Revision Submitted by: V. Strow/K. Turpin |  |  |
|  |  | JCAHO Standard: NA |  |  |
| POLICY GUIDELINE ON: Handling Blood Products Outside the Blood Bank |  |  |  |  |

- POLICY:

Transfusion services routinely transfer blood and blood components to blood suppliers and locations within the hospital.

- PURPOSE:

To ensure the safety and integrity of blood products transferred within the hospital and to the American Red Cross (ARC) or ARC affiliated hospitals.

- SCOPE:

This policy applies to all blood bank technologists.

- PROCEDURE:

- Transporting Blood products within the Hospital

Blood and blood products taken to the floors must be transfused immediately. Blood and blood products should always be transported in the validated unit transport cooler.  If for some reason, a blood product is taken to the floor and not started, the blood product must be returned to the Blood Bank immediately.   BLOOD PRODUCTS CANNOT BE STORED IN NURSING STATIONS’ REFRIGERATORS.

B.  Transporting Packed Red Blood Cells

1. Packed red blood cells taken to the nursing floors or units.

- Packed red blood cells should be transfused in approximately two hours and no longer than four hours.

- Prolonged warming should be avoided.

- If units are not infused within four hours, they must be discontinued.

- Occasionally, physicians will order a unit to be transfused over 6-8 hours; In this case the unit must be split into two portions so that each portion is transfused in four hours or less.

- If blood is not stored at 1-6ºC, or transported at 1-10ºC, or not returned within 30 minutes, the blood must be destroyed.

2. Blood taken to surgery and open heart must be stored in one of the temperature monitored blood bank

refrigerators at 1-6ºC.

- When blood is returned from surgery and open heart, the Safe-T-Vue sticker must be checked for a white or red color.  White indicates blood was kept at the proper temperature of below 10ºC.  Red indicates the blood was warmed to 10ºC or higher and must be discarded and incident investigated.

C. Transporting Blood Products within the Hospital

1. Fresh Frozen Plasma

- Thawed Fresh Frozen plasma must be destroyed if it has not been stored at 1-6 ºC  or transported at 1-10 ºC for more than 30 minutes.  FFP is acceptable for use if stored at 1-6 ºC for up to  24 hours.

- 5 Day plasma is acceptable for use if stored at 1-6 ºC for up to 5 days.

2. Cryoprecipitate

- Thawed Cryo must be destroyed if it was stored or transported outside 20-24 ºC . Cryo that has been stored at 20-24 ºC is acceptable for use for up to 6 hours.

3.  Plateletpheresis

- Plateleletpheresis must be destroyed if stored or transported outside of 20-24 ºC. To ensure proper agitation and storage temperature platelets must be returned to the lab if not transfused within 30 min of leaving the lab or 6 hours if transported at 20-24 ºC in a cooler.

- Pack cooler with the plateletpheresis on the bottom of the cooler and a room temperature gel pack on top of the plateletpheresis.  The room temperature gel packs will be stored in the coolers.

IMPORTANT NOTES:

- Any damage that occurs to a blood product bag resulting in leakage must be noted in the computer when the unit is returned. Return the unit to a Status of Disposed and discard in a red biohazard bag.  Nursing policy states the Blood Bank must be notified of damages.

- Any blood product that does not appear normal must be discarded (with biohazard precautions) and documented in the computer the final disposition. Form #BP008.HS.134 “Credit Request” must be filled out to receive credit for the discarded unit.

- A blood product which has been returned to the Blood Bank shall not be returned to the available inventory unless the following conditions have been observed.

- The unit closure has not been disturbed.

- During storage and transportation, the blood product has been continuously stored/or transported at the appropriate temperatures.

- The records indicate that the blood product has been inspected prior to return to available inventory.

- Packed Red Blood Cells have not been warmed using a blood warmer.

- For Packed Red Blood Cells, a sealed segment of integral donor tubing has remained attached to the container.

- The  crossmatch tag must be attached to the returned unit.

Returning Blood  Products To ARC and  Transferring Blood Products to ARC Affiliated Hospitals

1. Returning Blood Products to ARC

- Occasionally the ARC will request we return a blood product to them.  When this happens they will fax a copy of “Authorization for Return” form with all the appropriate information about the product to be returned already filled in.

- When the appropriate ARC shipping container arrives, the blood product will be placed on a shipment in the LIS. This will document in the LIS that the blood product was returned to ARC.

- A copy of the form must be made for our records. This copy and the packing list titled Shipping Document from the LIS should be saved with the packing slips from ARC.

2.  Transferring blood products to ARC affiliated hospitals

- Complete “Hospital to Hospital Accounting” form . These forms are located in the desk drawer. (Do not forget to sign the bottom of the form.)

- Remove the appropriate copy when American Red Cross picks the units up for transfer. (Remember it is important to document the correct time and date of pick-up on this form.)We retain the copy that says “sending hospital” .

- When the appropriate ARC shipping container arrives, the unit will be placed on a shipment in the LIS. This will document in the LIS that the product was returned.

- Retain a copy of the form and the Shipping Document with our ARC packing slips.

IMPORTANT NOTE:

- Blood or components must be shipped in a Red Cross shipping container that will maintain a temperature of 1-10º for blood, -18ºC or below for frozen components (Red Cross will supply the dry ice), and 20-24ºC for platelets.

IV.      MAINTENANCE AND STORAGE

  - All policies and procedures are reviewed every two years, (except for Safety procedures which are yearly) by Laboratory Administration and or the Medical Director of the Laboratory or designee. There are changes in practice standards, or requirements.

  - All policies and procedures are reviewed every two years (except for Safety procedures which are yearly) by staff or at the    time new or revised ones are put in effect.

  - All policies are retained 8 years after being discontinued or revised.

  - All procedures are retained 2 years after being discontinued or revised.

| REVISION HISTORY (began tracking 2012) |  |  |  |
| --- | --- | --- | --- |
| Rev | Description of Change | Author | Effective Date |
| 1 | Added how to pack cooler for transporting of platelets | K. Turpin | 7/23/12 |
| 2 | Added scope. Removed form numbers and changed form name per ARC changes. | Kathy Turpin | 7/24/14 |
| 3 | Removed LIS specific references. Added validated transport unit cooler to section A. Added 5 day plasma to section C-1. | Vincent Strow Kathy Turpin | 2/11/16 |

| Reviewed by | Date | Coordinator | Date | Medical Director | Date |
| --- | --- | --- | --- | --- | --- |
| K. Maher | 6/28/12 |  | 6/28/12 |  | 7/3/12 |
|  |  |  | 7/23/12 |  | 7/24/12 |
| D. Allen | 8/1/14 |  | 7/24/14 |  | 8/27/14 |
| V. Strow | 6/23/16 |  | 6/23/16 |  | 6/24/16 |