| METHODIST |  |  | Page 1 of 3 | Section: UPM BBPRO | Policy #: 02.002 |
| --- | --- | --- | --- | --- | --- |
|  | BLOOD BANK PROCEDURES |  | Approved by: see signature block at end of document |  | Date: 2/5/19 Review by: 2/5/21 |
|  | LABORATORY |  | Created: 6/19/03 Supersedes: 9/23/16, 10/12/18, 2/5/9 |  |  |
|  |  |  | Primary Responsible Parties: Michelle Ehrich Secondary Responsible Parties: June Bembenek |  |  |
|  |  |  | CAP Standard: NA |  |  |
| SUBJECT: |  | ABO Slide Typing |  |  |  |

- Principle

ABO slide typing is performed at the patient’s bedside prior to transfusing the initial unit of packed red blood cells. ABO slide typing insures compatibility of the blood product and is a secondary means of verifying patient identity. Agglutination of red cells with a given antiserum indicates the presence of the corresponding antigen on the red cells and is a positive test result. Absence of agglutination indicates the corresponding antigen is not present.

Any patient presenting for ABO/Rh testing and transfusion of packed red blood cells for the first time must have the ABO/Rh type confirmed by a second specimen drawn at a separate time or by a second phlebotomist. If a second specimen cannot be collected, a bedside type can be used for the confirmation.

Any patient, regardless of ABO type, must have a bedside type completed prior to the transfusion of the first unit of packed red blood cells if the type has not been confirmed. Additionally, all patients with an ABO type other than O must have a bedside type completed prior to the transfusion of the first unit of packed red blood cells once per admission.

- Specimen

Whole blood from a finger stick or anticoagulated blood in K2 EDTA.

- Policy Scope

Blood bank technologist

- Reagents

| Reagent | Storage | Stability |
| --- | --- | --- |
| Monoclonal Anti-A Antisera | 2-8°C | Stable through marked expiration |
| Monoclonal Anti-B Antisera | 2-8°C | Stable through marked expiration |

- Instrumentation/Equipment

- Micropipette

- Glass slide

- Applicator Stick

- Quality Control
To be performed once per lot number per day of testing. Refer to UPM BBQA 03.003: Reagent Quality Control for complete procedure.

- Procedure

- Place one drop of Anti-A on a clean, labeled glass slide.

- Place one drop of Anti-B on a clean, labeled glass slide.

- Add one drop of patient whole blood to each drop of antisera on slide.

- Mix antisera and blood thoroughly, using a clean applicator stick for each.

- The mixture should spread over an area of approximately 20 x 40 mm.

- Gently tilt the slide continuously for up to two minutes. Do not place slide over heated surface.

- Read, interpret, and record results and your initials on patient wristband and the patient’s blood bank identification (BBID) band. If the patient is in OH/OR, record the results and your initials on the face-sheet and crossmatch tag.

- Interpretation of Results

Negative:  Test that shows no agglutination within 2 minutes.

Positive:  Any agglutination present.

| Anti-A | Anti-B | ABO Group Interpretation |
| --- | --- | --- |
| Neg | # Neg | O |
| Pos | Neg | A |
| Neg | Pos | B |
| Pos | Pos | AB |

- Reporting Results

Add on a bedside type in the LIS and record reaction results.

- Procedural Notes:

In the event the bedside slide typing does not match the documented patient blood type, proceed through the following steps until the error is found and can be corrected:

- Do not administer blood or blood components.

- Perform clerical check to verify correct patient, and that the component tag and patient’s historical blood bank record correlate.

- Repeat patient bedside slide type and patient type from original tube in blood bank.

- Redraw patient and repeat blood type from new specimen.

- References

AABB Technical Manual, 17th Edition, 2011, 877-878.

| POLICY CREATION : |  |
| --- | --- |
| Author: Gayle Shaikh | Date: September 17, 1985 |
| Medical Director: Douglas McGrady, MD | Date: June 19, 2003 |

| REVISION HISTORY |  |  |  |
| --- | --- | --- | --- |
| Rev | Description of Change | Author | Effective Date |
| 0 | Initial electronic release. | M. McCallister | 6/19/03 |
| 1 | Minor formatting changes, added test interpretations, added Document ID in header. Updated for HLAB. | Seth Schaffer | 7/14/11 |
| 2 | Grammatical changes | Kathy Turpin | 7/22/13 |
| 3 | Changed storage temp of reagents. Added if the patient is in OH/OR record the results and your initials on the facesheet, crossmatch slips, and patient ID tag attached to the cross matched units of blood. | Kathy Turpin | 02/27/14 |
| 4 | Removed LIS specific references. Removed requirement to document bedside type on HBB forms. | Kathy Turpin | 9/16/16 |
| 5 | Removed requirement for bedside type per admission. | Vincent Strow | 3/6/17 |
| 6 | Added that the bedside type should also be recorded on the patient’s BBID band. Minor formatting. Bedside type on all patients regardless of ABO type. | Leah Miller | 10/12/18 |
| 7 | Clarified bedside type requirements. Referred QC to appropriate procedure. Minor formatting | Michelle Ehrich | 01/31/19 |

REVIEWED BY

| Lead | Date | Coordinator/ Manager | Date | Medical Director | Date |
| --- | --- | --- | --- | --- | --- |
| K. Maher | 12/30/11 |  | 8/26/11 |  | 8/24/11 |
| K. Maher | 8/6/13 |  | 7/10/13 |  | 8/8/13 |
| D. Allen | 2/27/14 |  | 2/27/14 |  | 2/27/14 |
|  |  |  | 9/16/16 |  | 9/23/16 |
| L. Miller | 10/12/18 |  | 10/12/18 |  | 10/12/18 |
| Michelle Ehrich | 1/31/19 |  | 1/31/19 |  | 2/5/19 |