Entering Blood, Blood Products and Tissues into the Blood Bank Meditech System

# Principle

  - All blood, blood products and tissues must be entered into the hospital computer system for future use.  All products must be traceable from time of receipt to issue.

# General Policies

## All blood, blood products and tissues must be entered into the system prior to use.

## Bar codes on blood and blood products must be used, whenever possible.

## As blood products are removed from the shipping container, the unit numbers must be compared to the O & D sheet for accuracy of shipment.  Any discrepancies should be brought to the attention of the supervisor or designee and the blood provider.

## Acceptability of product and initials of the personnel removing products from the shipping container must be documented on the O & D sheet.

### This is to include:

#### Date received

#### Time received

#### Tech initials

## Non-red cell products may be placed on the shelf for use after entry in the hospital computer system.

## Red cell products will be placed in the unprocessed refrigerator until confirmation typing is completed.  (Refer to BBBP 5.0-Processing Blood Products in the Meditech System)

## Directed donor units and autologous will not be accepted into inventory.

# Specimen Collection and Preparation

  - N/A

# Equipment

## Meditech LIS

# Supplies

## Bar code labels

## Test tubes

# Reagents

  - N/A

# Quality Control

  - N/A

# Safety

  - Refer to Chemical Hygiene and Blood Borne Pathogen Plan for Memorial Hospital Laboratory.

- Procedure

  - ISBT Labeled Products

    - From the main desktop, open the BBK Unit desktop.

    - Choose the Single option on the right menu bar.

    - Open Enter Units or Enter Units (Quick).

    - Source:  Scan the ISBT unit number.  Verify source is correct.

    - Scan the following bar codes in any order:

      - ISBT unit number (again)

      - ISBT product code

      - ABO/Rh

      - Expiration Date

      - CMV negative label, if applicable.

    - Enter the Received Date:  Enter date received from O & D sheet, if different from default.

    - Enter the Received Time:  Use current default time.

      - New date and time will remain until menu item is exited.

    - Volume:

      - A default volume will be entered for all red cell products

      - For non-red cell products, enter to the volume field and type the volume.

    - Click Save after each product is entered.

    - Click Close after the specimen number message window appears.

    - Click Cancel when all units have been entered.

    - For Red Cell Products only:

      - Apply printed Meditech specimen label to a test tube.

      - Remove one (1) or two (2) segments from unit and place in properly labeled test tube for ABO/Rh confirmation processing.

#### Remove one (1) segment from unit and place in properly labeled specimen bag for that week’s unit segments for any potential additional testing.

        - Segments will be kept in walk-in refrigerator in appropriate bag until needed or discarded.

## Codabar labeled products

### Refer to supervisor or designee if received.

## Editing Blood Products for Additional Attributes

### From the main desktop, open the BBK Unit desktop.

### Choose the Single option on the right menu bar.

### Choose Change Units on the right menu bar.

### Choose Edit Units.

### Scan the unit number bar code.

### Open the More Data tab

#### Enter to the desired field (antigens, antibodies, markers, comment, etc.)

##### NOTE: CMV negative should be scanned from product label.

##### Antibodies and antigens should only be entered in this module if typed or identified by blood center.

#### Press the F9 (look up) key to choose desired entry, or

#### Type desired mnemonic, if known.

### Click Save button to file.

## Tissue Products

### From the main desktop, open the BBK Unit desktop.

### Choose the Single option on the right menu bar.

### Choose Enter Unit

### Click Save to pass the barcode field

### Unit Source:

#### Press the F9 key and choose correct source, or

#### Type mnemonic, if known.

### Product:

#### Press the F9 key and choose correct product, or

#### Type mnemonic, if known

### Received date and time will default to current

### Volume:  enter 1 (one). Each tissue must be brought in separately.

### BBK Unit Number:  The next number in the series will be assigned by the Meditech system.

#### All tissue products:  Type T and enter

        - NOTE:  The assigned Meditech unit number will not be visible at this screen.         The unit number will appear when the product is filed.

### Lot Number:  Type the lot number of the product.

### Enter to Expiration date:

### Enter expiration date listed by manufacturer

### Expiration time will default to 2359

### Open the More Data tab and place the cursor in the Comments field.

#### Type the dimensions of any product, if applicable.

#### Type any other information specific to that product

### Click the Save button to file.

## Printing Tissue Inventory Cards

### From the main desktop, open the BBK Unit desktop.

### Choose the Single option on the right menu bar.

### Choose Print Units.

### Enter the desired tissue unit number.

### Type Y in the Internal field.

### Click OK button.

### Click Print on the right menu bar.

### Enter desired printer location and click OK.

## Printing Tissue Labels

### From the main desktop, open the BBK Unit desktop.

### Choose the Single option on the right menu bar.

### Choose Unit Labels.

### Type desired unit number(s).

### Label format:

#### Type BBKNONBLD or

#### Press the F9 key and select the correct format

### Enter desired printer location.

#### Blood Bank processing counter – LABBBPL02

#### Blood Bank component area – LABBBPL01

### Attach label to the Tissue Inventory Card prior to storage in the incomplete drawer.

### Repeat steps a-e to print another label.

### Attach the second label to tissue in such a way that the product will not be defaced.

# References

## Meditech Users Manual.

PROCEDURE AND FORM CHANGE CONTROL

| #### Title: Entering Blood Components into the Blood Bank Meditech System |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ##### Written |  | Validated |  | Path Review |  | Review |  | Effective |  | Reason for Revision |
| ##### Date | By | Date | By | Date | By | Date | By | Date | By |  |
| 2/10/10 | PAB | 2/14/10 | GJM | 3/2/10 | ESB |  |  | 3/2/10 | PAB |  |
| Revised |  |  |  |  |  |  |  |  |  |  |
| 5/11/10 | PAB | 5/14/10 | GJM |  |  |  |  |  |  | Include rhogam, factor and tissue |
| 07/10/10 | gjm |  |  |  |  |  |  | 7/19/10 | PAB | Adding viral testing to tissues |
| 11/30/10 | PAB | 11/30/10 | GJM |  |  |  |  | 11/30/10 | PAB | Adding directions for donor-directed and autologous unit entry |
| 2/4/11 | PAB | 2/10/11 | CPZ | 2/14/11 | ESB |  |  | 2/15/11 | PAB | Added editing for additional attributes |
| 4/18/11 | PAB | 4/25/11 | MLH | 4/28/11 | ESB |  |  | 6/1/11 | PAB | Updated for new Meditech version |
|  |  |  |  |  |  | 8/2/12 | PAB |  |  |  |
| 5/7/13 | PAB |  |  | 5/15/13 | ESB |  |  | 6/1/13 | PAB | Removed autologous/directed units |
| 1/21/15 | JLH |  |  | N/A | N/A |  |  | 1/21/15 | JLH | Removed Codabar instructions and coag factor instructions. Added scanning of CMV label. |
| 10/9/15 | JLH |  |  | N/A | N/A |  |  | 10/9/15 | JLH | Added new header and reformatted to fit. |
| 1/26/16 | JLH |  |  |  |  |  |  | 1/26/16 | JLH | Updated segment storage from with unit to refrigerator bags. |

Location of any copy(s) of the procedure:

Out of use:

Date: _______________By:_______________Reason:___________________________