FMH Rapidscreen Procedure

Blood Bank

| ### Policy Type: Departmental Policy Owner: Director of Laboratory Services Responsible Executive: VP Operations | Policy Contact: Dir. of Lab., ext. 1787 |
| --- | --- |

TRM.40790

Policy Statement

The erythrocyte rosetting test is more sensitive than the microscopic examination of the weak D test.  This method will detect fetomaternal hemorrhages of approximately 10 ml.  Such sensitivity provides a margin of safety that is desirable for a screening test.  This screening test detects those cases in which there may have been a larger than usual feto-maternal hemorrhage.  Positive results require confirmatory testing by a quantitative method at a reference laboratory.

Purpose or Reason for Policy

The erythrocyte rosetting test is more sensitive than the microscopic examination of the weak D test.  This method will detect fetomaternal hemorrhages of approximately 10 ml.  Such sensitivity provides a margin of safety that is desirable for a screening test.  This screening test detects those cases in which there may have been a larger than usual feto-maternal hemorrhage.  Positive results require confirmatory testing by a quantitative method at a reference laboratory.

A red blood cell suspension from the D-negative mother is first incubated with monoclonal IgM anti-D serum and washed to remove all unbound antibody, as in the incubation and washing phases of the weak D test.  Instead of adding Anti-Human Globulin to the washed cells, however, a weak suspension of the D-positive red cells is added.  After shaking the tube, the cell mixture is centrifuged and examined microscopically for mixed-field agglutination.  Since any minor population of D-positive cells will have become coated with anti-D during the initial incubation, the D-positive indicator cells added after washing form clumps around the individual cells of the minor population, leading to larger and more readily-detected agglutinates than could be expected to occur through the agglutination of a relatively small number of coated D-positive cells by Anti-Human Globulin.  In most cases the feto-maternal hemorrhage is not sufficient to cause a positive test, but in those cases where a significant volume of fetal blood enters the maternal circulation the test alerts the technologist that a quantitative test is required to determine if the bleed was sufficient to warrant the administration of a larger dose of Rh Immune Globulin to the mother.

Procedures

EQUIPMENT:

1.  Calibrated centrifuge                                                 3.   Microscpe                                   5.  Timer

2.  Test tube rack                                                            4.  Cell washer

SUPPLIES AND REAGENTS:

1.  12 X 75 mm. & 10 X 75 mm. test tubes                     3.  Blood Bank saline

2.  Disposable pipettes                                                   4.  FMH RapidScreen Kit (NEW 1/11/13)

FMH RapidScreen Procedure

SPECIMEN:

1.  Refer to the General Specimen Collection Procedure for specimen collection and

identification/handling requirements.

2.  Collect the specimen in properly labeled (refer to the Specimen ID Requirements), pink top tube

drawn from the mother after delivery (post-partum blood - within 72 hours after delivery).

Specimens in red top tubes (with gel) are not acceptable.

PROCEDURE:

1.  Make a 3-4% suspension of the maternal red blood cells in saline according to written procedure.

2.  Label three 12 x 75 mm. test tubes, one positive control, one negative control, and one patient's

last name.

3.  Place one drop of controls and one drop maternal cell suspension in the appropriate tube.

4.  Add one drop of monoclonal Anti-D serum.

5.  Mix well and incubate for 5 minutes at room temperature (18°C to 30°C).

6.  Place tubes in cells washer and wash for EIGHT cycles.

7.  At the end of the last cycle, remove tubes and add one drop of Indicator cells and mix thoroughly

by shaking the tube.

8.  Centrifuge for 20 seconds.

9.  Resuspend the cell button completely.  Transfer the cell suspension into a 10 X 75 test tube

and examine five (5) low-power fields (10X) microscopically for mixed-field agglutination.

Microscopic examination can be carried out either in the tube or on a microscope slide.  If clumps

are seen in the tube, the contents should be transferred to a microscope slide so that the number of

clumps per low-power field can be evaluated.

10.  Interpret results immediately upon completion of the test as stated below.

11.  Record the test results in the computer.

12.  If the test result is positive, send the specimen to the reference laboratory according to the

Kleihauer - Betke Stain Send Off Procedure.

INTERPRETATION OF RESULTS:

Positive test:  After examining five low-power fields, if five or more clumps of agglutinated red blood

cells are observed, the test is positive and indicates the presence of D-positive fetal red blood cells in

possibly significant numbers in the maternal blood.

Negative test:  After examining five low-power fields, if four or fewer clumps of agglutinated red blood

cells are observed, the test is negative, indicating that a large feto-maternal hemorrhage did not occur.

The presence of agglutinated cells constitutes a positive test result and indicates the presence of D-positive cells in possibly significant numbers in the maternal blood.  A positive test merely provides evidence that a potentially large feto-maternal bleed may have occurred.  A quantitative test procedure is required to determine the volume of feto-maternal hemorrhage as a means of estimating dosage of Rh Immune Globulin needed to prevent Rh immunization.  The absence of agglutinated cells constitutes a negative test result and indicates that a large feto-maternal bleed did not occur.

QUALITY CONTROL:

1.  Only those technologists whose names are listed on the Technologist Authorization Record may

perform this procedure.

2.  Quality control red cells and serum undergo daily testing to assure the reliability of the test results

with the commercial ABO and Rh Antisera, commercial red blood cells, LISS, and anti-human (IgG)

serum.

3.  Perform patient testing only if all QC testing demonstrates the expected results.  Any discrepant QC

result must be resolved prior to patient testing.

4.  The positive control test with the supplied mixture of adult D-negative and fetal D-positive red cells

confirms the reactivity of the indicator cells and provides an indication that the test is being

employed has been sufficient to remove all unbound Anti-D.

5.  Record the QC results in the computer along with the patient test result.

FMH RapidScreen Procedure

NOTES AND COMMENTS:

1.  The FMH RapidScreen Test will detect feto-maternal hemorrhage whenever 30 ml. or more of

ABO compatible D-positive fetal blood has entered the maternal circulation.

2.  Care should be taken in the washing phase to ensure adequate washing of the red cells after

incubation at room temperature.

3.  Follow the computer downtime procedure during any computer downtime.  Use manual

worksheets, forms and tags during this time.

REFERENCES:

1.  American Association of Blood Banks:   Perinatal Issues in Transfusion Practice in TECHNICAL

MANUAL, 15th Edition; Brecher, Mark, editor.  AABB, Bethesda, Maryland, 535-556, 2005.

2.  CAP Transfusion Medicine Checklist, Revised 01/04/2012.

3.  Immucor, Inc.:  Insert to "FMH RapidScreen," Norcross, GA, Rev. 3/12.

Forms/Instruction

NA

Definitions

NA

Signatures

| Written By: | Elizabeth Fowler, MT(ASCP) |  | Original Date: Effective Date: | 1/10/13 1/10/13 |  |
| --- | --- | --- | --- | --- | --- |
| Supersedes: | BBK/Type and Cross Procedures/Fetal Bleed Screening Test Procedure (Reviewed 12/2/12) |  |  |  |  |
| CRHS | Medical Director |  |  |  | Date |
|  | Mett B. Ausley, Jr., MD |  |  |  |  |
| ACTION |  | Signature |  |  | Date |
| Reviewed |  |  |  |  |  |
| Reviewed |  |  |  |  |  |
| Reviewed |  |  |  |  |  |
| Reviewed |  |  |  |  |  |