PURPOSE:

To determine the presence or absence of bacterial pathogens in urine specimens.

SAMPLE INFORMATION:

Early-morning specimens are preferable – allowing urine to remain in the bladder for at least 4 hours will decrease the number of false – negative results.

| Type | The urine specimens submitted to the laboratory for culture are either: - Fresh urine collected in sterile container. - Fresh urine collected in urine transport tube. |  |
| --- | --- | --- |
| Source | Voided Urines (non-sterile) | • Midstream urine (MSU) • Neonatal bagged urine • Indwelling catheter (Foley) urine • Ileal conduit urine |
|  | Aseptically collected Urines (sterile) | • Straight or ”in and out” catheter • Nephrostomy urine • Cystoscopy urine • Suprapubic bladder aspirate |
| Stability | - Fresh urine collected in sterile container is acceptable for 24 hours if refrigerated. - Fresh urine collected in urine transport tube is acceptable for 72 hours (refrigeration not necessary). |  |
| Storage Requirements | - Fresh urine without preservative should be refrigerated until processing. - Fresh urine collected in urine transport tube can be kept at room temperature. |  |
| Criteria for rejection and follow up action | - Unrefrigerated fresh urine received longer than 2 hours after collection. - Refrigerated fresh urine specimens received longer than 24 hours after collection. - 24 hour urine collections. - Duplicate specimens obtained with same collection method within 48 hours. - Foley catheter tips. - Urine from the bag of a catheterized patient. - Specimens in leaking container and unlabeled specimens. |  |

REAGENTS and/or MEDIA:

- Blood Agar and MacConkey Agar

- Identification reagents:  catalase, oxidase, spot indole, PYR, tube coagulase, etc.

SUPPLIES:

- 0.001 mL loops

- Vitek 2 Compact and supplies

- Biosafety cabinet

- 35° ambient air incubator

SPECIAL SAFETY PRECAUTIONS:

Containment Level 2 facilities, equipment, and operational practices for work involving infectious or potentially infectious materials or cultures.

- Lab gown must be worn when performing activities with potential pathogens.

- Gloves must be worn when direct skin contact with infected materials is unavoidable.

- Eye protection must be used where there is a known or potential risk of exposure to splashes.

- All procedures that may produce aerosols, or involve high concentrations or large volumes should be conducted in a biological safety cabinet (BSC).

- The use of needles, syringes, and other sharp objects should be strictly limited.

All patient specimens are assumed to be potentially infectious.  Universal precautions must be followed.  Since viable micro-organisms are used, all cultures must be handled with appropriate precautions.  All equipment in contact with cultures should be decontaminated by appropriate methods.

QUALITY CONTROL:

Refer to Quality Control procedures as required for individual identification and susceptibility procedures used in evaluating and testing.

PROCEDURE INSTRUCTIONS:

| Step | Action |  |
| --- | --- | --- |
| Follow the steps in the table below to set-up the culture for a urine specimen |  |  |
| 1 | - Hold a 0.001 mL loop vertically and immerse just below the surface of a well-mixed urine specimen. - Deliver a loopful of urine onto appropriate agar plates. - Make a straight line down the center of the plate. - Streak the urine by making a series of passes at 90° angles through the inoculum. ***Note: the same loop can be used for each plate per patient sample. The loop must be re-dipped for every plate. |  |
| 2 | If: | Then: |
|  | Voided Urines (non-sterile): • Midstream urine (MSU) • Neonatal bagged urine • Indwelling catheter (Foley) urine • Ileal conduit urine | - Inoculate BAP and MAC - Incubate plates for 18-24 hours at 35° in the ambient air incubator |
|  | Aseptically collected Urines (sterile): • Straight, intermittent or ”in and out” catheter • Nephrostomy urine • Cystoscopy urine • Suprapubic bladder aspirate | - Inoculate BAP and MAC - Incubate plates for 48 hours at 35° in the ambient air incubator |

INTERPRETATION OF RESULTS:

Reporting Results:

- Using a 0.001 mL loop, 1 colony equals 1 X E6 cfu/L.

- Determine the colony count and extent of the work-up required for each morphotype on the plates.

Type of Urine:  Non-sterile urine

| Colony count | 1 uropathogen | 2 uropathogens | 3 or more uropathogens |
| --- | --- | --- | --- |
| ≤ 10 (≤ 10 X E6 cfu/L) | No workup; Report: “No Significant Growth” | No workup; Report: “No Significant Growth” | No workup; Report: “No Significant Growth” |
| 11-99 (11-99 X E6 cfu/L) | ID and susceptibility testing | ID and susceptibility testing on both | No workup; Report: “}CON1 Mixed Culture, Repeat” |
| ≥ 100 (≥ 100 X E6 cfu/L) | ID and susceptibility testing | ID and susceptibility testing on both | No workup; Report: “}CON1 Mixed Culture, Repeat” |
| Uropathogens >10 XE6 cfu/L and other isolates ≤10 XE6 cfu/L | ID and susceptibility on isolate >10 XE6 cfu/L and ignore isolates ≤ 10 XE6 cfu/L | ID and susceptibility on both isolates >10 XE6 cfu/L and ignore isolates ≤10 XE6 cfu/L | No workup; Report: “}CON1 Mixed Culture, Repeat” |
| >10 XE6 cfu/L Normal skin/urogenital flora | No workup; Report: “Usual Urogenital Flora” | No workup; Report: “Usual Urogenital Flora” | No workup; Report: “Usual Urogenital Flora” |

Type of Urine:  Sterile urine

| Colony Count | Any number of morphotypes |
| --- | --- |
| Any growth | Perform ID and susceptibility testing |
| No growth after 48 hour incubation | “}NG2D” |

List of Uropathogens and Non-Uropathogens:

| Uropathogens | Non-uropathogens (normal skin/urogenital flora) |
| --- | --- |
| Enterobacteriacea Pseudomonas aeruginosa Other gram negative bacilli Enterococcus species Streptococcus pyogenes Streptococcus agalactiae Yeast Aerococcus urinae* Corynebacterium urealyticum Staphylococcus aureus Staphylococcus saprophyticus Other coagulase negative staphylococcus | Lactobacillus Diptheroids (not C.urealyticum) Viridans Streptococci ( not A.urinae) Bacillus species Neisseria spp. |

* Considered a uropathogen only if colony count is 10 times greater than that of all other microbiota.

| ORGANISM | BIOCHEMICAL TESTS & EXPECTED RESULTS | ID & SENS |
| --- | --- | --- |
| E.coli | LF, flat, deep pink Spot Indole: Positive Oxidase: Negative PYR: Negative | Vitek AST- N213 |
| Enterobacteriaceae | LF or NLF Spot Indole: Negative/Positive Oxidase: Negative PYR: Positive/Negative | Vitek GN & AST-N213 |
| P.aeruginosa | NLF, typical smell/morphology Spot Indole: Negative Oxidase: Positive | Vitek GN & AST-N213 |
| Non-fermenters | NLF Oxidase: Positive/Negative | Vitek GN & AST-N213 |
| Group A Strep | Beta hemolysis GAS latex: Positive | Add comment: “This organism is generally susceptible to beta lactam antibiotics” |
| Group B Strep | Small or no zone of beta hemolysis GBS latex: Positive | Add comment: “This organism is generally susceptible to beta-lactam antibiotics. This organism is resistant to TMP-SMX”. |
| S.aureus | Catalase Positive, Slide Coagulase Positive, Tube Coagulase Positive | Vitek AST-GP67 |
| S. saprophyticus | Catalase Positive, Slide Coagulase Negative, Novobiocin Resistant | Add comment: “This organism is generally susceptible to TMP-SMX, nitrofurantoin and urinary quinolones” |
| Enterococcus | Catalase Negative PYR Positive | Vitek AST-GP67 |
| Yeast | Wet prep = yeast | Vitek YST ID card: |
| Aerococcus urinae | Resembles viridans Streptococcus Gram stain: GPC, tetrads Catalase: Negative PYR: Negative | Vitek GP Perform susceptibility testing as per DynaLIFE ASTM |
| Diptheroides | Catalase positive - Rule out uropathogenic Corynebacterium urealyticum (urea+) if numbers are significant. | Urea |

Limitations:

- A mixed culture in an uncomplicated outpatient population likely indicates contamination.

- For uncomplicated UTI, culture is usually not indicated.

- False-negative results may be due to interfering substances, diluted urine, low urine pH and subjective interpretation of the criteria for further workup of the culture.

REFERENCES:

- Clinical Microbiology Procedures Handbook, 4th edition, ASM Press, 2016

- Jorgensen J.H., Pfaller M.A., Carroll K.C., Funke G., Landry M.L., Richter S.S., Warnock D.W. 2015.  Manual of Clinical Microbiology, 11th edition, ASM Press, Washington, D.C.

- J Obs Gyn Can 2004; 26(9):826-32

REVISION HISTORY:

| REVISION | DATE | Description of Change | REQUESTED BY |
| --- | --- | --- | --- |
| 1.0 | 21Apr17 | Initial Release | L. Steven |