PROCEDURE: 	Processing of Blood Samples for Testing at the Coro Molecular Microbiology Lab.

- PRINCIPLE

  - The following procedure is utilized for the processing of blood samples to obtain plasma or serum for testing and storage in the Coro Molecular Microbiology Lab

  - Test orders to be processed:

    - HIVL2– lavender only

    - HIVLX - lavender only

    - PHSV – lavender only

    - HCVL1 – lavender or PPT

    - HCVCK – lavender or PPT

    - HBVL1 – lavender or PPT

    - CMVLD - lavender or PPT

    - BKVLD - lavender or PPT

    - XHIV - gold only

    - XVLD – red only

- SPECIMEN TYPE

  - The order codes to be processed and their corresponding acceptable collection tube is listed above.

  - If the wrong tube is collected call and ask for a recollection in the correct tube.

- PROCEDURE

  - Phlebotomy receive the sample and reprint a new label.

  - Place the reprinted label on an aliquot tube and place in rack in front of sample to be processed (use the tubes and caps provided).

    - If multiple specimens are sent on one patient all tubes should be aliquoted. If there is a short draw process as usual and send over to Coro to investigate.

    - Samples can remain at room temperature until processed.  Processing can occur in batches, but they have to be spun and transferred to the labeled aliquot tube within 6 hours of collection.

  - Tubes should be spun at 3000 rpm for 20 minutes. Remove plasma/serum and transfer to aliquot tube using a sterile disposable plastic pipet.

    - Each order SHOULD have its own collection tube, however occasionally a tube will have more than 1 test order on it. When this happens, please print 2 barcode labels and aliquot into separate tubes. Cross out one of the orders on each aliquot.

  - PPT tube orders received at TMH do not need to be aliquoted prior to shipping to Coro.

    - Store tubes refrigerated in sample racks at TMH.

    - Sample racks are placed in biohazard specimen transport bags and sent to RIH Micro.

    - Sample racks are stored in the -70C freezer at RIH.

    - Frozen PPT tubes are packed in the appropriate rack with ice packs, placed in the transport cooler, and transported to the Coro Molecular Micro Lab

  - Storage and shipping

    - PHSV samples should be placed in the refrigerator at 2-8˚C.

    - All other samples at RIH are stored in the freeze at -70˚C

    - All other samples at TMH and NH are stored in the refrigerator at 2-8˚C

    - Track samples and ship to RIH with ice packs.

    - Samples can be sent on every courier run (including weekends).

    - Samples received at RIH from TMH and NH should be frozen at -70˚C.

      - All frozen samples should be sent on a tracking list in a cooler with ice packs to the Coro Molecular Microbiology lab on the 2 daily runs Mon-Fri. On weekends they will be stored at RIH at -70C until Monday AM.

- REVISIONS

  - 3/6/2017 Added HIVLX tube processing instructions.

  - 3/6/2017 Added AP A for processing primary tubes when received at Coro.

  - 7/19/2017 Deleted Hepatitis C Qualitative test from procedure

  - 7/19/2017 Added PPT tubes as an acceptable collection tube for Hepatitis C Quantitative Viral Load assay.

  - 1/24/2018 Updated to be specific to Coro Molecular Micro

  - 9/25/2019 Added PPT tubes from TMH on HCVCKs do not need to be aliquotted.

  - 9/25/2019 Reformatted procedure to remove footnote with performing laboratory name and location and updated signage page.

  - 3/20/2020: Updated test names per changes made due to Cobas 6800 implementation.

  - 1/22/2024: Updated test menu

AP: A

Processing EDTA and PPT Tubes at Coro

- Procedure for Processing plasma samples received in EDTA tubes at Coro:

  - Orders may include: HIVL2, HIVLX, HCVL1, HBVL1, HCVG1, and HCVCK.

- Print a label for aliquot tube.

- Tubes should be spun at 3000 rpm for 20 minutes.

  - Centrifuge is located in the pre area.

  - Turn on with switch in back on left side.

  - Use Program 2

- Aliquot tubes, Caps, and Sterile Transfer Pipettes are located in the cabinet below the centrifuge.

- Label aliquot tube with specimen label.

- Remove plasma and transfer to aliquot tube (using the face shield) with a sterile disposable plastic pipette.

- Cap tube and freeze in -80˚C freezer “RI MM F3”. Place tube in the labeled rack for the test order to be processed.

- Processing plasma samples received in PPT (Pearl Top) tubes at Coro:

  - Orders may include: HCVL1, HCVG1, and HCVCK.

- Print a label for aliquot tube.

- Tubes should be spun at 3000 rpm for 20 minutes- see above for centrifuge instructions.

Note: If plasma is already separated in tube go to next step.

- Aliquot tubes, Caps, and Sterile Transfer Pipettes are located in the cabinet below the centrifuge and on the wire shelf in the Hood room.

- Label aliquot tube with specimen label.

- Remove plasma and transfer to aliquot tube (using the face shield) with a sterile disposable plastic pipette.

- Cap tube and add to labeled rack for testing or freeze in -80˚C freezer “RI MM F3”. Place tube in the labeled rack for the test order to be processed.