| ## POLICY STATEMENT | When organisms split urea, ammonia is formed during incubation which makes the reaction of this media alkaline, producing a red- pink color. Consequently, urease production may be detected by the change in the phenol red indicator. |
| --- | --- |
| ## PURPOSE | This procedure provides technical instruction for the performance of the Urea Slants test. |
| ## SCOPE | This procedure applies to testing personnel authorized to perform testing. This group includes, but is not limited to Laboratory Technologists as well as leads and supervisory personnel. |
| ## RESPONSIBILITY | All the above personnel are responsible for following the Urea Slants procedure without exception. In addition, testing personnel are also responsible for evaluating the results and taking proper remedial action. |

# REAGENTS

Urea Agar Slant

Store at 2ºC to 8ºC. DO NOT use the beyond the expiration date.

# QUALITY CONTROL

Quality control is performed by the manufacturer according to CLSI Guidelines.

##### PROCEDURE

- Using an isolated colony from an 18 - 24 hour culture, inoculate the urea slant by streaking back and forth. Do not stab the butt since it serves as a color control.

- Incubate with loose caps at 35ºC.

- Observe tubes after 2, 4, 6 and 24 hours. (Some organisms may require longer incubation periods).

# REPORTING

- Positive = intense pink-red (red-violet) color on the slant.

The color may permeate into the agar butt. The extent of the color indicates the rate of urea hydrolysis. Most Proteus Species will give a positive reaction within 4 hours. Other Enterobacteriaceae require 24 hours or more to demonstrate a positive slant reaction.

- Negative = no color change; the agar medium remains pale yellow to buff.

Enter the result in the appropriate spot in the Meditech workcard.

# REFERENCES

BD Quality Control and Product Information Manual for Tubed Media, January 2011