TITLE: Pathology Specimen Transport

POLICY: To ensure that pathology specimens are handled and transported to Pathology according to policy to avoid lost specimens.

PROCESS:

There are 2 different processes according to what Winchester Hospital site the specimen first arrives at:

- Core Process:

  - Pathology specimens that arrive at the Core will be placed in a blue cooler.

  - The specimen will be transported to the hospital via courier.

  - When the specimen arrives at the hospital, the blue cooler will NOT be opened in Processing. The blue cooler will be brought directly to the designated specimen area in Pathology.

  - A Pathology department staff member will open the cooler and retrieve the specimens. If it is after hours or there is no one available to retrieve the specimens from the cooler, leave the cooler in the designated specimen area in the Pathology department. Do not unpack the cooler.

- Hospital Process:

  - Pathology specimens that are at the hospital are brought to the processing department from the units.

  - Place the specimens in clear, blue-handled containers with covers except fresh specimens.

  - The units should notify the Lab if the specimen is a “fresh” specimen. A “fresh” specimen is not in formalin. They are not in the usual formalin transport cups. If a “fresh” specimen is received in the Lab, a Pathologist needs to be contacted immediately.  Do not leave in pathology.  A pathologist or pathologist assistant must be given the specimen.

  - The formalin preserved specimens in the containers should be left in the designated area in Pathology and not unpacked on any shifts by the processing staff.

  - Pathology will open the containers during normal working hours.

TITLE: Waste Containers

POLICY STATEMENT:  To ensure waste containers are located in their designated areas to avoid the risk of specimens being discarded inadvertently.

PROCESS:

- All biohazard and waste containers have a designated location in each area of specimen processing in the laboratory.   The location must be away from the specimen receiving and accessioning bench.  All containers should be kept away from the edge of the work bench.

- Each shift will be responsible to ensure the containers are in their designated location.  The location is designated with a trash barrel symbol on the floor and corresponding number, for example 1-1, 2-2, etc.

- A person assigned to the task in each area is responsible for checking the location of the containers and initialing a log sheet during operational hours.

Processing Core -     1st and 2nd shift Lab Specimen Processor

Processing Hospital- 1st, 2nd and 3rd shift Lab Administrative Associate                                                     Cytology Core           1st shift   Cytology Prep Associate

2nd shift  Cytology Lab Administrative Associate

Pathology                  1st shift   Pathology Accessioning

2nd shift  Grossing Technician

Any containers not in their designated area will be relocated and the supervisor, staff working in that area, and housekeeping will be informed. This will be noted in the corrective action section of the waste container log sheet.

- The supervisor or delegate of the area will review the log.

| Trash Location Log to Ensure All Waste Containers Are in Assigned Areas |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Location/Dept |  |  |  |  |  |  |  |
| Month/Year |  |  |  |  |  |  |  |
| # biohazard containers |  | Locations |  |  |  |  |  |
| #Waste containers |  | Locations |  |  |  |  |  |
| Biohazard and Waste Containers |  | Assigned Area | initials 1st shift | Assigned Area | initials 2nd shift | Assigned Area | Initials 3rd shift |
| Day | 1 |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |
|  | 4 |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |
|  | 7 |  |  |  |  |  |  |
|  | 8 |  |  |  |  |  |  |
|  | 9 |  |  |  |  |  |  |
|  | 10 |  |  |  |  |  |  |
|  | 11 |  |  |  |  |  |  |
|  | 12 |  |  |  |  |  |  |
|  | 13 |  |  |  |  |  |  |
|  | 14 |  |  |  |  |  |  |
|  | 15 |  |  |  |  |  |  |
|  | 16 |  |  |  |  |  |  |
|  | 17 |  |  |  |  |  |  |
|  | 18 |  |  |  |  |  |  |
|  | 19 |  |  |  |  |  |  |
|  | 20 |  |  |  |  |  |  |
|  | 21 |  |  |  |  |  |  |
|  | 22 |  |  |  |  |  |  |
|  | 23 |  |  |  |  |  |  |
|  | 24 |  |  |  |  |  |  |
|  | 25 |  |  |  |  |  |  |
|  | 26 |  |  |  |  |  |  |
|  | 27 |  |  |  |  |  |  |
|  | 28 |  |  |  |  |  |  |
|  | 29 |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |
|  | 31 |  |  |  |  |  |  |