New Vision Medical Laboratories St. Rita’s Medical Center           Lima, Ohio                          Technical Procedure

Procedure Number: nvml.chem.t.348.0

Policy Initiated: 02.25.14

This procedure is electronically produced, revised, monitored, reviewed and/or retired. For a full document history of initiation, revisions, and reviews with electronic signatures and dates, please open document in the procedure publishing software.

SUBJECT:  Lipase

PROCEDURE: (Addendum to the above pages)

- Rejection Criteria:

- Specimens that are improperly labeled or of the incorrect specimen type must be rejected.

- Quality Control:

- Controls must be run on each day that this test is requested and are good for 24 hours unless trouble is encountered that requires another evaluation. See posted Quality Control information sheet for levels of control to be run. Follow guidelines for Quality Control using the 2 SD ranges, which are posted and available in the LIS.

- Quality Control Failure:

1. Unacceptable QC results should be entered in to the computer along with appropriate actions taken.

2. Various corrective actions include:

a. Check reagent expiration dates and check for proper storage conditions.

b. Rerun QC (use same reagent).

c. Calibrate and run freshest control material

d. Open new reagent and run qc

e. Call hotline for assistance and document actions taken

f. Inform a Technical Specialist

- Calculations:

- N/A

- Interpretation:

- No interpretation is required.

- Normal Ranges:

- 5.6-51.3 U/L

- Analytical Measurement Range:

- 3 – 300 U/L

- Clinical Reportable Range:

- 3 – 3000 U/L

- Critical Values:

- None Stated

- Reporting Results:

- Results may be entered directly into the LIS.

- Backup for Inoperable System:

- Use backup analyzer.

- Referral of Specimens:

- Use backup analyzer.

- Submission and Handling of Referral Specimens:

- See requirements of reference lab.

- Method Validation:

- This test was validated by means of startup studies (on file).  Subsequent validation is by virtue of QC and CAP surveys.

Approved By:

Ashley Neidert, MLS

Technical Specialist

Date Approved: 04/05/19

Dr. Amber Patton, MD

Chemistry Medical Director

Date Approved: 04/03/19

Dr. Matt Kuhn, MD

Medical Director

Date Approved: 04/03/19