#### Blood Product Entry

| ##### Purpose | The Blood Product Entry function is used to enter a variety of blood and manufactured products into computer inventory. This function allows for the assignment of directed or Autologous products to intended recipients, and for entry of antigen/antibody/attributes to blood products. Some modifications to previously entered blood products are also made using this function. |
| --- | --- |

| ##### Policy | - This procedure is to be used for product entry and modifications that are allowed by system. - Units that are received from supplier with special antigen typing will have these types entered at time of product entry. - Unit attributes MUST be entered manually for Sickle Cell Neg (BBSCN), or Platelet Crossmatched units (BBPLXM). CMV Neg attribute is entered by scanning the corresponding bar code. Irradiation ,Leuko-reduction and HLA attributes default as part of the blood component code. - All Autologous and Designated donor units are assigned to patient using Medical Record Number at time of entry into the computer and will be entered by CLS and MLT only. “Duplicate RBC” units may only be entered by CLS. All other products may be entered by Sr. LA’s or MLT who have been trained for Transfusion Service. - Incorrect unit number(s) that have been accepted into the system MUST be corrected by changing status to WN in Blood Status Update function. The correct unit number is then re-entered into the system. - Incorrect component code(s) that have been accepted into the system, will be corrected in Blood Component Preparation using site specific prep code FIX - The component code on products received as Pediatric aliquots must be hand entered rather than scanned in to ensure correct billing. |
| --- | --- |

| ##### Procedure: Blood Product Entry | All products other than Rhogam are entered under this tab |
| --- | --- |

| Step | Action |
| --- | --- |
| 1. | Using the mouse, double click on the Blood Product Entry folder in the Misys Gateway. |
| 2. | The HID/Shift /Tech from the sign on are defaulted automatically. - If any of these needs to be modified, click the Modify key in the lower left and insert changes as needed. A second tech ID will be added to the one logged onto the session if this key is used. |

Continued on next page

| 3. | Follow instructions below to enter product into LIS inventory |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | If: | Then: |  |
|  |  | Product number is ISBT 128 format | - Scan in unit number - Scan in unit blood type, expiration date and component (No particular order is required, correct fields will populate) - Do not scan component barcode for Medicare autologous donor units or pediatric aliquot products. Type in appropriate code from list in step 4 below or consult product list in Computer Manual - Note: If component does not default in after scanning, look it up on the product list in the Computer Manual and hand enter mnemonic on the component line - Type in volume for FFP products or use volume bar code list to scan in appropriate volume(200 for single and 400 for Jumbo, use volume on bag for pediatric products) - Scan CMV Neg bar code onto Special test line on ISBT field tab. (Not required for frozen products) - Click Add |  |
| 4. | - If a bar code does not scan, hand-enter information from list below followed by a tab. - Note: Refer to Product Lists in Manual for component mnemonic not in list below. - If:; - Then:. - Medicare Autologous PC; - APM. - Medicare Autologus WB; - AWBM. - Pediatric FFP Division 1; - FD1. - Pediatric FFP Division 2; - FD2. - Pediatric FFP Division 3; - FD3. - Pediatric PPIL Division 1; - PPILA. - Pediatric PPIL Division 2; - PPILB. - Pediatric PPIL Division 3; - PPILC. - Pediatric PPIL2 Division 1; - PPIL2A. - Pediatric PPIL2 Division 2; - PPIL2B. - Pediatric PPIL2 Division 3; - PPIL2C. - Pediatric PPIL3 Division 1; - PPIL3A. - Pediatric PPIL3 Division 2; - PPIL3B. - Pediatric PPIL3 Division 3; - PPIL3C. |
| 5. | A warning message “Duplicate unit. A unit with the same unit #, component type and division# already exists in the database. Use modify unit if you wish to modify the existing unit.” dialog box appears? |  |  |  |
|  |  | If: | Then: |  |
|  |  | No | Skip to step 7. |  |
|  |  | Yes | - Click OK - Scan the component bar code |  |
| 6. | Message from above reappears? |  |  |  |
|  |  | If: | Then: |  |
|  |  | No | - Rescan unit number - Continue with next step. |  |
|  |  | Yes | - Set unit aside - Enter all other units - Proceed to step 15 |  |
| 7. | Received date & time defaults to current date & time. Modify time and date to match receiving invoice. |  |  |  |
| 8. | Use drop down menu in Visual Inspection field and select appropriate response to document Visual. |  |  |  |
| 9. | All yellow fields are mandatory and MUST be populated. Complete fields in right hand column as appropriate. - Sun on tab changes to yellow when information is saved on these tabs. |  |  |  |
| 10. | Ag/Ab/Attributes tab: - Ag/Ab: Type the ETC code for antigen into the Ag/Ab field. The codes can be found on a list in the front of the BB Computer Manual. Click Add between each ETC code. Continue clicking Ag/Ab field until all antigens have been added. Selected items move to larger box on right side of tab. Note: UCMVN code is entered for CMV Neg units or USCN for Sickle Cell negative at this prompt (Not required for frozen products) - Attributes: Some attributes will default in according to component type .(Ie. Irradiated, Leukoreduced, HLA matched) or have bar code on unit, (CMV Neg) Add the following ETC codes manually: Washed (BBWASH), Sickle Cell Neg (BBSCN) or Platelet Crossmatched (BBPLXM). Add to list. |  |  |  |
| 11. | Comments tab: - Add ETC comment codes or type in free text as needed. (ie. Platelets cross matched for “patient name”) - Click Add. |  |  |  |

Continued on next page

| 12. | Assignees tab: - Only accessible when an autologous or designated donor component type is entered. - Enter Medical Record number of assignee on Patient ID line, hit tab key. Patient Name will default in. - Click on ID information field. Enter date of Surgery, Dr. and Exp. Date of unit. |  |  |  |
| --- | --- | --- | --- | --- |
| 13. | Location Tab: |  |  |  |
|  |  | If: | Then: |  |
|  |  | Units to be stored in mobile refrigerator | - Select Location - Select either CT-1 or HM (Helmer mobile) |  |
|  |  | Autologous or designated donor unit(s) | - At location, select HS2 (Helmer Single Door #2) |  |
|  |  | Units other than Autologous/Designated or those to be stored in mobile | - No Location Selection necessary Note: If no location is selected, location defaults to RV (RV is primary storage location for product. RBC=Helmer Double Door Frozen products=Jewett Freezer, Platelets=Helmer Platelet Incubator) |  |
| 14. | After all necessary information is entered, click the Add button above the Summary List area. - To modify information on a unit from the summary list, double click on the unit line to display associated data and reply Yes to Replace information question. - To sort units on the list, use up & down arrows at the side of the list to move highlighted unit. |  |  |  |
| 15. | After all units to be entered have been added to the Summary List, compare scanned information against shipping invoice. Does the description for any of RBC products contain Bag or part numbers? If:; Then:. No; Continue to next step. Yes; - Attach note to RBC units indicating the Bag or Part number - Continue to next step. |

Continued on next page

| 16. | Are there any errors or omissions between screen and shipping invoice? |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | If: | Then: |  |
|  |  | No | Click Save. |  |
|  |  | Yes | Correct errors or omissions. Refer to step 14 for instructions. Click Save |  |
|  | Note: It is not necessary to record the work list number that is automatically generated for products that require confirmation testing. |  |  |  |
| 17. | Unit has been in inventory at another Sutter hospital. Exit Blood Product Entry. |  |  |  |
|  |  | If: | Then: |  |
|  |  | SLA | - Make notation on shipping invoice - Place note on unit identifying it as a “Duplicate unit” - Place unit on “Inventory to be entered” shelf of Helmer refrigerator - Alert tech assigned to Transfusion Service that there is a “duplicate unit” that needs to be entered |  |
|  |  | CLS | - Continue with step 18. |  |
| 18. | Open function Blood Status Update. |  |  |  |
| 19. | Select Update Option: “Ship out to inventory” from drop down list. |  |  |  |
| 20. | Scan unit number and component bar code. |  |  |  |
| 21. | Complete required fields using the date and time received from Shipping Invoice and INV as New Status. |  |  |  |
| 22. | Choose the “Location” button at the bottom of the screen to select the desired HID and storage location. - For primary default location, click RV folder - Be sure to click OK on pop up box. Refer to Changing Blood Product Storage Location. |  |  |  |
| 23. | Click Save. |  |  |  |
| 24. | Is this a product that requires type confirmation? |  |  |  |
|  |  | If: | Then: |  |
|  |  | No | Proceed to next step |  |
|  |  | Yes | - Open function Blood Product Testing - Check to see status of unit confirmation is pending. - If unit confirmation is completed, order Uchek, Uchek2, Uchek3, Uchek4, as applicable. |  |

Continued on next page

| 25. | Are antigen types present that were not at performed at RV or Blood Source or CMV Neg attribute/antigen is missing? |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | If | Then: |  |
|  |  | No | Place unit on “To be confirmed shelf” in Helmer double door refrigerator. |  |
|  |  | Yes | - Call up unit in Blood Product Entry. - Open up Modify Unit tab - Scan in unit number - Remove antigen types that were not performed at RV or Blood Source from antigen field or add attribute and CMV Neg antigen code, as applicable. - Save edits. - Place unit on “To be confirmed shelf” in Helmer double door refrigerator. |  |

| ##### Procedure: Modifying Data for a Unit in Inventory |  |
| --- | --- |

Note:   The unit status, the component type, the supplier, and the supplier unit number cannot be changed in the Blood Product Entry function.

| Step | Action |
| --- | --- |
| 1 | Click Modify Unit button in lower left of screen. |
| 2 | Scan or type unit number. (If the Blood Supplier box, pops up, click cancel to proceed.) Select component code from drop down list if, component code does not default in. |
| 3 | Make necessary changes. |
| 4 | Click Save. |

| ##### Procedure: Manufactured Product Entry |  |
| --- | --- |

| Step | Action |
| --- | --- |
| 1. | Select the Manufactured Product tab on the Blood Product Entry screen. |
| 2. | Type in Mfr ID/supplier code ORTHO2 followed by tab key, or use lookup. |
| 3. | Type in component type code RHG followed by tab key, or use lookup. |
| 4. | Type in RV followed by the Manufacturer’s lot # . - If “Duplicate unit. A unit with the same unit #, component type and division# already exists in the database. Use modify unit if you wish to modify the existing unit” appears, use instructions for Modifying Manufactured Product in Inventory (below). |
| 5. | Type in the expiration date at that prompt. Press TAB. Expiration time defaults in, but may be modified. |
| 6. | The receive date and time defaults in. Modify as needed. |
| 7. | Click Save. - A message displays with the unit number and the range of derivatives in the format lot number/sequence number. The number of derivatives is based on the packaged quantity. - Barcoded labels print for each unit, as defined by maintenance. |

| ##### Procedure: Modifying Data for a Manufactured Product in Inventory |  |
| --- | --- |

| Step | Action |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | On the Manufactured Product tab, press the “Modify Unit” key in the lower left of the screen. |  |  |  |
| 2. | Type unit number in the dialog box. - If modifying entire lot, enter lot number without slash number. If modifying a single derivative include the slash number (example: RGF192A1/4). |  |  |  |
| 3. | Select component type code, if necessary. |  |  |  |
| 4. | Click OK. |  |  |  |
| 5. | Modifying entire |  |  |  |
|  |  | If: | Then |  |
|  |  | Adding a new shipment of an existing lot number | - Select <New Shipment> - Enter expiration date & time, packaged quantity and receive date & time. |  |
|  |  | Modifying data on all derivatives of current lot | - Select<Lot Update> - Enter IU (International Units) |  |
| 6. | Enter data on Comments or Location, as needed. |  |  |  |
| 7. | Click Save. |  |  |  |

| ##### Related Documents | - Performing a Blood Status Update - Changing Blood Product Storage Location |
| --- | --- |