#### Performing a Blood Status Update

| ##### Purpose | The purpose of this procedure is to provide instructions on how to update the Blood Product status in the Sunquest Laboratory Information System, GUI system. |
| --- | --- |

| ##### Policy | - Status changes are to be recorded directly into the computer system whenever it is operational. - When the computer system is non operational, the status will be recorded using the downtime procedure and information transferred into the system as soon as possible. - Status Update is to be performed by Transfusion Service trained MLT or CLS only. |
| --- | --- |

| ##### Procedure: Unit Update |  |
| --- | --- |

| Step | Action |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | Log onto Sunquest Gateway. |  |  |  |
| 2. | Open up Blood Status Update folder. |  |  |  |
| 3. | In the Update Option box, select Unit Update. |  |  |  |
| 4. | On the Update Process tab, click on the Unit # box, |  |  |  |
| 5. | Scan a unit bar code number or type in unit number. |  |  |  |
|  |  | If: | Then: |  |
|  |  | Unit number scanned in and Blood Supplier box pops up | Click cancel to close the pop up window |  |
|  |  | Unit number hand entered | Press Tab to advance to next field |  |
| 6. |  |  |  |  |
|  |  | If: | Then: |  |
|  |  | Only one product with that unit number in inventory | - Component will automatically populate field |  |
|  |  | More than one unit with that number is in system | - Scan component bar code - If bar code is not recognized, use drop down menu to select correct component |  |
| 7. | The following information for the selected unit is available under each tab if tab is not grayed out: |  |  |  |
|  |  | If: | Then: |  |
|  |  | Assignees, on the Assignee Data tab | Click the tab to view assignee information |  |
|  |  | Unit activity (such as allocation or issue) | Click the tab to view unit activity. |  |
| 8. | Press the TAB key to move to the Date box. |  |  |  |
| 9. | TAB through date & time fields to default current date & time - If recording after the fact, enter the correct date and time update occurred. |  |  |  |
| 10. | Enter a new status in the New status box. Use the drop down box to select the appropriate status for this unit. |  |  |  |
| 11. | Enter the return temperature or appropriate ETC code in the Temperature field. |  |  |  |
| 12. | Press the TAB key twice. |  |  |  |
| 13. | Complete Reason/Comment entry as needed: - A reason code is required, if field is highlighted in yellow. Select one from the search field by typing in RE and clicking the Code search - Comments can be entered using an ETC or free text. |  |  |  |
| 14. | Click on Unit Location. Highlight the appropriate unit location, then click OK. |  |  |  |
| 15. | Click Save |  |  |  |
| 16. | Unit Activity tab opens if an allocated unit is being returned to inventory. Reallocate or release this unit from the accession number. |  |  |  |
| 17. | Save |  |  |  |

| ##### Procedure: Shipping a Unit out of Inventory |  |
| --- | --- |

| Step | Action |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | Log onto Sunquest Gateway. |  |  |  |
| 2. | Open up Blood Status Update folder. |  |  |  |
| 3. | In the Update Option box, select Ship Out. |  |  |  |
| 4. | On the Update Process tab, click on the Unit # box, |  |  |  |
| 5. | Scan a unit bar code number or type in unit number. |  |  |  |
|  |  | If: | Then: |  |
|  |  | Unit number scanned in and Blood Supplier box pops up | Click cancel to close the pop up window |  |
|  |  | Unit number hand entered | Press Tab to advance to next field |  |

Continued on next page

| 6. |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | If: | Then: |  |
|  |  | Only one product with that unit number in inventory | - Component will automatically populate field |  |
|  |  | More than one unit with that number is in system | Use drop down menu to select correct component |  |
| 7. | The following information for the selected unit is available under each tab if not grayed out: |  |  |  |
|  |  | If: | Then: |  |
|  |  | Assignees, on the Assignee Data tab | Click the tab to view assignee information |  |
|  |  | Unit activity (such as allocation or issue) | Click the tab to view unit activity. |  |
| 8. | Press TAB to move unit onto the batch Update list. |  |  |  |
| 9. | Repeat steps 4-8 units that are to be shipped out at same date & time are on the batch Update list. |  |  |  |
| 10. | Click Submit box. |  |  |  |
| 11. | TAB through date & time fields to default current date & time - If recording after the fact, enter the correct date and time update occurred. |  |  |  |
| 12. | Enter in Destination - Use drop down list to select destination, if pneumonic is not known. |  |  |  |
|  |  | If: | Then: |  |
|  |  | Blood Source has requested that you do a direct transfer to another hospital | - Enter the Destination code for the facility BSS has requested it to be sent or select from drop down menu - Add ETC “DTR”(Re: Direct Transfer per Blood Supplier) on the comment line. |  |
| 13. | Press TAB. |  |  |  |

Continued on next page

| 14. | Click Yes or No for pass visual inspection for all units. |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | If: | Then: |  |
|  |  | Yes | - Fill in ETC for Reason code - Click Add - Enter all applicable codes - Click Continue - Skip to step 16 |  |
|  |  | No | - Proceed to next step |  |
| 15. | Does the unit number begin with W0358? |  |  |  |
|  |  | If: | Then: |  |
|  |  | Yes | - Choose “Fail” from Visual Inspection from drop down menu - Click OK to pop up box - Enter BSS as the destination - Enter ETC in Reason Code box - Click Continue |  |
|  |  | No | - Click Clear and follow steps in Unit Update procedure above. - Use RTD as the New Status of unit. - Return unit to Blood Source |  |
| 16. | Click on Unit Location. Highlight the appropriate unit location, then click OK. |  |  |  |
| 17. | Click Save |  |  |  |