Returning Blood Issued in a Hand Held Biohazard Cooler

| ##### Purpose: | This procedure describes how determine whether blood issued in a cooler is acceptable for reissue. |
| --- | --- |

| ##### Policy: | - Units that have been taken into an “isolation” room are ineligible for return regardless of whether or not they have been stored in a Cooler. - The acceptability for reissue of RBC units issued in a Biohazard Cooler is governed by the attached Safe-T-Vue indicator and general rules of acceptability for reissue. - If a unit is returned in a Cooler without a Safe-T-Vue attached, refer to Returning of Issued Components to determine suitability of product for reissue. - Only Transfusion Service trained MLT/CLS may determine whether or not a unit is acceptable for reissue - Units that do not meet the reissue criteria will have their final status updated to DS in the LIS and be discarded in the biohazard trash. - A PSR must be filled out on all units that have a final disposition of DS. |
| --- | --- |

| ##### Procedure: | Follow the steps in the table to perform this procedure: |
| --- | --- |

| Step | Action |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | Blood is returned in a hand held biohazard cooler. |  |  |  |  |
| 2. | Erase cooler number from the grease board located in the back of the Transfusion Service. |  |  |  |  |
| 3. | Remove unit(s) from cooler. |  |  |  |  |
| 4. | Write date & time of return at bottom of Transfusion Record. |  |  |  |  |
| 5. | Examine the bag, checking that the unit passes visual inspection, there is at least 1 useable segment still attached to the bag and that the hermetic seals are intact. |  |  |  |  |
| 6. | Turn unit over and evaluate the Safe-T-Vue sticker according to the instructions contained in “Activating and Interpreting the Safe-T-Vue Temperature Indicator” procedure. |  |  |  |  |
| 7. |  |  |  |  |  |
|  |  | If: | Then: |  |  |
|  |  | Hermetic seals are in tact, the unit passes visual inspection and the interpretation of the sticker indicates acceptability | Unit may be returned to inventory for reissue |  |  |
|  |  | The hermetic seals have been broken or the unit fails visual inspection or the Safe-T-Vue sticker indicates that the units are not acceptable for reissue | Unit cannot be reissued or returned to inventory |  |  |

| 8. |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | If: | Then: |  |
|  |  | The unit(s) are acceptable for reissue | - Update the status of the unit to inventory in the LIS. Add ETC RETINP (Temperature Indicator Pass) in the Temperature field in BSU - Locate & discard the lower portion of the cardboard copy of the transfusion record in the Issued bin - Remove the unit tag from the unit. - Discard unit tag, Transfusion Record, if applicable, and cardboard copies of the Transfusion Record form in Shred-it trash - Return the unit(s) to the blood refrigerator |  |
|  |  | The unit(s) are not acceptable for reissue | - Update the status of the unit to DS the LIS. Add ETC RETINF (Temperature Indicator Fail) in the Temperature field in BSU - Locate & discard the lower portion of the cardboard copy of the transfusion record in the Issued bin - Remove the unit tag and Transfusion Record, if applicable, from the unit. - Discard unit tag, Transfusion Record, if applicable, and cardboard copies of the Transfusion Record form in Shred-it trash - Discard the unit in the biohazard trash - Write PSR. |  |
|  |  | If unsure | - Locate & discard the lower portion of the cardboard copy of the transfusion record in the Issued bin - Leave Safe-T-Vue sticker attached to unit - Record the date/time of return and brief description of the problem on the unit tag. - Mark unit with a Quarantine sticker and place it on the Quarantine shelf of Helmer double door refrigerator - Make an entry on the Communication log and request a second opinion of the incoming CLS. |  |
| 9. |  |  |  |  |
|  |  | If: | Then: |  |
|  |  | Water is pooled in bottom of ice bag | - Discard melted ice in sink - Clean and dry interior of cooler - Return cooler to storage area |  |
|  |  | Ice is OK | Return cooler with ice to cooler staging area |  |

| ##### Technical Notes | - After application to the unit, the Safe-T-Vue indicator is good for up to 42 days - Refrigerated stock of unused Safe-T-Vue indicators must be reactivated every 2 months by bringing box to RT and then returning box to the refrigerator. |
| --- | --- |

| ##### Related Documents | Returning of Issued Components |
| --- | --- |

| ##### Attachment | - Example of Safe-T-Vue Indicator for acceptable and unacceptable unit |
| --- | --- |

| ##### Reference | Safe-T-Vue Manufacturer’s Insert |
| --- | --- |