#### Labeling a Blood Component for Transfusion

| ##### Purpose | The purpose of this procedure is to provide instructions for printing an allocation label form in Sunquest and labeling blood components for transfusion. |
| --- | --- |

| ##### Policy | - Blood Bank SLA/MLT/CLS may attach blank Uncrossmatched Universal Donor tags to “O” units. Refer to Preparing Universal Donor “Trauma” Packs procedure - Only Clinical Lab Scientists (CLS) may attach allocation labels containing patient or unit identification to red cell components. - CLS or MLT may attach allocation labels containing patient or unit identification to non-red cell components - The allocation label must be initialed and applied by the MLT/CLS responsible for checking that the unit information on the allocation label corresponds to the unit the label is attached to. |
| --- | --- |

| ##### Equipment/ ##### Reagent/ ##### Supplies |  |
| --- | --- |

| Equipment | Reagents | Supplies |
| --- | --- | --- |
| - Tach-it gun - Unit tag printer |  | - Allocation label form - Manila tag - Tach-it barbs - Blood Component |

| ##### Procedure: | The allocation label form prints automatically on the assigned printer when the transfusion status “OK to transfuse” is approved in the LIS. |
| --- | --- |

| Step | Action |
| --- | --- |
| 1. | Remove the computer generated allocation label form from the printer. |
| 2. | Remove components from appropriate storage area. |
| 3. | Verify that the BB armband number on the allocation label matches the BB armband paperwork for products containing RBCs. |

Continued on next page

| 4. | Review the patient and testing information printed on the allocation label form for completion and accuracy. Note: The patient’s full name is required. If the allocation label has truncated the name, handwrite the missing information and initial the change. |  |  |  |
| --- | --- | --- | --- | --- |
| 5. |  |  |  |  |
|  |  | If: | Then: |  |
|  |  | Complete and accurate | Proceed to step 6. |  |
|  |  | Incomplete or incorrect | Complete or correct as appropriate. Reprint allocation label form. |  |
| 6. | Initial the allocation label in the blank space next to the Blood Bank Armband number. |  |  |  |
| 7. | Select the unit to be tagged. Compare the unit information on allocation label to the bag label. - Unit number - Component type and division number - Blood type - Expiration date - Special transfusion requirements, if applicable (ie. Irradiated, CMV Neg, antigen negative) |  |  |  |
|  |  | If: | Then: |  |
|  |  | Information on allocation label matches bag label | Peel allocation label off form and adhere to exposed plastic on front of unit, above or below the bag label. DO NOT apply over any of the existing bag label. |  |
|  |  | Unit number on allocation label does not match bag label | Locate correct unit and apply alloction label to unit |  |
|  |  | Expiration date, blood type, component or special requirements on allocation label does not match bag label. | Do not label component for transfusion. Troubleshoot problem, fill out QAR as appropriate. |  |
|  | If there is no unobstructed space for allocation label on front of bag, apply allocation label to manila tag and attach tag to unit using Tach-it barb. |  |  |  |

Continued on next page

| 8. | Fold remaining form into quarters and place it with unit into a unit holder. |  |  |  |
| --- | --- | --- | --- | --- |
| 9. | Repeat steps 4-9 until all units have been tagged. |  |  |  |
| 10. | Use table below to select appropriate storage device. Store labeled product in storage device until picked up for transfusion of released from allocation. |  |  |  |
|  |  | Product | Storage |  |
|  |  | - RBC | - 1-6C, Allogeneic in Helmer Double door Refrigerator - Autologous and Designated in Helmer Single door Refrigerator. - Emergent units for MT Protocol support in CT-1 or Helmer mobile Refrigerator |  |
|  |  | - Thawed FFP - Thawed Plasma - Thawed Cryo Reduced Plasma | 1-6C, Helmer Double door Refrigerator |  |
|  |  | - Cryo, Cryo Pool | RT, counter |  |
|  |  | - Platelets | 20-24C, Platelet Incubator with continuous agitation |  |

| ##### References: | Sunquest User Manual: Using the Blood Bank System |
| --- | --- |

| ##### Attachments | A: Sample of Allocation Label form |
| --- | --- |