Issuing Blood for Storage in the Infusion Center Blood Bank Refrigerator

| ##### Purpose: | The purpose of this procedure is to provide instructions on the issue of blood products for storage in the Infusion Center Blood Bank Refrigerator. |
| --- | --- |

| ##### Policy: | - This procedure applies only to refrigerated products transferred to IVC Cancer Center for administration. - Platelet products will be sent in a Blood Source platelet box on the next scheduled courier delivery or may be picked up in between courier runs by IVC staff, once patient has arrived. - Issuing blood for multiple patients at the same time is only permitted when the units are to be transferred for storage to a remote blood bank storage refrigerator. - The Outpatient Infusion Center/Cancer Center is the only area that currently has a remote blood bank storage refrigerator. - Units for remote refrigerated storage may only be transported in the IVC Transport Cooler. - A completed SRMC Cancer Center (Building 8) Blood Request Form identifying each patient, the number of units to be issued and that the units are intended for the Cancer Center must be received by the Transfusion Service before the Cooler may be packed. - Downtime request will be faxed at close of day (1800-1830) for products to be provided the following day. |
| --- | --- |

| ##### Procedure: | The steps outlined in Issuing Blood Components for Transfusion will be followed except that a second person is not required to verify when packing the IVC Transport Cooler. |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Step | Action |  |
|  |  | 1. | PM shift will retrieve Cancer Center faxed requests for the following day from the Fax machine and place next day SRMC Cancer Center (Building 8) Blood Request Form on the clip dedicated for IVC requests. |  |
|  |  | 2. | The Day shift SLA will check for Cancer Center requests at the beginning of their shift and ensure that requested units are packed and ready for 1st scheduled courier run. |  |
|  |  | 3. | Double bag 6 scoops of ice. |  |
|  |  | 4. | Place ice into large Infusion Center cooler. |  |
|  |  | 5. | Using the Sutter Roseville IVC Transport Cooler Log, call up units one patient at a time in Blood Product Issue |  |
|  |  | 6. | Locate and remove requested units from the blood bank storage refrigerator. |  |
|  |  | 7. | Scan unit number from bag to select each unit to be issued. |  |
|  |  | 8. | Verify that patient information on blood request, computer screen and allocation label match on each unit. |  |
|  |  | 9. | Verify that the unit information from computer, allocation label and bag label match each unit. |  |
|  |  | 10. | Verify that unit(s) meets patient history requirements. |  |
|  |  | 11. | Perform visual inspection of each unit. If:; Then:. Units passes visual inspection; - Click Pass. Unit(s) fail visual inspection; - Click Fail - Quarantine and investigate source of failure.. |  |
|  |  | 12. | Tab through date and time. Complete Issue to field with “IVC Transport”. |  |
|  |  | 13. | Complete the Issued and Inspected by section of the Transfusion Record/Transfusion Reaction form with your initials, date and time. |  |
|  |  | 13. | Remove sticker from back of each unit and place on the |  |
|  |  | 14. | Save the remaining Allocation Label form (AKA Transfusion Record/Transfusion Reaction form) in the unit reconciliation tray |  |
|  |  | 15. | Remove ice bag from container. |  |
|  |  | 16. | Place issued units at the bottom of container. |  |
|  |  | 17. | Place bag of ice on top of the units making sure that the unit tag provides a barrier between the ice bag and the blood component. |  |
|  |  | 18. | Close lid of container. |  |
|  |  | 19. | Repeat steps 5-18 until all units have been issued. Note: If there are units requested that are not available/ready, pack the remaining units for transport and contact the Cancer Center charge RN for follow up after 0700. |  |
|  |  | 20. | After all requested units have been loaded into the Transport Cooler, secure the lid opening with a piece of packing tape. |  |
|  |  | 21. | Save the SRMC Cancer Center (Building 8) Blood Request Form in the appropriate tray. |  |
|  |  | 22. | Complete IVC Transport log and mark which bar code is attached to the transport device. |  |
|  |  | 23. | Unless notified that the IVC staff will come to pick up Transport Cooler, it will be prepared to be ready for the next scheduled courier run. |  |

Continued on next page

| 23. | The courier will check in with Transfusion Service tech/SLA at each assigned run. If:; Then:. No products needing transport; Notify courier upon arrival that there is no delivery. Products needing transport; Request that the courier time and sign log when transport container is picked up.. Pre-transfusion samples are being dropped off from IVC; Have BB SLA process samples for testing or deliver to IP processor for processing. Transport Cooler is returned at end of day; Open cooler Remove and discard remaining ice bag Process returned products File paperwork in the appropriate places.. |
| --- | --- |

| ##### Related Documents | - Infusion Center Remote Blood Storage - Issuing Blood Components for Transfusion |  |
| --- | --- | --- |
| ##### Attachment | - Sutter Roseville IVC Transport Cooler Log - SRMC Cancer Center (Building 8) Blood Request Form |  |