#### Resolving Patient History Discrepancies

| ##### Purpose | This procedure describes how to resolve discrepancies between patient historical testing and current testing. |
| --- | --- |

| ##### Policy | This procedure must be followed each time a history discrepancy is discovered. The Transfusion Service Coordinator/Supervisor will review Investigation of Patient History Discrepancy Forms and include occurrences in the Monthly QA Summary report. |
| --- | --- |

| ##### Procedure | Follow the table below when the current blood type does not match the historical blood type: |
| --- | --- |

| If ABO discrepancy | Then |
| --- | --- |
| Current ABO type does not match the historical ABO type | - Repeat the test using a new cell suspension. - Recheck clerical records, comparing spelling of name and date of birth to make sure that the current and historical patient is the same person. - If not resolved, initiate an Investigation of Patient History Discrepancy Form. - Check the reagent expiration date and QC records to make sure that the reagent is working properly. - Request the specimen be recollected, paying close attention to patient identification. Verbally verify this information with the patient at the time of recollection, if possible. |
| Unresolved ABO discrepancy after recollection. | - Have second technologist repeat typing - Check with care provider for possible transfusion of incompatible products or bone marrow transplant. - Notify pathologist immediately if patient has been transfused with incompatible products. - If the transfusion is urgent, give type O red blood cells. - Route completed Investigation of Patient History Discrepancy Form to Transfusion Service Coordinator/Supervisor |
| Determined an ABO error | - Notify pathologist immediately if patient has been transfused with incompatible products. Hold all transfusions for pathologist instructions. - Once the accuracy of the current testing has been established, type specific or compatible components may be issued. - Amend the Blood Type in the history file as needed, prior to issuing blood. Make entry in BAD Edit Log. - Add a PB comment and answer it with a dated Free text comment documenting that the Blood Type was changed. - Route completed Investigation of Patient History Discrepancy Form to Transfusion Service Coordinator/Supervisor - Transfusion Service Coordinator will review investigation and take additional action as needed. - The completed Investigation of Patient History Discrepancy Form, supporting documentation of amended history files and reason for edit will be maintained in a file. |

| If Rh discrepancy | Then |
| --- | --- |
| Current Rh type does not match previous Rh Pos result without Weak D positive comment | - Perform manual Rh type using a new cell suspension. - If the current repeat test is Rh negative, perform Weak D test. - If Weak D Positive, result patient as Rh Pos. Add REDUP and ABO1 to patient history. - If not resolved, initiate an Investigation of History Discrepancy Form - Request the specimen be recollected paying close attention to patient identification. Verbally verify this information with patient at time of recollection, if possible. - Retest new sample. |
| Current Rh type does not match previous Rh Pos result with Weak D comment | - Perform manual Rh type using a new cell suspension. - If the current repeat test is Rh negative, perform Weak D test. - If Weak D Positive, result patient Rh as RHDU. Add ABO1 to patient history - Initiate an Investigation of History Discrepancy Form - Change Rh in BAD file to RHDU |
| Current Rh Pos type does not match previous RHDU result | - Perform manual Rh type using a new cell suspension. - If the current repeat test is Rh negative, perform Weak D test. - If Weak D Positive, report as Rh Pos - Initiate an Investigation of History Discrepancy Form - Change Rh in BAD file to Pos |
| Current Rh Type does not match previous Rh Neg result | - Perform manual Rh type using a new cell suspension. - If not resolved, initiate an Investigation of History Discrepancy Form - Request the specimen be recollected paying close attention to patient identification. Verbally verify this information with patient at time of recollection, if possible. |
| Rh History Discrepancy resolved after recollection | - Recollection matched patient history. - Result type. - Report mislabel sample. - Route completed Investigation of History Discrepancy Form to Transfusion Service Coordinator/Supervisor for resolution. - Transfusion Service Coordinator will review investigation and take additional action as needed. |
| Unresolved Rh history discrepancy after recollection. | - Recollection matched original collection but not history - Locate sample used to result history and retest including Weak D, if available. - Check to see if formulation of Anti D has changed between testing or if different manufacturers were used. - If difference in formulation is suspected to be the cause of discrepancy, record results of testing but do not report Rh interpretation. - Give Rh negative red cell products until the discrepancy is resolved. - Route completed Investigation of Patient History Discrepancy Form to Transfusion Service Coordinator/Supervisor for resolution. - Transfusion Service Coordinator will review investigation and take additional action as needed. - The completed Investigation of Patient History Discrepancy Form, supporting documentation of amended history files and reason for edit will be maintained in a file. |

| ##### Reporting Results | Document all information and resolutions in the LIS. Complete an Investigation of Patient History Discrepancy Form. |
| --- | --- |

| ##### Attachments | - ABO History Discrepancy Flowchart - Rh History Discrepancy Flowchart - Reagent Formulation, Method and Policy History of Rh Typing at SRMC |
| --- | --- |

| ##### Related Documents | Performing a Patient History Check |
| --- | --- |