## Washing Cells Manually

| ##### Purpose: | To provide instruction in washing red cells to remove the unbound proteins before testing or the addition of Anti-Human Globulin for direct and indirect antiglobulin tests or anti-human globulin phase of manual crossmatch. |
| --- | --- |

| ##### Equipment/Reagents/Supplies: | Equipment | Reagents | Supplies |
| --- | --- | --- | --- |
|  | - Centrifuge | - 0.9% Blood Bank saline | - Test tubes - Disposable pipets - Segment sampler - Test tube rack |

| ##### Specimen Requirement: | Specific to individual procedure(s) being performed. |
| --- | --- |

| ##### Procedure: | - Follow the steps below to wash red cells manually during testing. - Prior to each cycle of washing, resuspend the red cells |
| --- | --- |

| Step | Action |
| --- | --- |
| 1. | Using a saline squeeze bottle, fill each tube to be washed 3/4 full with isotonic blood bank saline. |
| 2. | Centrifuge the tube(s) for the calibrated wash time and speed. |
| 3. | Decant the supernatant fluid. |
| 4. | Repeat Steps 1-3 for a total of 3-4 cycles or number of times specified in SOP. |
| 5. | After last wash, decant the saline completely to a dry button. |
| 6. | Complete the test procedure as described in the particular SOP. |

| ##### Limitations: | - Loss or hemolysis of the red blood cells from improper washing may result in false results. - Failure to adequately wash can cause false-negative reactions due to neutralization of the AHG reagent by unbound globulins - False negative results may occur if the washing process is delayed or interrupted. |
| --- | --- |

_________________________________________________________________

| ##### Related Procedures: | - Performing Direct Antiglobulin Test by Tube Method - Performing a Low Ionic Strength (LISS) Additive Tube Method - Performing and Antibody Screen by PeG Tube Method - Performing a Saline AHG Antibody Screen and/or Panel - Antibody Identification Tube Procedures - Performing a Maternal Antibody Titration - Antigen Typing Procedure, Indirect - Performing an Antiglobulin Crossmatch by PeG Tube Method - Performing a LISS (AHG) Crossmatch - Performing an Antiglobulin Crossmatch by Saline Tube Method |
| --- | --- |