Reagent quality control shall be performed on each reagent daily and/or when new lot is opened.  LIS, PeG ,Panoscreen and Manual Capture will be QC’d on each day of use. Quality control will be run on new lot of antiserum and recorded on the log before it is used for patient testing. Maintenance will be performed daily on Manual Capture system.

| DATE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Anti-A monoclonal BIORAD | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| Anti-B monoclonal BIORAD | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| Anti-A,B monoclonal BIORAD | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| Anti-D monoclonal BIORAD | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| Monoclonal Control BIORAD | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| Reverse Cells IMMUCOR | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| AHG-Polyspecific GAMMA | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| AHG-IgG GAMMA | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| Polyspecific, IgG Check Cells IMMUCOR | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| Anti C3b,C3d GAMMA | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| Complement Check Cells GAMMA | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| Cor QC Cells | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| Cor QC Serum | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| ImmuAdd | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| 0.9% Buffered Blood Bank Saline | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| Panoscreen IMMUCOR | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| PeG GAMMA | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| Manual Capture Ready Screen (3) | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| Select R strips | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| Manual Capture R Control | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| Manual Capture LISS | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| Manual Capture Indicator Cells | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| Fetal Bleed Cells GAMMA –**Perform lot to lot QC when opening new lot | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| Fetal Bleed Anti D Serum GAMMA | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: | Lot#: Exp. Date: |
| TECHNOLOGIST |  |  |  |  |

Results of QC Testing

| DATE |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | corQC | Pan/MonCt |  | corQC | Pan/MonCt |  | corQC | Pan/MonCt |  | corQC | Pan/MonCt |  |
| ANTI-A + corQC CELL or Pano 1 celll |  |  |  |  |  |  |  |  |  |  |  |  |
| ANTI-B + corQC CELL or Pano1 cell |  |  |  |  |  |  |  |  |  |  |  |  |
| ANTI-AB + corQC CELL or Pano2 cell |  |  |  |  |  |  |  |  |  |  |  |  |
| ANTI-D + corQC CELL or Pano 3 cell |  |  |  |  |  |  |  |  |  |  |  |  |
| A1 CELL + corQC or Monoclonal Cntl |  |  |  |  |  |  |  |  |  |  |  |  |
| A2 CEL L+ corQC or Monoclonal Cntl |  |  |  |  |  |  |  |  |  |  |  |  |
| B CELLS + corQC or Monoclonal Cntl |  |  |  |  |  |  |  |  |  |  |  |  |
| MONOCLONAL CONTROL + corQC CELL |  |  |  |  |  |  |  |  |  |  |  |  |
| AHG-PS + CHECK CELLS |  |  |  |  |  |  |  |  |  |  |  |  |
| AHG-IgG + CHECK CELLS |  |  |  |  |  |  |  |  |  |  |  |  |
| ACCEPTABLE? |  |  |  |  |  |  |  |  |  |  |  |  |
| TECHNOLOGIST |  |  |  |  |  |  |  |  |  |  |  |  |
| Backup Reagent Rack | corQC | Pan/MonCt |  | corQC | Pan/MonCt |  | corQC | Pan/MonCt |  | corQC | Pan/MonCt |  |
| ANTI-A + corQC CELL or Pano 1 celll |  |  |  |  |  |  |  |  |  |  |  |  |
| ANTI-B + corQC CELL or Pano1 cell |  |  |  |  |  |  |  |  |  |  |  |  |
| ANTI-D + corQC CELL or Pano 3 cell |  |  |  |  |  |  |  |  |  |  |  |  |
| MONOCLONAL CONTROL + corQC CELL |  |  |  |  |  |  |  |  |  |  |  |  |
| ACCEPTABLE? |  |  |  |  |  |  |  |  |  |  |  |  |
| TECHNOLOGIST |  |  |  |  |  |  |  |  |  |  |  |  |
| Panoscreen with LISS/PeG | 37C IgG CC |  |  | 37C IgG CC |  |  | 37C IgG CC |  |  | 37C IgG CC |  |  |
| Cell 1 + corQC serum |  |  |  |  |  |  |  |  |  |  |  |  |
| Cell 2 + corQC serum |  |  |  |  |  |  |  |  |  |  |  |  |
| Cell 3 + corQC Serum |  |  |  |  |  |  |  |  |  |  |  |  |
| Cell 1 + Monoclonal Control |  |  |  |  |  |  |  |  |  |  |  |  |
| Cell 2 + Monoclonal Control |  |  |  |  |  |  |  |  |  |  |  |  |
| Cell 3 + Monoclonal Control |  |  |  |  |  |  |  |  |  |  |  |  |
| ACCEPTABLE? |  |  |  |  |  |  |  |  |  |  |  |  |
| TECHNOLOGIST |  |  |  |  |  |  |  |  |  |  |  |  |
| Performed Capture Daily Maintenance |  |  |  |  |  |  |  |  |  |  |  |  |
| Manual Capture Ready Screen (3) | Pos Cont | Neg Cont |  | Pos Cont | Neg Cont |  | Pos Cont | Neg Cont |  | Pos Cont | Neg Cont |  |
| Cell 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Cell 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Cell 3 |  |  |  |  |  |  |  |  |  |  |  |  |
| Pos Control |  |  |  |  |  |  |  |  |  |  |  |  |
| ACCEPTABLE? |  |  |  |  |  |  |  |  |  |  |  |  |
| TECHNOLOGIST |  |  |  |  |  |  |  |  |  |  |  |  |