#### Thawing Fecal Microbiota Transplant

| ##### Purpose | The purpose of this procedure is to provide instructions on how to change a frozen Microbiota Transplant product into a thawed one in the computer. |
| --- | --- |

| ##### Policy | - Frozen components must be changed into thawed ones before the products can be issued in the computer system. - Transfusion Service trained MLT and CLS may Component Prep Fecal Microbiota Transplant products. |
| --- | --- |

| ##### Procedure: Thawing FMT | FMT must be thawed at 30C Step; Action. 1.; - Adjust water bath temperature to 30C - Reset temperature setting to 30C by holding down * button until flashing - While depressing button, press down arrow until display is 30.0C - Discard 3-4 quart sized containers of water from bath and replace with cold tap water. - Add small amount of ice and stir contents to evenly mix temperature - Discard and stir cycle until temperature is adjusted.. 2.; Select shortest date of appropriate size product from inventory list posted on freezer.. 3.; Remove unit from freezer and record time on the order requisition.. Note: Expiration date and time of thawed product will be based on time removed from freezer.. 4.; Remove water bath cover and verify the water bath temperature is 30C. 5.; Press the Basket Access Button to raise the basket assembly.. 6.; Double bag frozen component with overwrap bag.. 7.; Place component in pouch/basket of water bath. Snap the top of the overwrap bag with the Security Snap to prevent product from being exposed to water during the thaw cycle.. 8.; Set cycle to 55 minutes.. 9.; Press “Start” to begin thaw cycle.. 10.; Remove the overwrap bag from the basket assembly and take out the component once thawing is complete. Note: If product still has ice chunks, Return to pouch and thaw for an additional 5-10 minutes. 11.; Proceed to Computer Processing FMT procedure below. |
| --- | --- |

| ##### Procedure: Computer Processing FMT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Step | Action |  |  |  |  |  |  |
|  |  | 1. | Open Blood Component Preparation folder in the Misys Gateway. |  |  |  |  |  |  |
|  |  | 2. | Enter STHFM in the value field. |  |  |  |  |  |  |
|  |  | 3. | Enter the information in the following fields. Use the TAB key to advance to the next field - Date - Time - Shift (this will default with current shift, it only needs to be changed if computer entry is being made after the fact as in the case of downtime) - Tech (will default in as one signed onto module, if data is being entered after the fact as in downtime, tech who actually performed thawing can be entered by placing tech code in value field followed by add button) |  |  |  |  |  |  |
|  |  | 4. | Click the continue button on bottom of screen. |  |  |  |  |  |  |
|  |  | 5. | Type in the unit number at Unit # field. |  |  |  |  |  |  |
|  |  | 6. | If component doesn’t fill in automatically, use drop down arrow to select appropriate product. |  |  |  |  |  |  |
|  |  | 7. | Find the component code in the table below. Enter the corresponding code as the output component |  |  |  |  |  |  |
|  |  |  |  | Component Code: |  | Output Component |  |  |  |
|  |  |  |  | 30 ml ending on 01 |  | FMTE1T |  |  |  |
|  |  |  |  | 30 ml ending in 02 |  | FMET2T |  |  |  |
|  |  |  |  | 30 ml ending in 03 |  | FMET3T |  |  |  |
|  |  |  |  | 30 m ending in 04 |  | FMET4T |  |  |  |
|  |  |  |  | 250 ml ending in 01 |  | FMTC1T |  |  |  |
|  |  |  |  | 250 ml ending in 02 |  | FMTC2T |  |  |  |
|  |  |  |  | 250 ml ending in 03 |  | FMTC3T |  |  |  |
|  |  |  |  | 250 ml ending in 04 |  | FMTC4T |  |  |  |
|  |  |  |  | 250 ml ending in 05 |  | FMTC5T |  |  |  |
|  |  |  |  | 250 ml ending in 06 |  | FMTC6T |  |  |  |
|  |  |  |  | 250 ml ending in 07 |  | FMTC7T |  |  |  |
|  |  |  |  | 250 ml ending in 08 |  | FMTC8T |  |  |  |
|  |  | 8. | Review the input and output information. |  |  |  |  |  |  |
|  |  |  |  | If: | Then: |  |  |  |  |
|  |  |  |  | Input and output information is correct | No action needed |  |  |  |  |
|  |  |  |  | Output information is incorrect | - Using cursor, click yellow component box, type in the correct component - Press tab. |  |  |  |  |
|  |  | 9. | Click Save. |  |  |  |  |  |  |
|  |  | 10. | This is the final step before unit changes are filed If:; Then:. Information is to be saved as displayed; Click OK when “File all units?” appears. Information needs to be corrected; Click Cancel when “File all units?” appears. When this is chosen the previous screen will be displayed. At this time you can correct errors, choose clear which will not file any of the changes that were performed but allow you to re-enter new units or choose Exit which will allow you to discard all data and exit the Blood Component Preparation function.. |
|  |  | 11. | Click close when output box displays. |  |  |  |  |  |  |
|  |  | 12. | Create a new expiration date label and place over frozen expiration date on bottle |  |  |  |  |  |  |
|  |  | 13. | Place EPIC patient label on bottle |  |  |  |  |  |  |
|  |  | 14. | Call Endoscopy or nursing unit to alert them that product is ready |  |  |  |  |  |  |
|  |  | 15. | Store product at Room Temperature until picked up |  |  |  |  |  |  |