#### Performing an Immediate Spin Crossmatch

| ##### Purpose | This procedure describes how to perform an Immediate Spin (I.S.) Crossmatch. |
| --- | --- |

| ##### Indications for Immediate Spin Crossmatch | I.S. crossmatches are used instead of the electronic crossmatch, when the following conditions are met: - Patient has no history of clinically significant antibodies - Patient’s current antibody screen is negative - The Laboratory system is “down” or the patient does not have 2 Blood Type results (by 2 CLS or 1 CLS and an automated analyzer) performed on samples that are acceptable for transfusion purposes using the same medical records number Otherwise, an AHG crossmatch must be done. Refer to procedures: Processing Assays on the ECHO Analyzer or Performing Selected Cells or AHG Crossmatches Using the Manual Capture System |
| --- | --- |

| ##### Policy | - The specimen must meet the criteria outlined in the Determining Specimen Acceptability for Transfusion Testing” procedure. - The technologist performing the crossmatch is responsible for verifying the specimen identification and reviewing test results prior to performing the crossmatch. - A patient history review must be performed prior to selecting donor units for crossmatching. - Units selected for crossmatch will be kept on the processing shelf of refrigerator until compatibility testing is completed |
| --- | --- |

| ##### Equipment/ Reagents/ Supplies |  |
| --- | --- |

| Equipment | Reagents | Supplies |
| --- | --- | --- |
| - Centrifuge - Agglutination viewer | - Saline | - 10x75 Tubes - Pipettes/droppers - Test tube rack - Hematype Segment device |

| ##### Specimen Requirements | Blood Bank band required - Adult: 10 ml EDTA - Pediatric: 5 ml EDTA |
| --- | --- |

| ##### Procedure | Follow the steps below to perform an I.S. crossmatch. |
| --- | --- |

| Step | Action |
| --- | --- |
| 1 | Verify patient identification on the specimen, Blood Bank ID band card, and computer order, including: patient name, medical record number, and Blood ID band number. |
| 2 | Verify that all indicated pre-transfusion testing is complete and that the patient qualifies for an Immediate Spin crossmatch.. |
| 3. | Review order for Special Blood Product requirements. |
| 4. | Select appropriate donor unit(s) and prepare a 3-5% donor cell suspension for each unit. Note: Complete donor unit type confirmation, if not already done. |
| 5. | Label a separate tube for each crossmatch with the initials of the patient and at least the last 3 digits of the unit number. |
| 6. | Add 2 drops of patient plasma to each tube. |
| 7. | Add 1 drop of the appropriate donor cell suspension to each tube. Mix well. |
| 8. | Spin at Immediate Spin setting of centrifuge. |
| 9. | Read macroscopically for hemolysis and /or agglutination. Record results immediately in LIS resulting grid. |
| 10. | Attach a Unit Tag to each compatible unit. Place the crossmatched unit(s) on the appropriate Crossmatched shelf of refrigerator. |

| ##### Interpreting Results | Interpret I.S. crossmatch as follows: |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | If… | Then crossmatch is… |  |
|  |  | No hemolysis and/or agglutination present | Compatible |  |
|  |  | Hemolysis and/or agglutination present | - Incompatible. Additional testing may be indicated. - Refer to Troubleshooting Incompatible I.S. Crossmatch Flowchart |  |

Continued on next page

| ##### Reporting Results | Complete the testing grid for each allocated unit J=ND, 5=ROU |
| --- | --- |

Crossmatch Result

| Unit Number | XIS | Interp | TS |
| --- | --- | --- | --- |
|  | 0 | [ or CMP | ] or OK |
|  | 5 | XMSR=NEG, [ or CMP XMSR=POS, { OR ICP | ] or OK } or NOTOK |
|  | 1+-4+ or H | { or ICP | } or NOTOK |
|  | J | See AHGXM Interp |  |

Add tests according to chart below as indicated:

| Situation | Test | Result |
| --- | --- | --- |
| Trauma patient | PB PI | Free text date followed by trauma name & trauma DOB Free text trauma name & trauma DOB |
| Unidentified patient, not trauma | PB PI | Free text date followed by unidentified name and unidentified DOB Free text unidentified name and unidentified DOB |
| No Info patient | PB PI | Free text in date followed by name prior to “No Info” and DOB Free text in name prior to “No Info” and DOB |
| Rouleaux | XMSR | Saline replacement result: N=Neg, P=Pos |
| Least incompatible units | BBC thru BBC9 | Free text names of Dr. & pathologist who authorized transfusion |
| Keep ahead order added | BBCNC thru BBCNC9 | Free text “Keep ahead ___units at all times” |
| Change to order as placed | BBCNC thru BBCNC9 | Free text change to order, including name of RN spoken to |

Continued on next page

| ##### Related Documents | - Determining Specimen Acceptability for Transfusion Service - Policy for Specimen Identification Verification - Policy on Compatibility Testing Method - Evaluating of Patient for Special Blood Components - Selecting Blood and Components for Transfusion - Preparing a 3-5% Patient or Donor Cell Suspension - Grading Serological Tube Reactions - Confirming a Blood Type on Donor RBC Units - Labeling a Blood Component for Transfusion - Troubleshooting Incompatible Immediate Spin Crossmatch |
| --- | --- |