#### Performing a Peg Additive Tube Method

| ##### Purpose | This procedure describes how to perform a PeG (Polyethylene Glycol) tube method in order to screen for unexpected blood group antibodies. |
| --- | --- |

| ##### Policy | This procedure is used: - when troubleshooting routine antibody screen methods - as part of an antibody identification work up, when indicated - As Selected cell rule outs when Capture antibody panel(s) have not ruled out Clinically Significant antibodies. Blood Bank Armbanded specimens are required for antibody screen when part of Type & Screen or Crossmatch order. |
| --- | --- |

| ##### Equipment / Reagents & Supplies |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Equipment | Reagents | Supplies |  |
|  |  | - Centrifuge - Heat block (36-38C) - Optical aid - Serological cell washer | - 3-5% reagent red blood cells - Peg Additive - Anti-IgG human Globulin - Coombs control cells - 0.9% Saline | - Test tubes - Test tube rack - Plastic pipettes |  |

| Specimen Requirements | One 10 ml EDTA stoppered tube is preferable. Serum collected from a red stoppered tube without separator gel or activator is also acceptable. |
| --- | --- |

| Method Limitations | - IgM antibodies may not be detected by the Peg test method. - Precipitation of fibrinogen may be observed when testing plasma samples. It may be necessary to wash the cells more than 3 times to remove all unbound human protein. |
| --- | --- |

| ##### Quality Control | Test screening cells with additive against QC serum each day of use. Record on QC form. |
| --- | --- |

| Procedure | Follow the steps below to perform this procedure: |
| --- | --- |

| Step | Action |  |
| --- | --- | --- |
|  | Bring reagents to room temperature. |  |
|  | Label each tube with a patient and cell identifier. |  |
|  | Add the following to each tube: - 2 drops of patient plasma or serum - 1 drop of the appropriate cell suspension - 2 drops of Peg enhancement reagent |  |
|  | Mix well and incubate all tubes at 37oC for 15 minutes (maximum of 30 minutes). |  |
|  | Examine the supernatant for gross hemolysis and record if present. |  |
|  | Do not centrifuge. Wash the cells 3-4 times with 0.9% saline manually or with serological cell washer |  |
|  | Immediately add 2 drops of Anti-IgG to the dry cell button and mix well. |  |
|  | Centrifuge and examine supernatant for hemolysis. |  |
|  | Examine macroscopically and microscopically for agglutination. |  |
|  | Grade and immediately record results in LIS. |  |
|  | Add 1 drop Coombs Control cells to each macroscopically negative reaction. |  |
|  | Mix and centrifuge. |  |
|  | Gently resuspend the cell button and observe for agglutination. |  |
|  | Grade and immediately record results in LIS. |  |
|  | If: | Then: |
|  | No agglutination after the addition of coombs control cells. | Invalid test. The test must be repeated. |
|  | Macroscopic agglutination in the IgG coated coombs control cells. | Valid test. Indicates that the anti-human globulin added was capable of reacting. |

| Reference Range: | Negative |
| --- | --- |

| ##### Interpreting Results: |  |
| --- | --- |

| If: | Then: |
| --- | --- |
| No agglutination or hemolysis is present in any of the reagent screening cells. | Result is negative - Indicates the absence of an antigen/antibody reaction. |
| Hemolysis and/or agglutination are present in one or more of the reagent screening cells. | Result is positive. - Indicates the presence of an antigen/antibody reaction. - Refer to Identifying an Antibody Procedure |

| Reporting Results | Insert Testing grid by adding test ASP or using W on the keyboard N=Neg P=Pos J=Not done |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Antibody Screen PeG |  |  | Interp |  |
|  |  |  | AHG | CC | N |  |
|  |  | S1 | 0 | 1-4 |  |  |
|  |  | S2 | 0 | 1-4 |  |  |
|  |  | S3 | 0 | 1-4 |  |  |
|  |  |  | AHG | CC | INTER |  |
|  | S1 |  | 0-4+ | ND-4+ | Any cell S1-S3 Pos at AHG P |  |
|  | S2 |  | 0-4+ | ND-4+ |  |  |
|  | S3 |  | 0-4+ | ND-4+ |  |  |

Record AHG and check cell results in grid for each screening cell tested.

Interpret Screen as Pos or Neg

| Related Documents | - Grading Serological Reactions - Interpreting an Antibody Identification Panel |
| --- | --- |