Manually Entering Echo Results into the Computer

| ##### Purpose: | The purpose of this procedure is to provide instructions on the process of manually entering Echo results into the computer. |
| --- | --- |

| ##### Policy: | - Bar code on specimen tube will be rechecked against computer screen to verify specimen identification at time of result entry. - Hand entered results will be verified immediately after acceptance by redisplaying the results and verifying data entry against original Echo print out and print out will be initialed to document the verification. - Capture AHG crossmatches performed on the Echo are not defined in the interface and will be resulted using this procedure. |
| --- | --- |

| ##### Procedure: |  |
| --- | --- |

| Step | Action |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | At end of run, remove tube from analyzer. |  |  |  |
| 2. | Locate result print out. |  |  |  |
| 3. | Apply a pour off label with patient name and CID number next to the corresponding CID number on the print out. |  |  |  |
| 4. | Enter function Blood Order Processing in the computer. |  |  |  |
| 5. | Bring up prompt to access patient by CID number. |  |  |  |
| 6. | Scan bar code on tube using computer hand held bar code reader. |  |  |  |
| 7. | Confirm that the patient name, medical record number and BB arm band number, if applicable, on computer screen is the same as that on the test tube. |  |  |  |
| 8. |  |  |  |  |
|  |  | If: | Then: |  |
|  |  | Name and/or medical record number does not match | - The wrong bar code has been placed on specimen. - Do not result. - Locate correct bar code and rerun specimen. - Notify Transfusion Service Coordinator/Supervisor. |  |
|  |  | BB armband number does not match | - Change armband number in computer. - If no patient results have been recorded on specimen, proceed to step 9. - If patient results have been recorded, repeat tests that have already been reported. |  |

| 9. | Locate print out and match up patient name on computer screen to corresponding result on print out. |  |  |  |
| --- | --- | --- | --- | --- |
| 10. | Use table below to insert grids need for test results If:; Then:. ABO Rh type; - Enter results into the ABORh grid - If Rh has been interpreted on print out, enter D1 results from Echo print out in the D testing position - If Rh has not been interpreted on print out due to discrepancy between D1 and D2 results, follow instructions in Performing Rh Type procedure. - Enter Monoclonal Control test results into the DC position of grid.. Antibody Screen; - Insert test ASSP3 or s. Capture Crossmatch; - Insert test XMSPH or f. |
| 11. | Enter results into appropriate grids in computer. See resulting section below for instructions on how to record results in the testing grid. Note: An immediate spin crossmatch is required in addition to Capture testing when an AHG crossmatch is required. See Performing an Immediate Spin Crossmatch procedure for instructions on performing the test. |  |  |  |
| 12. | Review data entered and accept results. - Initial print out after data entry review to document double check of manual data entry. |  |  |  |
| 13. |  |  |  |  |
|  |  | If: | Then: |  |
|  |  | Patient has previous blood type history | - Computer will flag blood type discrepancies. - Initial Capture Report. |  |
|  |  | Non Emergent blood release patient with no previous blood type history | - Recall patient in BOP and review data entry of results. - Perform retype. - Initial Capture Report. |  |
|  |  | Emergent Blood Release patient with no previous blood type history ( if not performed by tube method) | - Request a second tech to perform blood type. - Recall patient in BOP and review data entry of results. - Initial Capture Report |  |
| 14. | Save initialed Capture Reports for Monthly QA review. |  |  |  |

| ##### Reporting Results |  |
| --- | --- |

ABORh	S=AB   Enter J or ND for A1 and B cell on infants less than 4 months old

If the D1 and D2 do not match, the Echo will not result a Rh type. When this occurs, resolve the Rh type using tube method. See Performing a Rh Blood Type by Tube Method for guidance.

| MC | A | B | D | D2 | A1C | BC O | ABR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1-4+ | 1-4+ | 1-4+ | 1-4+ | O Pos |
| 0 | 0 | 0 | 0 | 0 | 1-4+ | 1-4+ | O Neg |
| 0 | 1-4+ | 0 | 1-4+ | 1-4+ | 0 | 1-4+ | A Pos |
| 0 | 1-4+ | 0 | 0 | 0 | 0 | 1-4+ | A Neg |
| 0 | 0 | 1-4+ | 1-4+ | 1-4+ | 1-4+ | 0 | B Pos |
| 0 | 0 | 1-4+ | 0 | 0 | 1-4+ | 0 | B Neg |
| 0 | 1-4+ | 1-4+ | 1-4+ | 1-4+ | 0 | 0 | S Pos |
| 0 | 1-4+ | 1-4+ | 0 | 0 | 0 | 0 | S Neg |

ASSP3

| Ab Scn Solid Phase 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| S1 | S2 | S3 | PQC | Interp |
| 0 | 0 | 0 | 1-4+ | N |
| 1-4+ | 0 | 0 | 1-4+ | P |
| 0 | 1-4+ | 0 | 1-4+ |  |
| 0 | 0 | 1-4+ | 1-4+ |  |
| 1-4+ | 1-4+ | 0 | 1-4+ |  |
| 0 | 1-4+ | 1-4+ | 1-4+ |  |
| 1-4+ | 0 | 1-4+ | 1-4+ |  |
| 1-4+ | 1-4+ | 1-4+ | 1-4+ |  |

XMSPH	Complete the testing grid for each allocated unit

J=ND, 5=ROU, CH=Complete Hemolysis, PH=Partial Hemolysis

*Hemolysis or agglutination in any phase or combination of phases

| Crossmatch, Result |  | XM, Solid Phase |  |  |  |
| --- | --- | --- | --- | --- | --- |
| XIS | XM | XISP | XMSP | PQC | XMSPH |
| ND | CCMP | 5 or 0 | 0 | 1-4+ | CCMP |
| ND | CICP | CH,PH 1-4+ | 0 | 1-4+ | CICP |
| ND | CICP | 0 | 1-4+ | 1-4+ | CICP |
| ND | CICP* | CH,PH 1-4+* | 1-4+* | 1-4+* | CICP* |

| ##### Related Documents | Reviewing Echo Test Results |
| --- | --- |